Oracle® SuperCluster T5-8

Configuration Worksheets

Part No. E40168-13

July 2015

Copyright © 2013, 2014, 2015, Oracle and/or its affiliates. All rights reserved.

This software and related documentation are provided under a license agreement containing restrictions on use and disclosure and are protected by intellectual property laws. Except as expressly permitted in your license agreement or allowed by law, you may not use, copy, reproduce, translate, broadcast, modify, license, transmit, distribute, exhibit, perform, publish, or display any part, in any form, or by any means. Reverse engineering, disassembly, or decompilation of this software, unless required by law for interoperability, is prohibited.

The information contained herein is subject to change without notice and is not warranted to be error-free. If you find any errors, please report them to us in writing.

If this is software or related software documentation that is delivered to the U.S. Government or anyone licensing it on behalf of the U.S. Government, the following notice is applicable:

U.S. GOVERNMENT END USERS. Oracle programs, including any operating system, integrated software, any programs installed on the hardware, and/or documentation, delivered to U.S. Government end users are "commercial computer software" pursuant to the applicable Federal Acquisition Regulation and agency-specific supplemental regulations. As such, use, duplication, disclosure, modification, and adaptation of the programs, including any operating system, integrated software, any programs installed on the hardware, and/or documentation, shall be subject to license terms and license restrictions applicable to the programs. No other rights are granted to the U.S. Government.

This software or hardware is developed for general use in a variety of information management applications. It is not developed or intended for use in any inherently dangerous applications, including applications that may create a risk of personal injury. If you use this software or hardware in dangerous applications, then you shall be responsible to take all appropriate fail-safe, backup, redundancy, and other measures to ensure its safe use. Oracle Corporation and its affiliates disclaim any liability for any damages caused by use of this software or hardware in dangerous applications.

Oracle and Java are registered trademarks of Oracle and/or its affiliates. Other names may be trademarks of their respective owners.

Intel and Intel Xeon are trademarks or registered trademarks of Intel Corporation. All SPARC trademarks are used under license and are trademarks or registered trademarks of SPARC International, Inc. AMD, Opteron, the AMD logo, and the AMD Opteron logo are trademarks or registered trademarks of Advanced Micro Devices. UNIX is a registered trademark of The Open Group.

This software or hardware and documentation may provide access to or information on content, products, and services from third parties. Oracle Corporation and its affiliates are not responsible for and expressly disclaim all warranties of any kind with respect to third-party content, products, and services. Oracle Corporation and its affiliates will not be responsible for any loss, costs, or damages incurred due to your access to or use of third-party content, products, or services.

Copyright © 2013, 2014, 2015, Oracle et/ou ses affiliés. Tous droits réservés.

Ce logiciel et la documentation qui l'accompagne sont protégés par les lois sur la propriété intellectuelle. Ils sont concédés sous licence et soumis à des restrictions d'utilisation et de divulgation. Sauf disposition de votre contrat de licence ou de la loi, vous ne pouvez pas copier, reproduire, traduire, diffuser, modifier, breveter, transmettre, distribuer, exposer, exécuter, publier ou afficher le logiciel, même partiellement, sous quelque forme et par quelque procédé que ce soit. Par ailleurs, il est interdit de procéder à toute ingénierie inverse du logiciel, de le désassembler ou de le décompiler, excepté à des fins d'interopérabilité avec des logiciels tiers ou tel que prescrit par la loi.

Les informations fournies dans ce document sont susceptibles de modification sans préavis. Par ailleurs, Oracle Corporation ne garantit pas qu'elles soient exemptes d'erreurs et vous invite, le cas échéant, à lui en faire part par écrit.

Si ce logiciel, ou la documentation qui l'accompagne, est concédé sous licence au Gouvernement des Etats-Unis, ou à toute entité qui délivre la licence de ce logiciel ou l'utilise pour le compte du Gouvernement des Etats-Unis, la notice suivante s'applique :

U.S. GOVERNMENT END USERS. Oracle programs, including any operating system, integrated software, any programs installed on the hardware, and/or documentation, delivered to U.S. Government end users are "commercial computer software" pursuant to the applicable Federal Acquisition Regulation and agency-specific supplemental regulations. As such, use, duplication, disclosure, modification, and adaptation of the programs, including any operating system, integrated software, any programs installed on the hardware, and/or documentation, shall be subject to license terms and license restrictions applicable to the programs. No other rights are granted to the U.S. Government.

Ce logiciel ou matériel a été développé pour un usage général dans le cadre d'applications de gestion des informations. Ce logiciel ou matériel n'est pas conçu ni n'est destiné à être utilisé dans des applications à risque, notamment dans des applications pouvant causer des dommages corporels. Si vous utilisez ce logiciel ou matériel dans le cadre d'applications dangereuses, il est de votre responsabilité de prendre toutes les mesures de secours, de sauvegarde, de redondance et autres mesures nécessaires à son utilisation dans des conditions optimales de sécurité. Oracle Corporation et ses affiliés déclinent toute responsabilité quant aux dommages causés par l'utilisation de ce logiciel ou matériel pour ce type d'applications.

Oracle et Java sont des marques déposées d'Oracle Corporation et/ou de ses affiliés.Tout autre nom mentionné peut correspondre à des marques appartenant à d'autres propriétaires qu'Oracle.

Intel et Intel Xeon sont des marques ou des marques déposées d'Intel Corporation. Toutes les marques SPARC sont utilisées sous licence et sont des marques ou des marques déposées de SPARC International, Inc. AMD, Opteron, le logo AMD et le logo AMD Opteron sont des marques ou des marques déposées d'Advanced Micro Devices. UNIX est une marque déposée d'The Open Group.

Ce logiciel ou matériel et la documentation qui l'accompagne peuvent fournir des informations ou des liens donnant accès à des contenus, des produits et des services émanant de tiers. Oracle Corporation et ses affiliés déclinent toute responsabilité ou garantie expresse quant aux contenus, produits ou services émanant de tiers. En aucun cas, Oracle Corporation et ses affiliés ne sauraient être tenus pour responsables des pertes subies, des coûts occasionnés ou des dommages causés par l'accès à des contenus, produits ou services tiers, ou à leur utilisation.

Content

Using This Documentation	4

Related Documentation	4

Feedback	4

Access to Oracle Support	4

Understanding the Configuration Worksheets	5

SR-IOV Domain Availability	6

Configuration Worksheets Purpose	6

Networks Overview	7

Configuration Process	8

Providing the Configuration Information for Each Compute Server	10

Oracle Setup of Database Zones and I/O Domains Overview	11

General Configuration Rules	11

Configuration Information for Each Compute Server	13

Allocating CPU and Memory Resources	23

Cores and Memory Available for Database Zones and I/O Domains	24

CPU and Memory Resource Allocation for the Half Rack	27

CPU and Memory Resource Allocation for the Full Rack	35

Completing the General Configuration Worksheets	45

General Oracle SuperCluster T5-8 Configuration Information	45

General Rack Configuration Worksheet	51

Customer Details Configuration Worksheet	53

Backup/Data Guard Ethernet Network Configuration Worksheet	54

Operating System Configuration Worksheet	55

Home and Database Configuration Worksheet	57

(Optional) Cell Alerting Configuration Worksheet	59

(Optional) Oracle Configuration Manager Configuration Worksheet	60

Auto Service Request Configuration Worksheet	61

Determining Network IP Addresses	63

IP Addresses and Oracle Enterprise Manager Ops Center 12c Release 2	64

Management Network IP Addresses	66

Client Access Network IP Addresses	69

InfiniBand Network IP Addresses	77

Change Log	82

Preface

Using This Documentation

This guide provides the configuration worksheets that must be completed before receiving Oracle SuperCluster T5-8. There are two intended audiences for this document:

- Customers who purchased Oracle SuperCluster T5-8 and will have the system installed at their site. Customers should use this document to provide customer-specific networking information that is necessary for a successful installation of the system.

- Oracle installers who will be configuring the system at the customer site. Oracle installers should refer to the networking information that was provided by the customer in this document and input that information into the appropriate configuration utility.

# Related Documentation

| Description | Links |
| --- | --- |
| All Oracle products | http://oracle.com/documentation |

# Feedback

Provide feedback on this documentation at:

http://www.oracle.com/goto/docfeedback

# Access to Oracle Support

Oracle customers have access to electronic support through My Oracle Support. For information visit http://www.oracle.com/pls/topic/lookup?ctx=acc&id=info or visit http://www.oracle.com/pls/topic/lookup?ctx=acc&id=trs if you are hearing impaired.

Chapter

1

Understanding the Configuration Worksheets

This document is designed to help define Oracle SuperCluster T5-8 configuration settings for your environment. Working with the network and database administrators, evaluate the current environmental settings, such as current IP address use and network configuration. Next, define the settings for Oracle SuperCluster T5-8, such as network configuration and backup method.

This document includes the configuration worksheets for Oracle SuperCluster T5-8. The Oracle SuperCluster T5-8 Owner’s Guide contains additional information, such as site requirements for Oracle SuperCluster T5-8.

The information is used to create the Oracle SuperCluster T5-8 Installation Template. It is important to complete the worksheets, and provide them to your Oracle representative prior to installation. All information is required unless otherwise indicated. The Installation Template will be used to complete installation and configuration of your Oracle SuperCluster T5-8. Site-specific adjustments to the Installation Template must be made in consultation with your Oracle representative.

Note - Complete the configuration worksheets early in the process, and prior to receiving your Oracle SuperCluster T5-8, so that site-specific adjustments to the Installation Template do not delay installation.

Note - If you have purchased more than one Oracle SuperCluster T5-8 and you do not plan to cable them together, then you must complete one set of worksheets for each Oracle SuperCluster T5-8.

# SR-IOV Domain Availability

The following SuperCluster-specific domain types have always been available:

- Application Domain running Oracle Solaris 10

- Application Domain running Oracle Solaris 11

- Database Domain

These SuperCluster-specific domain types have been available in software version 1.x and are now known as dedicated domains.

In addition to the dedicated domain types, the following version 2.x SR-IOV (Single-Root I/O Virtualization) domain types are now also available for SuperCluster systems installed after a certain date:

- Root Domains

- I/O Domains

The information in this document on SR-IOV domains applies only to SuperCluster systems running version 2.x software or later. To determine the version of software running on your SuperCluster system, on the management network, log in to one of the compute servers and type:

# svcprop -p configuration/build svc:/system/oes/id:default

- If you see the output ssc-1.x.x, then you have version 1.x software running on your SuperCluster system. Information on SR-IOV domains in this document does not apply to your system and should be ignored.

- If you see the output ssc-2.x.x (or later), then you have version 2.x software running on your SuperCluster system. All the information in this document on SR-IOV domains apply to your system.

# Configuration Worksheets Purpose

When you order an Oracle SuperCluster T5-8, you will be asked to make the following configuration choices:

- Type of Oracle SuperCluster T5-8:

- Half Rack

- Full Rack

Before Oracle SuperCluster T5-8 can be shipped to your site, you must also provide to Oracle several pieces of information specific to your Oracle SuperCluster T5-8, including:

- Number of domains on each compute server in Oracle SuperCluster T5-8, depending on the type of Oracle SuperCluster T5-8 that you have:

- Half Rack: 1 to 4 domains

- Full Rack:  1 to 8 domains

- Type of domains on each compute server:

- Application Domain running the Oracle Solaris 10 OS (dedicated domain)

- Application Domain running the Oracle Solaris 11 OS (dedicated domain)

- Database Domain (dedicated domain)

- Root Domain

Note – The Database Domain (dedicated domain) can also be in two states, with zones or without zones.

- Amount  of CPU and memory resources allocated to each domain on each compute server

- Starting IP addresses and number of IP addresses available for the following networks:

- Management network

- 10GbE client access network

- InfiniBand network

- Backup/Data Guard network, if applicable

Use the configuration worksheets in this document to provide Oracle these pieces of information.

# Networks Overview

The following networks are used with Oracle SuperCluster T5-8:

- Management network:  A single network used for the 1GbE host management and Oracle Integrated Lights Out Manager (Oracle ILOM). The management network is used for administrative work for all components of Oracle SuperCluster T5-8. It connects the management network interface and Oracle ILOM on all the components in the rack to the Cisco Catalyst 4948 Ethernet switch. The following connections are used for the components in the rack for the two management networks:

| Component | 1GbE Host Management | Oracle ILOM |
| --- | --- | --- |
| Compute servers | NET0 to NET3 ports | NET MGT ports |
| Exadata Storage Servers | NET0 ports | NET MGT ports |
| ZFS storage controller 1 | NET0 port | NET0 port |
| ZFS storage controller 2 | NET1 port | NET0 port |
| InfiniBand leaf and spine switches | NET0 ports | N/A |
| Power distribution units | NET MGT ports | N/A |

- Client access network:  10GbE network, with connections to the 10GbE network interface cards in the compute servers.

- InfiniBand private network:  Used for communication between components installed in Oracle SuperCluster T5-8. The InfiniBand private network is a non-routable network fully contained in Oracle SuperCluster T5-8, and does not connect to your existing network. The InfiniBand network requires two separate subnets for configuration. This network is automatically configured during installation.

- Backup/Data Guard network: Used as a backup network, if applicable.

# Configuration Process

Prior to the delivery of your Oracle SuperCluster T5-8, you will be asked to decide on the number of domains and the types of domains on each compute server in your Oracle SuperCluster T5-8. Depending on these domain types, certain components and domains will need to have a unique IP address and host name assigned to them. The number of components and domains used by your Oracle SuperCluster T5-8 will vary depending on the type of domain configuration you choose for each compute server.

You and your Oracle representative will work together to gather site-specific IP address and host name information by going through the following process:

- You will use the worksheets in this document to provide your Oracle representative with site-specific information, including the following:

- Starting IP addresses for the management and client access networks

- Number of IP addresses you will need for the networks, depending on the configurations you chose for each compute server in your system

Note – You will also be asked to confirm that the default IP addresses used for the private InfiniBand network do not conflict with other IP addresses on your network. If there are conflicts, you will be asked for starting IP addresses for the InfiniBand network in addition to the management and client access networks.

- The name for your Oracle SuperCluster T5-8 and your company network domain name, which your Oracle representative will use to generate host names for the components and domains in your system.

- Once you have completed all of the worksheets in this document, you will then send the completed document back to your Oracle representative.

- Your Oracle representative will use the information you provided in this document to create an Oracle SuperCluster T5-8 Installation Template specific to your site.  This site-specific Installation Template will provide several pieces of information, including IP addresses and host names for each component and domain in your Oracle SuperCluster T5-8, depending on the configurations you chose for each compute server in your system.

- Your Oracle representative will then send your completed site-specific Installation Template back to you to verify that there are no conflicts with the IP addresses assigned to your system. Your Oracle representative will work with you to resolve any conflicts with the IP addresses, if any conflicts arise.

- Once the Installation Template is complete and all IP address conflicts have been resolved, you will use the information in the Installation Template to register the IP addresses and host names in DNS. All IP addresses and host names for your Oracle SuperCluster T5-8 must be registered in DNS before your Oracle SuperCluster T5-8 can be installed at your site.

Note - All IP addresses must be statically-assigned IP addresses, not dynamically-assigned (DHCP) addresses.

Chapter

2

Providing the Configuration Information for Each Compute Server

Complete the worksheets in this chapter to provide the following information:

- Number of domains on each compute server in Oracle SuperCluster T5-8, depending on the type of Oracle SuperCluster T5-8 that you have:

- Half Rack: 1 to 4 domains

- Full Rack: 1 to 8 domains

- Type of domains on each compute server:

- Application Domain running the Oracle Solaris 10 OS (dedicated domain)

- Application Domain running the Oracle Solaris 11 OS (dedicated domain)

- Database Domain (dedicated domain)

- Root Domain

- Whether the Database Domain will or will not contain zones, and the number of zones that you want your Oracle installer to configure during the initial installation

- Number of I/O Domains that you want your Oracle installer to configure during the initial installation, if you have one or more Root Domains configuredError: Reference source not found

Note – Only Database Domains that are dedicated domains can host database zones. Database I/O Domains cannot host database zones.

# Oracle Setup of Database Zones and I/O Domains Overview

As part of a typical initial installation of your Oracle SuperCluster, the Oracle installer will set up any dedicated domains (Database Domains or Application Domains) and any Root Domains that will be part of your Oracle SuperCluster configuration.

Additionally, your Oracle installer can configure a combination of up to eight of the following items:

- Database zones (zones hosted on Database Domains that are dedicated domains)

- I/O Domains (either Application I/O Domains or Database I/O Domains)

For example, as part of the initial installation of your Oracle SuperCluster, you could have your Oracle installer set up four database zones and four I/O Domains, or two database zones and six I/O Domains.

After the initial installation, you can set up additional database zones and I/O Domains using the instructions provided in the following documents:

- Database zones: Oracle SuperCluster T5-8 Zones With Oracle Database on Database Domains Configuration Guide

- I/O Domains: I/O Domain Administration Guide

# General Configuration Rules

Following are the general configuration rules for your Oracle SuperCluster T5-8:

- When deciding which domains will be a Root Domain, the last domain must always be the first Root Domain, and you would start from the last domain in your configuration and go in for every additional Root Domain. For example, assume you have four domains in your configuration, and you want two Root Domains and two dedicated domains. In this case, the first two domains would be dedicated domains and the last two domains would be Root Domains.

- If you have a mixture of dedicated domains and Root Domains, you cannot have all of the domains as Root Domains. At least one domain (the Control Domain) must be a dedicated domain in this case.

- Every compute server has a Control Domain, and on Oracle SuperCluster T5-8, the Control Domain always runs the Oracle Solaris 11 OS.

  - If one or more of your domains is a Root Domain, then the Control Domain must be a Database Domain (dedicated domain).

  - If all of your domains are dedicated domains, and one or more Database Domains are present on the compute server, then a Database Domain becomes the Control Domain. Otherwise an Application Domain running the Oracle Solaris 11 OS becomes the Control Domain.

- If you want multiple domains on the compute servers, the Control Domain is always the first domain on the servers.

- An Application Domain running Oracle Solaris 10 is valid as at the last domain location only if there are exactly two domains on the server (the H2-1 and F2-1 configurations).

- Only Database Domains that are dedicated domains can host database zones. If you want database zones on your Database Domains, you must select the Database Domain (dedicated domain) in order to have database zones.

- A domain cannot be a Root Domain if it has more than two IB HCAs associated with it. For the Oracle SuperCluster T5-8, either a Half Rack or a Full Rack, the following domains are the only acceptable domains for a Root Domain:

  - Small Domains (one IB HCA)

  - Medium Domains (two IB HCAs)

Note - Even though a domain with two IB HCAs is valid for a Root Domain, domains with only one IB HCA should be used as Root Domains. When a Root Domain has a single IB HCA, fewer I/O Domains will have dependencies on the I/O devices provided by that Root Domain. Flexibility around high availability also increases with Root Domains with one IB HCA.

- If you have a mixture of dedicated domains and Root Domains, the following rules apply when reallocating CPU and memory resources after the initial installation and after I/O Domains have been created:

  - You can reallocate CPU and memory resources between dedicated domains

  - You can park CPU and memory resources that were allocated to dedicated domains. Those parked core and memory resources are now available for future I/O Domains that you will create through the I/O Domain Creation tool.

  - Once you have parked CPU and memory resources from the dedicated domains, you cannot unpark them and reallocate them back to the dedicated domains once you begin creating I/O Domains. Once you begin creating I/O Domains, any parked CPU and memory resources are now used exclusively for I/O Domains and are no longer available for dedicated domains.

  - You cannot reallocate CPU and memory resources for Root Domains after the initial installation.

- See Allocating CPU and Memory Resource on page 26 for more information.

- You will be providing configuration information in this document where you are telling your Oracle installer whether you want Root Domains or dedicated domains at the time of the initial installation. In addition, your Oracle installer can configure up to eight database zones or I/O Domains as part of the initial installation, as described in Oracle Setup of Database Zones and I/O Domains Overview on page 11.

- While you will be providing information in this document on the I/O Domains that will be set up as part of the initial installation, you should also consider the size of the I/O Domains that you will be creating after the initial installation when deciding on Root Domains or dedicated domains. You should not create I/O Domains that are larger than one socket, so if you were planning to create I/O Domains that are that large, you should not choose a Root Domain and you should choose a dedicated domain instead.

# Configuration Information for Each Compute Server

Note – Refer to the Oracle SuperCluster T5-8 Owner’s Guide for more detailed information on the different configurations available.

Use the tables in the following sections to enter the configuration information for each compute server in your system, depending on the configuration for your Oracle SuperCluster T5-8:

- Half Rack on page 14

- Full Rack on page 18

Half Rack

The following configurations are available for the Half Rack version of your Oracle SuperCluster T5-8:

- Config H1-1: One domain (one Large Domain)

- Config H2-1: Two domains (two Medium Domains)

- Config H3-1: Three domains (one Medium Domain, two Small Domains)

- Config H4-1: Four domains (four Small Domains)

The following figure shows these available configurations for the Half Rack version of your Oracle SuperCluster T5-8.

For the most part, the domains can be any of the following domain types, keeping in mind the domain configuration rules outlined in General Configuration Rules on page 11:

- Application Domain running the Oracle Solaris 10 OS (dedicated domain)

- Application Domain running the Oracle Solaris 11 OS (dedicated domain)

- Database Domain  (dedicated domain), with or without zones

- Root Domain, with some I/O Domains set up at the initial installationError: Reference source not found

Use the tables in the following sections to enter the configuration information for each compute server in your Half Rack.

For example, assume you want the following configuration for the first compute server in your Half Rack:

- Config H4-1: Four domains (four Small Domains)

- These types of domains:

- First Small Domain: Database Domain, containing zones (DB-Z), with the Oracle installer setting up four database zones

- Second Small Domain: Application Domain running the Oracle Solaris 11 OS (APP-S11)

- Third Small Domain: Database Domain, where the Database Domain does not contain zones (DB)

- Fourth Small Domain: Root Domain, with the Oracle installer setting up four I/O Domains

For this configuration, you would fill out the configuration information for that compute server in this way:

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four |
|  | H1-1 |  |  |  |  |
|  | H2-1 |  |  |  |  |
|  | H3-1 |  |  |  |  |
| X | H4-1 | DB-Z: 4 zones | APP-S11 | DB | ROOT: 4 I/O Domains |

The following table shows the only valid options for each domain for each LDom configuration:

|  | Valid Domains on Each Compute Server |  |  |  |
| --- | --- | --- | --- | --- |
| Config | One | Two | Three | Four |
| H1-1 | APP-S11 DB DB-Z |  |  |  |
| H2-1 | APP-S11 DB DB-Z |  | APP-S10 APP-S11 DB DB-ZError: Reference source not found ROOTError: Reference source not found |  |
| H3-1 | APP-S11 DB DB-Z |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |
| H4-1 | APP-S11 DB DB-Z | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |

Note – If you want database zones on Database Domains (dedicated domains), either at the time of the initial installation or at some point in the future, enter DB-Z as the domain type in the following tables.

Use the following tables to enter the configuration information for each server in your system. Refer to the information in these tables when completing the configuration worksheets in the rest of this document:

- Server 1 Configuration Information (Half Rack) on page 17

- Server 2 Configuration Information (Half Rack) on page 17

Server 1 Configuration Information (Half Rack)

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four |
|  | H1-1 |  |  |  |  |
|  | H2-1 |  |  |  |  |
|  | H3-1 |  |  |  |  |
|  | H4-1 |  |  |  |  |

Server 2 Configuration Information (Half Rack)

|  |  | Number and Type of Domains on Compute Server 2 |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four |
|  | H1-1 |  |  |  |  |
|  | H2-1 |  |  |  |  |
|  | H3-1 |  |  |  |  |
|  | H4-1 |  |  |  |  |

Full Rack

Note – Keep the following information in mind if you choose a configuration that contains a Small Domain.

If you have the Full Rack, you cannot install a Fiber Channel PCIe card in a slot that is associated with a Small Domain. In Full Rack configurations, Fiber Channel PCIe cards can only be added to domains with more than one 10GbE NIC. One 10GbE NIC must be left for connectivity to the client access network, but for domains with more than one 10GbE NICs, other 10GbE NICs can be replaced with Fiber Channel HBAs. Refer to the Oracle SuperCluster T5-8 Owner’s Guide for more information on the configurations with Small Domains.

The following configurations are available for the Full Rack version of your Oracle SuperCluster T5-8:

- Config F1-1: One domain (one Giant Domain)

- Config F2-1: Two domains (two Large Domains)

- Config F3-1: Three domains (one Large Domain, two Medium Domains)

- Config F4-1: Four domains (four Medium Domains)

- Config F4-2: Four domains (one Large Domain, two Small Domains, one Medium Domain)

- Config F5-1: Five domains (two Medium Domains, two Small Domains, one Medium Domain)

- Config F5-2: Five domains (one Large Domain, four Small Domains)

- Config F6-1: Six domains (one Medium Domain, four Small Domains, one Medium Domain)

- Config F7-1: Seven domains (one Medium Domain, six Small Domains)

- Config F8-1: Eight domains (eight Small Domains)

The following figure shows these available configurations for the Full Rack version of your Oracle SuperCluster T5-8.

For the most part, the domains can be any of the following domain types, keeping in mind the domain configuration rules outlined in General Configuration Rules on page 11:

- Application Domain running the Oracle Solaris 10 OS (dedicated domain)

- Application Domain running the Oracle Solaris 11 OS (dedicated domain)

- Database Domain (dedicated domain), with or without zones

- Root Domain, with some I/O Domains set up at the initial installationError: Reference source not found

Use the tables in the following sections to enter the configuration information for each compute server in your Full Rack.

For example, assume you want the following configuration for the first compute server in your Full Rack:

- Config F5-2: Five domains (one Large Domain and four Small Domains)

- These types of domains:

- Large Domain: Database Domain , where the Database Domain does not contain zones (DB)

- First Small Domain: Application Domain running the Oracle Solaris 11 OS (APP-S11)

- Second Small Domain: Application Domain running the Oracle Solaris 10 OS (APP-S10)

- Third Small Domain: Database Domain, containing zones (DB-Z), with the Oracle installer setting up four database zones

- Fourth Small Domain: Root Domain, with the Oracle installer setting up four I/O Domains

For this configuration, you would fill out the configuration information for that server in this way:

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight |
|  | F1-1 |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |
| X | F5-2 | DB |  |  |  | APP-S11 | APP-S10 | DB-Z: 4 zones | ROOT: 4 I/O Domains |
|  | F6-1 |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |

The following table shows the only valid options for each domain for each LDom configuration:

|  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Config | One | Two | Three | Four | Five | Six | Seven | Eight |
| F1-1 | APP-S11 DB DB-Z |  |  |  |  |  |  |  |
| F2-1 | APP-S11 DB DB-Z |  |  |  | APP-S10 APP-S11 DB DB-ZError: Reference source not found |  |  |  |
| F3-1 | APP-S11 DB DB-Z |  |  |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found |  | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |  |
| F4-1 | APP-S11 DB DB-Z |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found |  | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |  |
| F4-2 | APP-S11 DB DB-Z |  |  |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |  |
| F5-1 | APP-S11 DB DB-Z |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (4th)Error: Reference source not found |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |  |
| F5-2 | APP-S11 DB DB-Z |  |  |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (4th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |
| F6-1 | APP-S11 DB DB-Z |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (5th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (4th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |  |
| F7-1 | APP-S11 DB DB-Z |  | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (6th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (5th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (4th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |
| F8-1 | APP-S11 DB DB-Z | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (7th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (6th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (5th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (4th)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (3rd)Error: Reference source not found | APP-S10 APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (2nd)Error: Reference source not found | APP-S11 DBError: Reference source not found DB-ZError: Reference source not found ROOT (1st)Error: Reference source not found |

Note – If you want database zones on Database Domains (dedicated domains), either at the time of the initial installation or at some point in the future, enter DB-Z as the domain type in the following tables.

Use the following tables to enter the configuration information for each server in your system. Refer to the information in these tables when completing the configuration worksheets in the rest of this document:

- Server 1 Configuration Information (Full Rack) on page 23

- Server 2 Configuration Information (Full Rack) on page 24

Server 1 Configuration Information (Full Rack)

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight |
|  | F1-1 |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |

Server 2 Configuration Information (Full Rack)

|  |  | Number and Type of Domains on Compute Server 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight |
|  | F1-1 |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |

What’s Next

Go to Allocating CPU and Memory Resource on page 26 to provide information on the amount of CPU and memory resources that you want to have allocated to each domain in each compute server in Oracle SuperCluster T5-8.

Chapter

3

Allocating CPU and Memory Resources

Depending on the number of domains that you have on each compute server, you can have from one to eight sockets of CPU and memory resources allocated for each domain on the compute servers in your Oracle SuperCluster T5-8, depending on the type of Oracle SuperCluster T5-8 (Half Rack or Full Rack) and the size of the domains on each compute server. In addition, the sockets can be broken down further to individual cores (16 cores per socket).

Use the tables in the following sections to enter the information for the amount of CPU and memory resources that you want allocated to each domain for each compute server in your system. Refer to the information that you provided in the chapter Providing the Configuration Information for Each Compute Server to determine the number and type of domains that you have on each compute server:

- Cores and Memory Available for Database Zones and I/O Domains on page 27

- CPU and Memory Resource Allocation for the Half Rack on page 30

- CPU and Memory Resource Allocation for the Full Rack on page 39

# Cores and Memory Available for Database Zones and I/O Domains

Note – Database zones can only be created on Database Domains that are dedicated domains. The information in this section applies to database zones and Database Domains that are dedicated domains.

Because the number of processor modules in each compute server varies, depending on whether you have a Half Rack or Full Rack version of the Oracle SuperCluster T5-8, the number of cores and the amount of memory available for each compute server in your Oracle SuperCluster T5-8 also varies:

- For the Half Rack version of the Oracle SuperCluster T5-8, each compute server contains two processor modules. There are two sockets or PCIe root complex pairs on each processor module. Sixteen cores are associated with each socket, for a total of 64 cores (four sockets or PCIe root complex pairs) for each compute server in the Half Rack. 1 TB, or 1024 GB, of memory is available for the Half Rack.

- For the Full Rack version of the Oracle SuperCluster T5-8, each compute server contains four processor modules. There are two sockets or PCIe root complex pairs on each processor module. Sixteen cores are associated with each socket, for a total of 128 cores (eight sockets or PCIe root complex pairs) for each compute server in the Full Rack.  2 TB, or 2048 GB, of memory is available for the Full Rack.

The following sections provide more information on the cores and memory available for database zones and I/O Domains:

- Cores and Memory Available for Database Zones on page 27

- Cores and Memory Available for I/O Domains on page 29

Cores and Memory Available for Database Zones

Note - See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information on the maximum number of database zones and I/O Domains that can be set up by your Oracle installer.

When you first install the operating system instances on a domain, that domain is automatically designated as the global zone. When creating zones on Database Domains, the Database Domain is designated as the global zone, and the zones created on that Database Domain are designated as nonglobal zones.

A certain number of cores will always be set aside for the global zone (the Database Domain), and the remaining cores in the Database Domain are available for the nonglobal zones (the zones in the Database Domain). The number of cores that are set aside for the global zone varies, depending on the size of the domain.

The following table provides information on the number of cores that are available for specific domains and zones that are created within those domains. See Configuration Information for Each Compute Server on page 14 and the Oracle SuperCluster T5-8 Owner’s Guide for more detailed information on the different configurations available.

|  | Total Number of Cores Available for This Domain | Number of Cores Set Aside for Global Zone | Number of Cores Available for Nonglobal Zones |
| --- | --- | --- | --- |
| Giant Domain (Full Rack Only) | 128 cores (8 sockets) | 4 cores | 124 cores |
| Large Domain | 64 cores (4 sockets) | 4 cores | 60 cores |
| Medium Domain | 32 cores (2 sockets) | 4 cores | 28 cores |
| Small Domain | 16 cores (1 socket) | 2 cores | 14 cores |

When using the information in the table above, keep in mind that the number of cores that are set aside for the global zone applies only when you are creating zones (nonglobal zones) on that Database Domain. In that case, a certain number of cores are reserved for the Database Domain (the global zone) and the remaining cores are available for the zones on that Database Domain (the nonglobal zones). If you have a Database Domain with no zones, then all the cores are available for that Database Domain.

For each zone that you want created, use a minimum of one core per zone. However, depending on the workload that you expect on a zone, a larger number of cores per zone might be preferable, thereby reducing the total number of zones on each compute server. Carefully consider the expected workload on each zone that you create, so that you allot the appropriate number of cores to those zones.

The amount of memory available for database zones depends, in part, on the following factors:

- The  amount of memory available for the entire system

- How that memory is divided between the domains in the system, in particular for Database Domains that contain zones

- How much memory you want to associate with each database zone, keeping in mind that you may need to reserve some memory for future database zones

For example, for a Half Rack, with 1 TB (1024 GB) of total memory available, you might have 50% of the memory (516 GB) assigned to a Database Domain that contains zones. If you have four zones set up at the initial installation, you might associate 50 GB of memory to each of the four database zones (200 GB total), with the remaining 316 GB of memory set aside for future database zones.

Cores and Memory Available for I/O Domains

Note - See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information on the maximum number of database zones and I/O Domains that can be set up by your Oracle installer.

If you want I/O Domains set up on your Oracle SuperCluster, either at the time of the initial installation or afterwards, you must have at least one Root Domain set up at the time of the initial installation. I/O Domains can then be created from these Root Domains.

A certain amount of CPU core and memory is always reserved for each Root Domain, depending on which domain is being used as a Root Domain in the domain configuration and the number of IB HCAs and 10GbE NICs that are associated with that Root Domain:

- The last domain in a domain configuration:

- Two cores and 32 GB of memory reserved for a Root Domain with one IB HCA and 10GbE NIC

- Four cores and 64 GB of memory reserved for a Root Domain with two IB HCAs and 10GbE NICs

- Any other domain in a domain configuration:

- One core and 16 GB of memory reserved for a Root Domain with one IB HCA and 10GbE NIC

- Two cores and 32 GB of memory reserved for a Root Domain with two IB HCAs and 10GbE NICs

The remaining CPU core and memory resources allocated with each Root Domain are parked in CPU and memory repositories, which can then be used by I/O Domains.

Note – For more information on the number of IB HCAs and 10GbE NICs associated with each domain, see Configuration Information for Each Compute Server on page 14.

CPU and memory repositories contain resources not only from the Root Domains, but also any parked resources from the dedicated domains. Whether CPU core and memory resources originated from dedicated domains or from Root Domains, once those resources have been parked in the CPU and memory repositories, those resources are no longer associated with their originating domain. These resources become equally available to I/O Domains.

In addition, CPU and memory repositories contain parked resources only from the compute server that contains the domains providing those parked resources. In other words, if you have two compute servers and both compute servers have Root Domains, there would be two sets of CPU and memory repositories, where each compute server would have its own CPU and memory repositories with parked resources.

For example, assume you have four domains on your compute server, with three of the four domains as Root Domains. Assume each domain has the following IB HCAs and 10GbE NICs, and the following CPU core and memory resources:

- One IB HCA and one 10GbE NIC

- 16 cores

- 256 GB of memory

In this situation, the following CPU core and memory resources are reserved for each Root Domain, with the remaining resources available for the CPU and memory repositories:

- Two cores and 32 GB of memory reserved for the last Root Domains in this configuration. 14 cores and 224 GB of memory available from this Root Domain for the CPU and memory repositories.

- One core and 16 GB of memory reserved for the second and third Root Domains in this configuration.

- 15 cores and 240 GB of memory available from each of these Root Domains for the CPU and memory repositories.

- A total of 30 cores (15 x 2) and 480 GB of memory (240 GB x 2) available for the CPU and memory repositories from these two Root Domains.

A total of 44 cores (14 + 30 cores) are therefore parked in the CPU repository, and 704 GB of memory (224 + 480 GB of memory) are parked in the memory repository and are available for the I/O Domains.

# CPU and Memory Resource Allocation for the Half Rack

For the Half Rack, each compute server would have two processor modules (PM0 and PM3), with two sockets or PCIe root complex pairs on each processor module, for a total of four sockets or PCIe root complex pairs for each compute server. Following are the maximum CPU and memory resources available for the Half Rack:

- Four sockets

- 64 cores

- 512  VCPUs

- 1 TB (1024 GB) of memory

The amount of CPU and memory resources that you can allocate to each domain depends on the following things:

- The size of the domain (Large Domain, Medium Domain, or Small Domain) on the compute server

- The size of the other domains that are also on that compute server and the number of sockets assigned to those other domains

The following guidelines describe the allowable number of sockets that you can assign to each domain:

- Large Domain: One domain is set up on the compute server in this configuration, taking up all of the server. 100% of the CPU and memory resources are allocated to this single domain on this server (all four sockets).

Note - You can use the CPU/Memory tool (osc-setcoremem) to change this default allocation after the initial installation of your system, if you want to have some CPU or memory resources parked (unused). Refer to the Oracle SuperCluster T5-8 Owner’s Guide for more information.

- Medium Domain: You can assign between 1 to 3 sockets for a Medium Domain in each compute server, depending on the size of the other domains that are also on that compute server and the number of sockets assigned to those other domains.

- Small Domain: You can assign between 1 to 2 sockets for a Small Domain in each compute server, depending on the size of the other domains that are also on that compute server and the number of sockets assigned to those other domains.

You can allocate CPU and memory resources to the domains in either of the following ways:

- At a socket-based level, where between 1 and 4 sockets are assigned to each domain, based on the guidelines listed previously.

- At a granular level, where CPU resources are assigned based on the number of cores that you want assigned to the domain and memory resources are assigned based on the amount of memory, in GBs, that you want assigned to the domain.

Note that if you choose the socket-based level for one type of resource, then you will also have to choose the socket-based level for the other type of resource. For example, if you decided to assign one socket to a domain for the CPU resources (out of four, or 25%of the CPU resources). You would then have to also assign 25% of the memory resources, or 256 GB of memory, to this domain.

However, if you choose to assign resources at a granular level, you do not have to have the same proportions. For example, you could choose 32 cores (50% of the CPU resources) and 256 GB of memory (25% of the memory resources) for a domain in this case.

Provide the amount of CPU and memory resources that you want allocated to each domain in the following sections:

- CPU Resource Allocation for the Half Rack on page 31

- Memory Resource Allocation for the Half Rack on page 36

CPU Resource Allocation for the Half Rack

The total number of sockets that you have assigned to all the domains in each compute server in a Half Rack must add up to four sockets, unless you decide to have some CPU resources parked.

In addition, the sockets can be broken down further to individual cores (16 cores per socket). The total number of cores that you have assigned to all domains in each compute server in a Half Rack must add up to 64 cores, unless you decide to have some CPU resources parked.

Use the tables in this section to provide information on the number of sockets or cores that you want to have assigned for each domain in each compute server in Oracle SuperCluster T5-8.

For example, assume you want the following configuration for the first compute server in your Half Rack:

- Config H4-1: Four domains (four Small Domains)

- These types of domains:

- First Small Domain: Database Domain, containing zones (DB-Z), with the Oracle installer setting up four database zones

- Second Small Domain: Application Domain running the Oracle Solaris 11 OS (APP-S11)

- Third Small Domain: Database Domain, where the Database Domain does not contain zones (DB)

- Fourth Small Domain: Root Domain, with the Oracle installer setting up four I/O Domains

You could assign one socket to each domain, which totals four sockets altogether for all domains on the server.

In addition, the sockets can be broken down further to individual cores for the first domain, which would be a Database Domain (dedicated domain) that contains four zones. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, you would have the following cores available for that Database Domain (global zone) and the zones within that Database Domain (nonglobal zones):

- Database Domain: 2 cores set aside for global zone

- Zones within that Database Domain: 14 cores available for nonglobal zones

Because you have 14 cores available for the zones in that Database Domain, you could have 4 cores assigned to the first two zones (8 cores for both) and 3 cores assigned to the other two zones (6 cores for both), for a total of 14 cores. Or you could allocate a smaller number of available cores to each zone (for example, 2 cores to each zone, or 8 cores total) and save the remaining cores for future zones that you might want to create on that Database Domain.

Similarly, the sockets can be broken down further to individual cores for the fourth (last) domain, which would be a Root Domain with four I/O Domains configured off of that Root Domain. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, you would have the following cores available for the Root Domain and the I/O Domains:

- Root Domain: 2 cores set aside for the Root Domain

- I/O Domains: 14 remaining cores available for the I/O Domains

Note - Additional cores could be available for I/O Domains if cores from other domains were parked. For the purposes of this exercise, however, we are assuming that no other cores from other domains are parked, and the remaining 14 cores from this Root Domain are the only cores available for the I/O Domains.

Because you have 14 cores available for I/O Domains, you could create I/O Domains similar to the way you created database zones, where you could have 4 cores assigned to the first two I/O Domains (8 cores for both) and 3 cores assigned to the other two I/O Domains (6 cores for both), for a total of 14 cores. Or you could allocate a smaller number of available cores to each I/O Domain (for example, 2 cores to each I/O Domain, or 8 cores total) and save the remaining cores for future I/O Domains that you might want to create on that Root Domain.

Assuming you wanted to allocate 2 cores for each database zone and I/O Domain, saving the remaining cores for future database zones and I/O Domains, you would complete the table in this section in this manner in this situation:

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Total Number of Sockets or Cores |
|  | H1-1 |  |  |  |  |  |
|  | H2-1 |  |  |  |  |  |
|  | H3-1 |  |  |  |  |  |
| X | H4-1 | DB-Z: 1 socket (16 cores) - Global zone: 2 cores set aside - 4 nonglobal zones: 2 cores per zone | APP-S11: 1 socket (16 cores) | DB: 1 socket (16 cores) | ROOT: 1 socket (16 cores) - Root Domain: 2 cores set aside - 4 I/O Domains: 2 cores per I/O Domain | 4 sockets (64 cores) |
| NOTE: Total number of sockets for all domains must be 4, or total number of cores must be 64, unless some resources are parked. |  |  |  |  |  |  |

Note – As described in General Configuration Rules on page 11, if you have a mixture of dedicated domains and Root Domains, after the initial installation, you can reallocate CPU resources only with the dedicated domains. You cannot reallocate CPU resources for Root Domains after the initial installation. 

Because resources allocated to Root Domains at the initial installation cannot be used by dedicated domains, carefully consider the amount of CPU resources that you want to have allocated to Root Domains at the time of the initial installation. In addition, once you have parked CPU resources from the dedicated domains, you cannot unpark them and reallocate them back to the dedicated domains after the initial installation.

Use the following tables to enter the CPU resource allocation information for each compute server in your system:

- Compute Server 1 CPU Resource Allocation (Half Rack) on page 35

- Compute Server 2 CPU Resource Allocation (Half Rack) on page 35

Compute Server 1 CPU Resource Allocation (Half Rack)

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Total Number of Sockets or Cores |
|  | H1-1 |  |  |  |  |  |
|  | H2-1 |  |  |  |  |  |
|  | H3-1 |  |  |  |  |  |
|  | H4-1 |  |  |  |  |  |
| NOTE: Total number of sockets for all domains must be 4, or total number of cores must be 64, unless some resources are parked. |  |  |  |  |  |  |

Compute Server 2 CPU Resource Allocation (Half Rack)

|  |  | Number and Type of Domains on Compute Server 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Total Number of Sockets or Cores |
|  | H1-1 |  |  |  |  |  |
|  | H2-1 |  |  |  |  |  |
|  | H3-1 |  |  |  |  |  |
|  | H4-1 |  |  |  |  |  |
| NOTE: Total number of sockets for all domains must be 4, or total number of cores must be 64, unless some resources are parked. |  |  |  |  |  |  |

Memory Resource Allocation for the Half Rack

The total amount of memory that you have assigned to all domains in each compute server in a Half Rack must add up to 1 TB, or 1024 GB, unless you decide to have some memory resources parked.

Use the tables in this section to provide information on the amount of memory that you want to have assigned for each domain in each compute server in Oracle SuperCluster T5-8.

For example, assume you want the following configuration for the first compute server in your Half Rack:

- Config H4-1: Four domains (four Small Domains)

- These types of domains:

- First Small Domain: Database Domain, containing zones (DB-Z), with the Oracle installer setting up four database zones

- Second Small Domain: Application Domain running the Oracle Solaris 11 OS (APP-S11)

- Third Small Domain: Database Domain, where the Database Domain does not contain zones (DB)

- Fourth Small Domain: Root Domain, with the Oracle installer setting up four I/O Domains

You could assign 256 GB of memory to each domain, which totals 1024 GB of memory altogether for all domains on this server.

In addition, the amount of memory used can be broken down further for the first domain, which would be a Database Domain (dedicated domain) that contains four zones. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, if 256 GB of memory is assigned to this Database Domain, you could have 25 GB of memory assigned to each database zone in this Database Domain, for a total of 100 GB of memory for all four database zones. The remaining 156 GB of memory in this Database Domain could then be saved for future database zones that you might want to create on this Database Domain.

Similarly, the amount of memory can be broken down further for the fourth (last) domain, which would be a Root Domain with four I/O Domains configured off of that Root Domain. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, if 256 GB of memory is assigned to this Root Domain, you would have the following memory available for the Root Domain and the I/O Domains:

- Root Domain: 32 GB of memory set aside for the Root Domain

- I/O Domains: 224 GB of remaining memory available for the I/O Domains

Note - Additional memory could be available for I/O Domains if memory resources from other domains were parked. For the purposes of this exercise, however, we are assuming that no other memory resources from other domains are parked, and the remaining 224 GB of memory from this Root Domain are the only memory resources available for the I/O Domains.

Because you have 224 GB of memory available for I/O Domains, you could create I/O Domains similar to the way you created database zones, where you could have 25 GB of memory assigned to each I/O Domain, for a total of 100 GB of memory for all four I/O Domains. The remaining 124 GB of memory could then be saved for additional I/O Domains that you might want to create in the future.

Assuming you wanted to allocate 25 GB of memory for each database zone and I/O Domain, saving the remaining memory resources for future database zones and I/O Domains, you would complete the table in this section in this manner in this situation:

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Total Amount of Memory |
|  | H1-1 |  |  |  |  |  |
|  | H2-1 |  |  |  |  |  |
|  | H3-1 |  |  |  |  |  |
| X | H4-1 | DB-Z: 256 GB - 4 nonglobal zones: 25 GB per zone | APP-S11: 256 GB | DB: 256 GB | ROOT: 256 GB - Root Domain: 32 GB set aside - 4 I/O Domains: 25 GB per I/O Domain | 1024 GB |
| NOTE: Total amount of memory must be 1024 GB, unless some resources are parked. |  |  |  |  |  |  |

Note – As described in General Configuration Rules on page 11, if you have a mixture of dedicated domains and Root Domains, after the initial installation, you can reallocate memory resources only with the dedicated domains. You cannot reallocate memory resources for Root Domains after the initial installation. 

Because resources allocated to Root Domains at the initial installation cannot be used by dedicated domains, carefully consider the amount of memory resources that you want to have allocated to Root Domains at the time of the initial installation. In addition, once you have parked memory resources from the dedicated domains, you cannot unpark them and reallocate them back to the dedicated domains after the initial installation.

Use the following tables to enter the memory resource allocation information for each compute server in your system:

- Compute Server 1 Memory Resource Allocation (Half Rack) on page 39

- Compute Server 2 Memory Resource Allocation (Half Rack) on page 39

Compute Server 1 Memory Resource Allocation (Half Rack)

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Total Amount of Memory |
|  | H1-1 |  |  |  |  |  |
|  | H2-1 |  |  |  |  |  |
|  | H3-1 |  |  |  |  |  |
|  | H4-1 |  |  |  |  |  |
| NOTE: Total amount of memory must be 1024 GB, unless some resources are parked. |  |  |  |  |  |  |

Compute Server 2 Memory Resource Allocation (Half Rack)

|  |  | Number and Type of Domains on Compute Server 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Total Amount of Memory |
|  | H1-1 |  |  |  |  |  |
|  | H2-1 |  |  |  |  |  |
|  | H3-1 |  |  |  |  |  |
|  | H4-1 |  |  |  |  |  |
| NOTE: Total amount of memory must be 1024 GB, unless some resources are parked. |  |  |  |  |  |  |

# CPU and Memory Resource Allocation for the Full Rack

For the Full Rack, each compute server would have four processor modules (PM0 through PM3), with two sockets or PCIe root complex pairs on each processor module, for a total of eight sockets or PCIe root complex pairs for each compute server. Following are the maximum CPU and memory resources available for the Full Rack:

- Eight sockets

- 128 cores

- 1024 VCPUs

- 2 TB (2048 GB) of memory

The amount of CPU and memory resources that you can allocate to each domain depends on the following things:

- The size of the domain (Giant Domain, Large Domain, Medium Domain, or Small Domain) on the compute server

- The size of the other domains that are also on that compute server and the number of sockets assigned to those other domains

The following guidelines describe the number of sockets that you can assign to each domain:

- Giant Domain: One domain is set up on the compute server in this configuration, taking up all of the server. 100% of the CPU and memory resources are allocated to this single domain on this server (all eight sockets).

Note - You can use the CPU/Memory tool (osc-setcoremem) to change this default allocation after the initial installation of your system, if you want to have some CPU or memory resources parked (unused). Refer to the Oracle SuperCluster T5-8 Owner’s Guide for more information.

- Large Domain: You can assign between 1 to 7 sockets for a Large Domain in each compute server, depending on the size of the other domains that are also on that compute server and the number of sockets assigned to those other domains.

- Medium Domain: You can assign between 1 to 4 sockets for a Medium Domain in each compute server, depending on the size of the other domains that are also on that compute server and the number of sockets assigned to those other domains.

- Small Domain: You can assign between 1 to 2 sockets for a Small Domain in each compute server, depending on the size of the other domains that are also on that compute server and the number of sockets assigned to those other domains.

You can allocate CPU and memory resources to the domains in either of the following ways:

- At a socket-based level, where between 1 and 8 sockets are assigned to each domain, based on the guidelines listed previously.

- At a granular level, where CPU resources are assigned based on the number of cores that you want assigned to the domain and memory resources are assigned based on the amount of memory, in GBs, that you want assigned to the domain.

Note that if you choose the socket-based level for one type of resource, then you will also have to choose the socket-based level for the other type of resource. For example, if you decided to assign two sockets to a domain for the CPU resources (out of eight, or 25%of the CPU resources). You would then have to also assign 25% of the memory resources, or 512 GB of memory, to this domain.

However, if you choose to assign resources at a granular level, you do not have to have the same proportions. For example, you could choose 64 cores (50% of the CPU resources) and 512 GB of memory (25% of the memory resources) for a domain in this case.

Provide the amount of CPU and memory resources that you want allocated to each domain in the following sections:

- CPU Resource Allocation for the Full Rack on page 41

- Memory Resource Allocation for the Full Rack on page 46

CPU Resource Allocation for the Full Rack

The total number of sockets that you have assigned to all the domains in each compute server in a Full Rack must add up to eight sockets, unless you decide to have some CPU resources parked.

In addition, the sockets can be broken down further to individual cores (16 cores per socket). The total number of cores that you have assigned to all domains in each compute server in a Full Rack must add up to 128 cores, unless you decide to have some CPU resources parked.

Use the tables in this section to provide information on the number of sockets or cores that you want to have assigned for each domain in each compute server in Oracle SuperCluster T5-8.

For example, assume you want the following configuration for the first compute server in your Full Rack:

- Config F4-1: Four domains (four Medium Domains)

- These types of domains:

- First Medium Domain: Database Domain, containing zones (DB-Z), with the Oracle installer setting up four database zones

- Second Medium Domain: Application Domain running the Oracle Solaris 11 OS (APP-S11)

- Third Medium Domain: Database Domain, where the Database Domain does not contain zones (DB)

- Fourth Medium Domain: Root Domain, with the Oracle installer setting up four I/O Domains

You could assign two sockets to each domain, which totals eight sockets altogether for all domains on the server.

In addition, the sockets can be broken down further to individual cores for the first domain, which would be a Database Domain (dedicated domain) that contains four zones. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, you would have the following cores available for that Database Domain (global zone) and the zones within that Database Domain (nonglobal zones):

- Database Domain: 4 cores set aside for global zone

- Zones within that Database Domain: 28 cores available for nonglobal zones

Because you have 28 cores available for the zones in that Database Domain, you could have 8 cores assigned to the first two zones (16 cores for both) and 6 cores assigned to the other two zones (12 cores for both), for a total of 28 cores. Or you could allocate a smaller number of available cores to each zone (for example, 4 cores to each zone, or 16 cores total) and save the remaining cores for future zones that you might want to create on that Database Domain.

Similarly, the sockets can be broken down further to individual cores for the fourth (last) domain, which would be a Root Domain with four I/O Domains configured off of that Root Domain. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, you would have the following cores available for the Root Domain and the I/O Domains:

- Root Domain: 4 cores set aside for the Root Domain

- I/O Domains: 28 remaining cores available for the I/O Domains

Note - Additional cores could be available for I/O Domains if cores from other domains were parked. For the purposes of this exercise, however, we are assuming that no other cores from other domains are parked, and the remaining 28 cores from this Root Domain are the only cores available for the I/O Domains.

Because you have 28 cores available for I/O Domains, you could create I/O Domains similar to the way you created database zones, where you could have 8 cores assigned to the first two I/O Domains (16 cores for both) and 6 cores assigned to the other two I/O Domains (12 cores for both), for a total of 28 cores. Or you could allocate a smaller number of available cores to each I/O Domain (for example, 4 cores to each I/O Domain, or 16 cores total) and save the remaining cores for future I/O Domains that you might want to create on that Root Domain.

Assuming you wanted to allocate 4 cores for each database zone and I/O Domain, saving the remaining cores for future database zones and I/O Domains, you would complete the table in this section in this manner in this situation:

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight | Total Number of Sockets or Cores |
|  | F1-1 |  |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |  |
| X | F4-1 | DB-Z: 2 sockets (32 cores) - Global zone: 4 cores set aside - 4 nonglobal zones: 4 cores per zone |  | APP-S11: 2 sockets (32 cores) |  | DB: 2 sockets (32 cores) |  | ROOT: 2 sockets (32 cores) - Root Domain: 4 cores set aside - 4 I/O Domains: 4 cores per I/O Domain |  | 8 sockets (128 cores) |
|  | F5-1 |  |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |  |
|  | F6-2 |  |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |  |
| NOTE: Total number of sockets for all domains must be 8, or total number of cores must be 128, unless some resources are parked. |  |  |  |  |  |  |  |  |  |  |

Note – As described in General Configuration Rules on page 11, if you have a mixture of dedicated domains and Root Domains, after the initial installation, you can reallocate CPU resources only with the dedicated domains. You cannot reallocate CPU resources for Root Domains after the initial installation. 

Because resources allocated to Root Domains at the initial installation cannot be used by dedicated domains, carefully consider the amount of CPU resources that you want to have allocated to Root Domains at the time of the initial installation. In addition, once you have parked CPU resources from the dedicated domains, you cannot unpark them and reallocate them back to the dedicated domains after the initial installation.

Use the following tables to enter the CPU resource allocation information for each compute server in your system:

- Compute Server 1 CPU Resource Allocation (Full Rack) on page 44

- Compute Server 2 CPU Resource Allocation (Full Rack) on page 45

Compute Server 1 CPU Resource Allocation (Full Rack)

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight | Total Number of Sockets or Cores |
|  | F1-1 |  |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |  |
| NOTE: Total number of sockets for all domains must be 8, or total number of cores must be 128, unless some resources are parked. |  |  |  |  |  |  |  |  |  |  |

Compute Server 2 CPU Resource Allocation (Full Rack)

|  |  | Number and Type of Domains on Compute Server 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight | Total Number of Sockets or Cores |
|  | F1-1 |  |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |  |
| NOTE: Total number of sockets for all domains must be 8, or total number of cores must be 128, unless some resources are parked. |  |  |  |  |  |  |  |  |  |  |

Memory Resource Allocation for the Full Rack

The total amount of memory that you have assigned to all domains in each compute server in a Full Rack must add up to 2 TB, or 2048 GB, unless you decide to have some memory resources parked.

Use the tables in this section to provide information on the amount of memory that you want to have assigned for each domain in each compute server in Oracle SuperCluster T5-8.

For example, assume you want the following configuration for the first compute server in your Full Rack:

- Config F4-1: Four domains (four Medium Domains)

- These types of domains:

- First Medium Domain: Database Domain, containing zones (DB-Z), with the Oracle installer setting up four database zones

- Second Medium Domain: Application Domain running the Oracle Solaris 11 OS (APP-S11)

- Third Medium Domain: Database Domain, where the Database Domain does not contain zones (DB)

- Fourth Medium Domain: Root Domain, with the Oracle installer setting up four I/O Domains

You could assign 512 GB of memory to each domain, which totals 2048 GB of memory altogether for all domains on this server.

In addition, the amount of memory used can be broken down further for the first domain, which would be a Database Domain (dedicated domain) that contains four zones. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, if 512 GB of memory is assigned to this Database Domain, you could have 50 GB of memory assigned to each database zone in this Database Domain, for a total of 200 GB of memory for all four database zones. The remaining 312 GB of memory in this Database Domain could then be saved for future database zones that you might want to create on this Database Domain.

Similarly, the amount of memory can be broken down further for the fourth (last) domain, which would be a Root Domain with four I/O Domains configured off of that Root Domain. Using the information provided in Cores and Memory Available for Database Zones and I/O Domains on page 27, if 512 GB of memory is assigned to this Root Domain, you would have the following memory available for the Root Domain and the I/O Domains:

- Root Domain: 64 GB of memory set aside for the Root Domain

- I/O Domains: 448 GB of remaining memory available for the I/O Domains

Note - Additional memory could be available for I/O Domains if memory resources from other domains were parked. For the purposes of this exercise, however, we are assuming that no other memory resources from other domains are parked, and the remaining 448 GB of memory from this Root Domain are the only memory resources available for the I/O Domains.

Because you have 448 GB of memory available for I/O Domains, you could create I/O Domains similar to the way you created database zones, where you could have 50 GB of memory assigned to each I/O Domain, for a total of 200 GB of memory for all four I/O Domains. The remaining 248 GB of memory could then be saved for additional I/O Domains that you might want to create in the future.

Assuming you wanted to allocate 50 GB of memory for each database zone and I/O Domain, saving the remaining memory resources for future database zones and I/O Domains, you would complete the table in this section in this manner in this situation:

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight | Total Amount of Memory |
|  | F1-1 |  |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |  |
| X | F4-1 | DB-Z: 512 GB - 4 nonglobal zones: 50 GB per zone |  | APP-S11: 512 GB |  | DB: 512 GB |  | ROOT: 512 GB - Root Domain: 64 GB set aside - 4 I/O Domains: 50 GB per I/O Domain |  | 2048 GB |
|  | F5-1 |  |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |  |
|  | F6-2 |  |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |  |
| NOTE: Total amount of memory must be 2048 GB, unless some resources are parked. |  |  |  |  |  |  |  |  |  |  |

Note – As described in General Configuration Rules on page 11, if you have a mixture of dedicated domains and Root Domains, after the initial installation, you can reallocate memory resources only with the dedicated domains. You cannot reallocate memory resources for Root Domains after the initial installation. 

Because resources allocated to Root Domains at the initial installation cannot be used by dedicated domains, carefully consider the amount of memory resources that you want to have allocated to Root Domains at the time of the initial installation. In addition, once you have parked memory resources from the dedicated domains, you cannot unpark them and reallocate them back to the dedicated domains after the initial installation.

Use the following tables to enter the memory resource allocation information for each compute server in your system:

- Compute Server 1 Memory Resource Allocation (Full Rack) on page 48

- Compute Server 2 Memory Resource Allocation (Full Rack) on page 49

Compute Server 1 Memory Resource Allocation (Full Rack)

|  |  | Number and Type of Domains on Compute Server 1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight | Total Amount of Memory |
|  | F1-1 |  |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |  |
| NOTE: Total amount of memory must be 2048 GB, unless some resources are parked. |  |  |  |  |  |  |  |  |  |  |

Compute Server 2 Memory Resource Allocation (Full Rack)

|  |  | Number and Type of Domains on Compute Server 2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Check One Box | Config | One | Two | Three | Four | Five | Six | Seven | Eight | Total Amount of Memory |
|  | F1-1 |  |  |  |  |  |  |  |  |  |
|  | F2-1 |  |  |  |  |  |  |  |  |  |
|  | F3-1 |  |  |  |  |  |  |  |  |  |
|  | F4-1 |  |  |  |  |  |  |  |  |  |
|  | F4-2 |  |  |  |  |  |  |  |  |  |
|  | F5-1 |  |  |  |  |  |  |  |  |  |
|  | F5-2 |  |  |  |  |  |  |  |  |  |
|  | F6-1 |  |  |  |  |  |  |  |  |  |
|  | F7-1 |  |  |  |  |  |  |  |  |  |
|  | F8-1 |  |  |  |  |  |  |  |  |  |
| NOTE: Total amount of memory must be 2048 GB, unless some resources are parked. |  |  |  |  |  |  |  |  |  |  |

What’s Next

Go to Completing the General Configuration Worksheets on page 50 to complete the general configuration worksheets.

Chapter

4

Completing the General Configuration Worksheets

Complete the configuration worksheets in this chapter to provide general configuration information for your Oracle SuperCluster T5-8.

# General Oracle SuperCluster T5-8 Configuration Information

When filling out the worksheets in this chapter, note the following items:

- Oracle SuperCluster T5-8 ships with the Oracle Solaris Operating System (Oracle Solaris OS) installed on the compute servers.

- The name (prefix) of Oracle SuperCluster T5-8 is used to generate host names for network interfaces for the components and logical domains in the system.

The name of Oracle SuperCluster T5-8 is completely user-definable, but because the name for Oracle SuperCluster T5-8 is used to generate the host names for the components listed above, you should use six characters or fewer for the name of Oracle SuperCluster T5-8. You will enter the name of Oracle SuperCluster T5-8 in the Customer Details Configuration  section on page 59 in this document.

Note – It is possible to create a name for Oracle SuperCluster T5-8 that is longer than six characters; however, you may get the following error message with a longer name for Oracle SuperCluster T5-8: Maximum combined length of cell short hostname + diskgroup name is too long - max length is 23 characters. The installer can manually shorten the disk group name to accommodate the combined maximum length of 23 characters in this case.

- For certain components, the company network domain name, such as example.com, is also used to generate host names for network interfaces for those components. The company network domain name is completely user-definable. The company network domain name is defined in the Operating System Configuration  section on page 61 in this document.

- The backup method information is used to size the ASM disk groups created during installation. The amount of usable disk space varies depending on the backup method. The backup methods are as follows:

- Backups internal to Oracle SuperCluster T5-8 mean database backups are created only on disk in the Fast Recovery Area (FRA). In addition to the database backups, there are other objects such as Archived Redo Logs and Flashback Log Files stored in the FRA. The division of disk space between the DATA disk group and the RECO disk group (the FRA) is 40% and 60%, respectively.

- Backups external to Oracle SuperCluster T5-8 mean database backups are created on disk or tape media that is external to currently deployed on Oracle SuperCluster T5-8s, and not on existing Exadata Storage Servers. If you are performing backups to disk storage external to Oracle SuperCluster T5-8, such as to additional dedicated Exadata Storage Servers, an NFS server, virtual tape library or tape library, then do no reserve additional space in the RECO disk group. When choosing this option, the FRA internal to Oracle SuperCluster T5-8 will contain objects such as archived redo log files and flashback log files. The division of disk space between the DATA disk group and the RECO disk group (the FRA) is 80% and 20%, respectively.

- A valid time zone name is required for Oracle SuperCluster T5-8 installation. Time zone data provided with Oracle SuperCluster T5-8 comes from the zoneinfo database. A valid time zone name is suitable as a value for the TZ environment variable consisting of form Area/Location. For example, a valid entry is America/New_York. Invalid entries are EST, EDT, UTC-5, and UTC-4. For a list of time zone names, refer to the zone.tab file in the zoneinfo database available in the public domain at ftp://elsie.nci.nih.gov/pub/.

- Use high redundancy disk groups for mission critical applications. The location of the backup files depends on the backup method. To reserve more space for the DATA disk group, choose external backups. This is especially important when the RECO disk group is high redundancy. The following table shows the backup options and settings.

| Description | Redundancy Level for DATA Disk Group | Redundancy Level for RECO Disk Group |
| --- | --- | --- |
| High Redundancy for ALL Both the DATA disk group and RECO disk group are configured with Oracle ASM high redundancy. The DATA disk group contains data files, temporary files, online redo logs, and control file. The RECO disk group contains archive logs, and flashback log files. | High | High |
| High Redundancy for DATA The DATA disk group is configured with Oracle ASM high redundancy, and the RECO disk group is configured with Oracle ASM normal redundancy. The DATA disk group contains data files, online redo logs, and control file. The RECO disk group contains archive logs, temporary files, and flashback log files. | High | Normal |
| High Redundancy for Log and RECO The DATA disk group is configured with Oracle ASM normal redundancy, and the RECO disk group is configured with Oracle ASM high redundancy. The DATA disk group contains the data files and temporary files. The RECO disk group contains online redo logs, control file, archive logs, and flashback log files. | Normal | High |
| Normal Redundancy The DATA Disk Group and RECO disk group are configured with Oracle ASM normal redundancy. The DATA disk group contains data files, temporary files, online redo logs, and control file. The RECO disk group contains online redo logs, archive logs, and flashback log files. | Normal | Normal |

See Oracle Exadata Storage Server Software User’s Guide for information about maximum availability.

The following sections provide information on the storage capacities for the Exadata Storage Servers, depending on the type of Exadata Storage Server installed in your SuperCluster system or expansion rack:

- Storage Capacities for Exadata Storage Servers Prior to X5-2 Release on page 54

- Storage Capacities for X5-2 Exadata Storage Servers on page 56

Storage Capacities for Exadata Storage Servers Prior to X5-2 Release

See the following tables for more information on storage capacities based on the level of redundancy that you choose:

- Table 1: Exadata Storage Server Storage Capacity in Oracle SuperCluster T5-8, High-Capacity Version

- Table 2: Exadata Storage Server Storage Capacity in Oracle SuperCluster T5-8, High-Performance Version

- Table 3: Exadata Storage Server Storage Capacity in the Oracle Exadata Storage Expansion Rack, High-Capacity Version

- Table 4: Exadata Storage Server Storage Capacity in the Oracle Exadata Storage Expansion Rack, High-Performance Version

| Table 1: Exadata Storage Server Storage Capacity in Oracle SuperCluster T5-8, High-Capacity Version |  |  |
| --- | --- | --- |
| Capacity Type | 3 TB Disks | 4 TB Disks |
| Raw disk capacity | - Half Rack: 144 TB - Full Rack: 288 TB | - Half Rack: 192 TB - Full Rack: 384 TB |
| Raw flash capacityError: Reference source not found | - Half Rack: 6.4 TB - Full Rack: 12.8 TB | - Half Rack: 12.8 TB - Full Rack: 25.6 TB |
| Usable mirrored capacity (ASM normal redundancy) | - Half Rack: 64 TB - Full Rack: 128 TB | - Half Rack: 85.3 TB - Full Rack: 170.6 TB |
| Usable triple mirrored capacity (ASM high redundancy) | - Half Rack: 43 TB - Full Rack: 86 TB | - Half Rack: 57.3 TB - Full Rack: 114.6 TB |

| Table 2: Exadata Storage Server Storage Capacity in Oracle SuperCluster T5-8, High-Performance Version |  |  |
| --- | --- | --- |
| Capacity Type | 600 GB Disks | 1.2 TB Disks |
| Raw disk capacityError: Reference source not found | - Half Rack: 28 TB - Full Rack: 56 TB | - Half Rack: 57 TB - Full Rack: 114 TB |
| Raw flash capacityError: Reference source not found | - Half Rack: 6.4 TB - Full Rack: 12.8 TB | - Half Rack: 12.8 TB - Full Rack: 25.6 TB |
| Usable mirrored capacity (ASM normal redundancy) | - Half Rack: 13 TB - Full Rack: 26 TB | - Half Rack: 26 TB - Full Rack: 52 TB |
| Usable triple mirrored capacity (ASM high redundancy) Error: Reference source not found | - Half Rack: 8 TB - Full Rack: 16 TB | - Half Rack: 16 TB - Full Rack: 32 TB |

Table 3: Exadata Storage Server Storage Capacity in the Oracle Exadata Storage Expansion Rack, High-Capacity Version

| Capacity Type | 3 TB Disks | 4 TB Disks |
| --- | --- | --- |
| Raw disk capacityError: Reference source not found | - Quarter Rack: 144 TB - Half Rack: 324 TB - Full Rack: 648 TB | - Quarter Rack: 192 TB - Half Rack: 432 TB - Full Rack: 864 TB |
| Raw flash capacityError: Reference source not found | - Quarter Rack: 6.4 TB - Half Rack: 14.4 TB - Full Rack: 28.8 TB | - Quarter Rack: 12.8 TB - Half Rack: 28.8 TB - Full Rack: 57.6 TB |
| Usable mirrored capacity (ASM normal redundancy) | - Quarter Rack: 64 TB - Half Rack: 144 TB - Full Rack: 288 TB | - Quarter Rack: 85.3 TB - Half Rack: 192 TB - Full Rack: 384 TB |
| Usable triple mirrored capacity (ASM high redundancy) | - Quarter Rack: 43 TB - Half Rack: 97 TB - Full Rack: 193.5 TB | - Quarter Rack: 57.3 TB - Half Rack: 129 TB - Full Rack: 258 TB |

Table 4: Exadata Storage Server Storage Capacity in the Oracle Exadata Storage Expansion Rack, High-Performance Version

| Capacity Type | 600 GB Disks | 1.2 TB Disks |
| --- | --- | --- |
| Raw disk capacityError: Reference source not found | - Quarter Rack: 28 TB - Half Rack: 64 TB - Full Rack: 128 TB | - Quarter Rack: 57 TB - Half Rack: 129 TB - Full Rack: 258 TB |
| Raw flash capacityError: Reference source not found | - Quarter Rack: 6.4 TB - Half Rack: 14.4 TB - Full Rack: 28.8 TB | - Quarter Rack: 12.8 TB - Half Rack: 28.8 TB - Full Rack: 57.6 TB |
| Usable mirrored capacity (ASM normal redundancy) | - Quarter Rack: 13 TB - Half Rack: 29 TB - Full Rack: 58 TB | - Quarter Rack: 26 TB - Half Rack: 58 TB - Full Rack: 116 TB |
| Usable triple mirrored capacity (ASM high redundancy) | - Quarter Rack: 8 TB - Half Rack: 19 TB - Full Rack: 39 TB | - Quarter Rack: 16 TB - Half Rack: 39 TB - Full Rack: 78 TB |

Storage Capacities for X5-2 Exadata Storage Servers

The X5-2 Exadata Storage Server differs from previous versions of the Exadata Storage Servers in the following areas:

- The storage servers are available with either Extreme Flash or High Capacity storage.

- The expansion rack is available as a quarter rack, with four Exadata Storage Servers. You can increase the number of storage servers in the expansion rack up to a maximum of 18 storage servers.

See the following tables for more information on storage capacities based on the level of redundancy that you choose:

- Table 5: X5-2 Exadata Storage Server Storage Capacity, Extreme Flash Version

- Table 6: X5-2 Exadata Storage Server Storage Capacity, High Capacity Version

Table 5: X5-2 Exadata Storage Server Storage Capacity, Extreme Flash Version

| Capacity Type | Quarter Expansion Rack, With 4 Storage Servers | Single Exadata Storage Server |
| --- | --- | --- |
| Raw PCI flash capacity | 51.2 TB | 12.8 TB |
| Raw disk capacityError: Reference source not found | N/A | N/A |
| Usable mirrored capacity (ASM normal redundancy) | 23 TB | 5 TB |
| Usable triple mirrored capacity (ASM high redundancy) | 16 TB | 4.3 TB |

Table 6: X5-2 Exadata Storage Server Storage Capacity, High Capacity Version

| Capacity Type | Quarter Expansion Rack, With 4 Storage Servers | Single Exadata Storage Server |
| --- | --- | --- |
| Raw PCI flash capacityError: Reference source not found | 25.6 TB | 6.4 TB |
| Raw disk capacityError: Reference source not found | 192 TB | 48 TB |
| Usable mirrored capacity (ASM normal redundancy) | 85 TB | 20 TB |
| Usable triple mirrored capacity (ASM high redundancy) | 58 TB | 15 TB |

# General Rack Configuration Worksheet

Table 7: General Rack Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Use Client Hostnames? |  | Every Oracle Solaris domain on Oracle SuperCluster T5-8 has a hostname. By default, the hostname given is the same name associated with the management network interface. But the hostname can also be the set to the name associated with the 10GbE client network interface. Determine if you want to use the client interface hostname as the hostname for all Oracle Solaris domains. For example, you may choose to use the client interface hostnames if your applications require that the hostname match the interface over which the clients connect. - Select Yes if you want to have the client interface hostname as the hostname for all Oracle Solaris domains. - Select No if you want the default management hostname as the hostname for all Oracle Solaris domains. Options: Yes or No Default option is No. |
| Database Client Standalone Network |  | Determine if all the Database Domains will use a different client network from the Application Domains. Options: Yes or No |
| Number of Oracle RAC Instances |  | Determine how many RAC instances are needed. Note that a minimum of 2 RAC instances are required if Database Domains and zones on Database Domains are being deployed. Options: 1 – 16 RAC instances |
| Exadata Storage Server Type |  | The type of Exadata Storage Servers that you have in your Oracle SuperCluster T5-8. Options: High Performance or High Capacity |

# Customer Details Configuration Worksheet

Table 8: Customer Details Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Customer name |  | The customer name. The name can contain any alphanumeric characters, including spaces. This field cannot be empty. |
| Application |  | The application that will be used on the domains. |
| Region |  | Country where Oracle SuperCluster T5-8 will be installed. Example: United States |
| Time zone |  | Time zone name where Oracle SuperCluster T5-8 will be installed. Example: America/Los_Angeles |
| Compute OS | Oracle Solaris | The operating system for the domains on Oracle SuperCluster T5-8. Oracle Solaris is the only valid entry for this field, even if you have Database Domains in your Oracle SuperCluster T5-8. |
| Oracle SuperCluster T5-8 prefix |  | The prefix is used to generate host names for network interfaces for components in the system. For example, a value of sc01 results in a compute node host name of sc01db01, and an Exadata Storage Server host name of sc01cel01. Because this is used to generate host names for network interfaces for components in the system, Oracle recommends a name of fewer than six characters for the prefix. Example: sc01 |

# Backup/Data Guard Ethernet Network Configuration Worksheet

Table 9: Backup/Data Guard Ethernet Network Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Enable backup/Data Guard network |  | Determine if a backup network is being used for this system. Options: Enabled or Disabled. |
| Starting IP Address for Pool |  | This is the starting IP address for the IP addresses assigned to the backup network. Note: The pool should consist of consecutive IP addresses. If consecutive IP addresses are not available, then specific IP addresses can be modified during the configuration process. |
| Pool Size |  | The value of this field is defined by the type of Oracle SuperCluster T5-8 (Half Rack or Full Rack). |
| Ending IP Address for Pool |  | The value of this field is defined by the starting IP address and the pool size. |
| Subnet Mask |  | The subnet mask for the backup network. |
| Gateway |  | The gateway for the subnet. Ensure that the defined IP address is correct for the gateway. |
| Adapter Speed |  | The speed of the Ethernet cards. The options are 1 GbE/10 GbE Base-T when using copper cables, or 10 GbE SFP+ optical when using fiber optic cables. |
| Implement host based bonded network |  | This option is selected when using a bonded network. Options: Enabled or Disabled |

# Operating System Configuration Worksheet

Table 10: Custom Details Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Domain name |  | The company network domain name, such as example.com. The name can contain alphanumeric characters, periods (.), and hyphens (-). The name must start with an alphanumeric character. This field cannot be empty. |
| DNS servers |  | The IP address for the domain name servers. At least one IP address must be provided. |
| NTP servers |  | The IP address for the Network Time Protocol servers. At least one IP address must be provided. |
| Separate Grid Infrastructure owner from Database owner |  | Determine if the responsibilities and privileges are separated by role. Providing system privileges for the storage tier using the SYSASM privilege instead of the SYSDBA privilege provides a clear division of responsibility between Oracle ASM administration and database administration. Role separation also helps to prevent different databases using the same storage from accidentally overwriting each other's files. Options: Selected or Unselected. |
| Grid ASM Home OS User |  | The user name for the Oracle ASM owner. The default is grid. This user owns the Oracle Grid Infrastructure installation. This option is available when using role-separated authentication. |
| Grid ASM Home OS UserId |  | The identifier for the Oracle ASM owner. The default is 1000. This option is available when using role-separated authentication. |
| Grid ASM Home Base Location |  | The directory for the Oracle grid infrastructure. The default is /u01/app/grid. This option is available when using role-separated authentication. |
| ASM DBA Group |  | The name for the Oracle ASM DBA group. The default is asmdba. Membership in this group enables access to the files managed by Oracle ASM. This option is available when using role-separated authentication. |
| ASM DBA GroupID |  | The identifier for the Oracle ASM DBA group. The default is 1004. This option is available when using role-separated authentication. |
| ASM Home Operator Group |  | The name for the Oracle ASM operator group. The default is asmoper. This group of operating system users has a limited set of Oracle instance administrative privileges including starting up and stopping the Oracle ASM instance. This option is available when using role-separated authentication. |
| ASM Home Operator GroupId |  | The identifier for the Oracle ASM operator group. The default is 1005. This option is available when using role-separated authentication. |
| ASM Home Admin Group |  | The name for the Oracle ASM administration group. The default is asmadmin. This group uses SQL to connect to an Oracle ASM instance as SYSASM using operating system authentication. The SYSASM privileges permit mounting and dismounting of disk groups, and other storage administration tasks. SYSASM privileges provide no access privileges on an Oracle Database instance. This option is available when using role-separated authentication. |
| ASM Home Admin GroupId |  | The identifier for the Oracle ASM administration group. The default is 1006. This option is available when using role-separated authentication. |
| RDBMS Home OS User |  | The user name for the owner of the Oracle Database installation. The default is oracle. |
| RDBMS Home OS UserId |  | The identifier for the owner of the Oracle Database installation. The default is 1001. |
| RDBMS Home Base Location |  | The directory for the Oracle Database installation. The default is /u01/app/oracle. |
| RDBMS DBA Group |  | The name for the database administration group. The default is 1002. |
| RDBMS Home Operator Group |  | The name for the Oracle Database operator group. The default is racoper. |
| RDBMS Home Operator GroupId |  | The identifier for the Oracle Database operator group. The default is 1003. |
| Oinstall Group |  | The name for the Oracle Inventory group. The default is oinstall. |
| Oinstall GroupId |  | The identifier for the Oracle Inventory group. The default is 1001. |

# Home and Database Configuration Worksheet

Use this worksheet to provide information on the home and database configuration. The disk group sizes shown in the configuration page are approximate, based on the type of Oracle SuperCluster T5-8, and redundancy.

For more information about disk group redundancy and backups, see General Oracle SuperCluster T5-8 Configuration Information on page 50.

Table 11: Home and Database Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Inventory Location |  | The directory path for the Oracle inventory (oraInventory). The default is /u01/app/oraInventory. |
| Grid Infrastructure Home |  | The directory path for the Grid infrastructure. The default is /u01/app/release_number/grid. |
| Database Home Location |  | The directory path for Oracle Database. The default is /u01/app/oracle/product/release_number/dbhome_1. |
| Software Install Languages |  | The language abbreviation for the languages installed for the database. The default is English (en). |
| DATA Disk Group Name |  | The name of the DATA disk group. The default is DATA_DM01. |
| DATA Disk Group Redundancy |  | The type of redundancy for the DATA disk group. The options are NORMAL and HIGH. Use HIGH redundancy disk groups for mission critical applications. |
| RECO Disk Group Name |  | The name of the RECO disk group. The default is RECO_DM01. |
| RECO Disk Group Redundancy |  | The type of redundancy for the RECO disk group. The options are NORMAL and HIGH. Use HIGH redundancy disk groups for mission critical applications. |
| Reserve additional space in RECO for database backups |  | Determine if the backups will occur within Oracle SuperCluster T5-8. When backups occur within Oracle SuperCluster T5-8, the RECO disk group size increases, and the DATA disk group size decreases. Options: Selected or Unselected |
| Database name |  | The name of the database. The default is dbm. |
| Block size |  | The block size for the database. The default is 8192. Options are: - 4096 - 8192 - 16384 - 32768 |
| Database Type |  | The type of workload that will mainly run on the database. The options are OLTP for online transaction processing, and DW for data warehouse. |

# (Optional) Cell Alerting Configuration Worksheet

Cell alerts can be delivered by way of Simple Mail Transfer Protocol (SMTP), Simple Network Management Protocol (SNMP), or both. You can configure the cell alert delivery during or after installation.

Table 12: Cell Alerting Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Enable Email Alerting |  | If cell alerts should be delivered automatically, then select this option. |
| Recipients Addresses |  | The email addresses for the recipients of the cell alerts. You can enter multiple addresses in the dialog box. The number of email addresses is shown. |
| SMTP Server |  | The SMTP email server used to send alert notifications, such as mail.example.com |
| Uses SSL |  | Specification to use Secure Socket Layer (SSL) security when sending alert notifications. |
| Port |  | The SMTP email server port used to send alert notifications, such as 25 or 465. |
| Name |  | The SMTP email user name that is displayed in the alert notifications, such as Oracle SuperCluster T5-8. |
| Email Address |  | The SMTP email address that sends the alert notifications, such as dm01@example.com. |
| Enable SNMP Alerting |  | Determine if alerts will be delivered using SNMP. Options: Enabled or Disabled |
| SNMP Server |  | The host name of the SNMP server, such as snmp.example.com. Note: You can define additional SNMP targets after installation. Refer to Oracle Exadata Storage Server Software User's Guide for information. |
| Port |  | The port for the SNMP server. The default port is 162. |
| Community |  | The community for the SNMP server. The default is public. |

# (Optional) Oracle Configuration Manager Configuration Worksheet

Use the Oracle Configuration Manager to collect configuration information and upload it to the Oracle repository.

Table 13: Oracle Configuration Manager Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Enable Oracle Configuration Manager |  | Determine if Oracle Configuration Manager will be used to collect configuration information. Options: Enabled or Disabled |
| Receive updates via MOS |  | Determine if you are planning to receive My Oracle Support updates automatically for Oracle SuperCluster T5-8. Options: Enabled or Disabled |
| MOS Email Address |  | The My Oracle Support address to receive My Oracle Support updates. |
| Access Oracle Configuration Manager via Support Hub |  | Determine if you are planning to access Oracle Configuration Manager using Support Hub. Oracle Support Hub enables Oracle Configuration Manager instances to connect to a single internal port (the Support Hub), and upload configuration data, eliminating the need for each individual Oracle Configuration Manager instance in the database servers to access the Internet. Options: Enabled or Disabled |
| Support Hub Hostname |  | The host name for Support Hub server. See Also: Oracle Configuration Manager Companion Distribution Guide |
| Hub User name |  | The operating system user name for the Support Hub server. |
| HTTP Proxy used in upload to Oracle Configuration Manager |  | Determine if an HTTP proxy will be used to upload configuration information to the Oracle repository. Options: Enabled or Disabled |
| HTTP Proxy Host |  | The proxy server to connect to Oracle. The proxy can be between the following: - Database servers and Oracle - Database servers and Support Hub - Support Hub and Oracle Example: [proxy_user@]proxy_host[:proxy_port] The proxy_host and proxy_port entries are optional. Note: If passwords are needed, then provide them during installation. |
| Proxy Port |  | The port number for the HTTP proxy server. The default is 80. |
| HTTP Proxy requires authentication |  | Determine if the HTTP proxy requires authentication. Options: Enabled or Disabled |
| HTTP Proxy User |  | The user name for the HTTP proxy server. |

# Auto Service Request Configuration Worksheet

You can install and configure Auto Service Request (ASR) for use with Oracle SuperCluster T5-8.

Table 14: Auto Service Request Configuration Worksheet

| Item | Entry | Description and Example |
| --- | --- | --- |
| Enable Auto Service Request |  | Enable ASR for use with Oracle SuperCluster T5-8. The default is yes. |
| ASR Manager Host name |  | The host name of the server for ASR. Note: You should use a standalone server that has connectivity to Oracle SuperCluster T5-8. |
| ASR Technical Contact |  | The name of the person responsible as the technical contact for Oracle SuperCluster T5-8. |
| Technical Contact Email |  | The email address of the person responsible as technical contact for Oracle SuperCluster T5-8. |
| My Oracle Support Account Name |  | The name for the My Oracle Support account. |
| HTTP Proxy used in upload to ASR. |  | Determine if an HTTP proxy will be used to upload ASR. Options: Enabled or Disabled |
| HTTP Proxy Host |  | The host name of the proxy server. |
| Proxy Port |  | The port number for the HTTP proxy server. Default: 80. |
| HTTP Proxy requires authentication |  | Determine if the HTTP proxy server requires authentication. Options: Enabled or Disabled |
| HTTP Proxy User |  | The user name used with the proxy server. |

What’s Next

Go to Determining Network IP Addresses on page 69 to provide starting IP addresses and IP address ranges for the three networks for your system.

Chapter

5

Determining Network IP Addresses

Use the configuration worksheets in this chapter to determine the total number of IP addresses that you will need for the three networks on your system:

- Management network

- Client access network

- InfiniBand network

See Networks on page 7 for more information on the three networks. Also, see Configuration Process on page 8 for more information on how you will work with your Oracle installer to generate your site-specific Installation Template after you have completed the worksheets in this chapter.

Read and understand the information on IP addresses and Oracle Enterprise Manager Ops Center 12c Release 2 (12.2.0.0.0), then complete the configuration worksheets in this chapter to provide the starting IP address and to determine the total number of IP addresses that you will need for the three networks on your system:

- IP Addresses and Oracle Enterprise Manager Ops Center 12c Release 2 on page 70

- Management Network IP Addresses on page 72

- Client Access Network IP Addresses on page 76

- InfiniBand Network IP Addresses on page 87

# IP Addresses and Oracle Enterprise Manager Ops Center 12c Release 2

For previous versions of Oracle Enterprise Manager Ops Center, the Ops Center software was installed and run from the SuperCluster system. Beginning with the Oracle Enterprise Manager Ops Center 12c Release 2 (12.2.0.0.0) release, the Ops Center software must now run on a system (Enterprise Controller host) outside of the SuperCluster system.

The following conditions apply to Oracle Engineered Systems, such as SuperCluster systems.

One or more Oracle Engineered Systems can be discovered and managed by a single Oracle Enterprise Manager Ops Center instance based on the following conditions:

- None of Oracle Engineered System instances have overlapping private networks connected through IB, that is, networks that have the same CIDR (Classless Inter-Domain Routing) or networks that are sub-blocks of the same CIDR. For example, 192.0.2.1/21 and 192.0.2.1/24 are overlapping.

- None of the Oracle Engineered System instances or generic datacenter assets have overlapping management or client access networks connected through Ethernet, that is, networks that have the same CIDR or networks that are sub-blocks of the same CIDR. For example, 192.0.2.1/21 and 192.0.2.1/24 are overlapping. As an exception, you can use the same CIDR (not sub-block) for multiple systems. For example, you can use 192.0.2.1/22 as a CIDR for Ethernet network on one or more engineered systems and/or generic datacenter assets.

- None of the Oracle Engineered System instances have overlapping public networks connected through EoIB, that is, networks that have the same CIDR or networks that are sub-blocks of the same CIDR. For example, 192.0.2.1/21 and 192.0.2.1/24 are overlapping. As an exception, you can use the same CIDR (not sub-block) for multiple systems. For example, you can use 192.2.0.0/22 as a CIDR for public EoIB network on multiple engineered systems.

- None of the networks configured in Oracle Enterprise Manager Ops Center overlaps with any network, that is, overlapping networks are not supported by Oracle Enterprise Manager Ops Center.

Note – To manage two or more Oracle Engineered Systems that have overlapping networks or any networks already present in Oracle Enterprise Manager Ops Center, reconfigure one of the conflicting systems before it is discovered and managed by the same Oracle Enterprise Manager Ops Center.

Example Oracle SuperCluster Network Configurations

The following are example Oracle SuperCluster network configurations that you can use when configuring the network to discover and manage Oracle SuperCluster systems. Status OK indicates a valid configuration and status Fail indicates an invalid configuration.

Table 15: Example SuperCluster Network Configuration 1

|  | 1GbE | 10GbE | IB |
| --- | --- | --- | --- |
| First SuperCluster System | 192.0.251.0/21 | 192.4.251.0/24 | 192.168.8.0/22 |
| Second SuperCluster System | 192.0.251.0/21 | 192.4.251.0/24 | 192.168.12.0/22 |
| Status | OK | OK | OK |

Status:

OK – First SuperCluster system 1GbE and second SuperCluster system 1GbE share the same network.

OK – First SuperCluster system 10GbE and second SuperCluster system 10GbE share the same network.

OK – First SuperCluster system IB does not overlap with second SuperCluster system IB.

Table 16: Example SuperCluster Network Configuration 2

|  | 1 GbE | 10 GbE | IB |
| --- | --- | --- | --- |
| First SuperCluster System | 192.0.251.0/21 | 192.0.250.0/24 | 192.168.8.0/22 – IB fabric connected with second SuperCluster system |
| Second SuperCluster System | 192.6.0.0/21 | 192.0.250.0/24 | 192.168.8.0/22– IB fabric connected with first SuperCluster system |
| Status | OK | OK | OK |

Status:

OK – First SuperCluster system 1GbE and second SuperCluster system 1GbE represent different non-overlapping networks.

OK – First SuperCluster system 10GbE and second SuperCluster system 10GbE share the same network.

OK – First SuperCluster system IB and second SuperCluster system IB represent the same network as they are interconnected.

Table 17: Example SuperCluster Network Configuration 3

|  | 1 GbE | 10 GbE | IB |
| --- | --- | --- | --- |
| First SuperCluster System | 192.0.2.1/21 | 192.0.251.0/21 | 192.168.8.0/22 |
| Second SuperCluster System | 192.0.0.128/25 | 192.0.7.0/24 | 192.168.8.0/22 |
| Status | FAIL | OK | FAIL |

Status:

FAIL – First SuperCluster system 1GbE and second SuperCluster system 1GbE define overlapping networks.

OK – First SuperCluster system 10GbE and second SuperCluster system 10GbE represent different non-overlapping networks.

FAIL – First SuperCluster system 1GbE and second SuperCluster system 10GbE define overlapping networks.

FAIL – First SuperCluster system IB and second SuperCluster system IB do not define unique private networks (racks are not interconnected).

# Management Network IP Addresses

You will need management network IP addresses for the following components in Oracle SuperCluster T5-8:

- One 1GbE host management IP address for every dedicated domain (Database Domain or Application Domain) and Root Domain in each compute server:

- 2 to 8 IP addresses for a Half Rack (1-4 IP addresses per compute server)

- 2 to 16 IP addresses for a Full Rack (1-8 IP addresses per compute server)

- One 1GbE host management IP address for every database zone in a Database Domain that will be set up by your Oracle installer

- One 1GbE host management IP address for every I/O Domain that will be set up by your Oracle installerError: Reference source not found

- One 1GbE host management IP address for each of the following components:

- Cisco Catalyst switch

- InfiniBand switches (3)

- PDUs (2)

- Exadata Storage Servers:

- 4 for a Half Rack

- 8 for a Full Rack

- ZFS storage controllers (2)

- One ILOM IP address for each of the following components:

- Compute servers (2)

- Exadata Storage Servers:

- 4 for a Half Rack

- 8 for a Full Rack

- ZFS storage controllers (2)

Use the worksheets in this section to provide the starting IP address and to determine the total number of IP addresses that you will need for the management network for your system.

General Management Network Configuration Worksheet

Use this worksheet to provide the subnet mask and gateway IP address for the management network.

| Item | Entry | Description and Example |
| --- | --- | --- |
| Subnet mask |  | Subnet mask for the management network. Example: 255.255.255.0 |
| Gateway IP address |  | Gateway IP address for the management network. Example: 10.204.74.1 |
| Use management network gateway for default gateway? |  | For the default gateway, you can use either the management network gateway or the client access network gateway. Options for this field: - Yes if the management network gateway will be the default gateway - No if the management network gateway will not be the default gateway (if the client access network gateway will be the default gateway) |
| Starting IP address |  | Starting IP address for the management network. Example: 10.204.74.100 |

Management Network IP Addresses Configuration Worksheet

Complete this worksheet to determine the total number of IP addresses needed for the management network. These IP addresses should be sequential, beginning with the starting IP address that you provided in General Management Network Configuration Worksheet on 73.

Note – It is preferable to have all the IP addresses on this network in sequential order. If you cannot set aside the appropriate number of sequential IP addresses for this network, and you must break the IP addresses into non-sequential addresses, the Oracle installer can break the IP addresses on this network into non-sequential blocks. However, this will make the information in the Installation Template more complex, and will require additional communication between you and your Oracle representative to ensure that the non-sequential IP addresses are correctly assigned to the appropriate components or domains in the system.

Note that you will be asked for the total number of domains and database zones on each server. So, for example, if you have four domains in the compute server:

- One Database Domain without zones

- One Application Domain running the Oracle Solaris 10 OS

- One Database Domain containing zones, with the Oracle installer setting up four database zones

- One Root Domain, with the Oracle installer setting up four I/O Domains

In this situation, you would enter 12 as the total number of IP addresses for this network for this server, which covers the following:

- Three dedicated domains and one Root Domain (4)

- Four database zones

- Four I/O Domains

Note – Even though your Oracle installer can configure up to eight database zones or I/O Domains during the initial installation of your Oracle SuperCluster, keep in mind that you will be able to configure additional database zones and I/O Domains after the initial installation, so additional IP addresses might be needed for the management network for these future configurations you set up. Do not provide that information in this table, but keep this in mind so that you can plan accordingly for the total number of IP addresses needed for the management network for the future.

| Item | Entry |
| --- | --- |
| Do you have a Half Rack or a Full Rack? - Half Rack: Enter 20. - Full Rack: Enter 28. |  |
| For compute server 1, how many domains (dedicated domains, Root Domains, and I/O Domains) and database zones will the Oracle installer configure on this server? Enter the total number of domains and database zones on this server. |  |
| For compute server 2, how many domains (dedicated domains, Root Domains, and I/O Domains) and database zones will the Oracle installer configure on this server? Enter the total number of domains and database zones on this server. |  |
| Add the entries from the Entry column. This is the total number of IP addresses that you will need for the management network. |  |
| What’s Next: Go to Client Access Network IP Addresses on page 76 to enter information for the client access network for your system. |  |

# Client Access Network IP Addresses

You will need client access network IP addresses for the following components in Oracle SuperCluster T5-8:

- One 10GbE client access IP address for every dedicated domain (Database Domain or Application Domain) in each compute server:

- 2 to 8 IP addresses for a Half Rack (1-4 IP addresses per compute server)

- 2 to 16 IP addresses for a Full Rack (1-8 IP addresses per compute server)

Note that you do not need 10GbE client access network IP addresses for any Root Domains in the servers.

- One 10GbE client access IP address for every database zone in a Database Domain that will be set up by your Oracle installer

- One 10GbE client access IP address for every I/O Domain that will be set up by your Oracle installerError: Reference source not found

- 10GbE client access IP addresses for Oracle RAC VIP and SCAN for every Database Domain (either dedicated domain or Database I/O Domain) and database zone that are part of a RAC:

- One Oracle RAC VIP address for each Database Domain (either dedicated domain or Database I/O Domain) that is part of a RAC

- One Oracle RAC VIP address for every database zone within a Database  Domain (dedicated domain) that is part of a RAC

- Three SCAN IP addresses for each Oracle RAC in your Oracle SuperCluster T5-8

Use the worksheets in this section to determine the total number of IP addresses that you will need for the client access network for your system.

Understanding the Physical Connections for the Client Access Network

A 10GbE client access network infrastructure is a required part of the installation process for Oracle SuperCluster T5-8.

Oracle SuperCluster T5-8 ships with the following components:

- Half Rack:

- Four dual-ported Sun Dual 10GbE SFP+ PCIe NICs in each  compute server, with a total of 16 10GbE SFP+ ports

- Transceivers preinstalled in the 10GbE NICs

- Four 10-meter SFP-QSFP optical splitter cables

- Full Rack:

- Eight dual-ported Sun Dual 10GbE SFP+ PCIe NICs in each  compute server, with a total of 32 10GbE SFP+ ports

- Transceivers preinstalled in the 10GbE NICs

- Eight 10-meter SFP-QSFP optical splitter cables

If you plan to use the supplied SFP-QSFP cables for the connection to your client access network, you must provide the following 10GbE client access network infrastructure components:

- A 10GbE switch with QSFP connections, such as the Sun Network 10GbE Switch 72p

- Four QSFP transceivers for a Half Rack or eight QSFP transceivers for a Full Rack  to connect the QSFP end of the supplied SFP-QSFP cable to your 10GbE switch

If you do not want to use the supplied SFP-QSFP cables for the connection to your client access network, you must provide the following 10GbE client access network infrastructure components:

- A 10GbE switch

- Suitable optical cables with SFP+ connections for the compute server side

- Suitable transceivers to connect all cables to your 10GbE switch

If you do not have a 10GbE client access network infrastructure set up at your site, you must have a 10GbE network switch available at the time of installation that Oracle SuperCluster T5-8 can be connected to, even if the network speed drops from 10 Gb to 1 Gb on the other side of the 10GbE network switch. Oracle SuperCluster T5-8 cannot be installed at the customer site without the 10GbE client access network infrastructure in place.

Client Access Network IP Addresses Configuration Worksheets

There are two options available for the client access network for your Oracle SuperCluster:

- Client access network for the entire Oracle SuperCluster T5-8 on a single subnet. All Database Domains and Application Domains would be on the same subnet with this option.

- Client access network for the entire Oracle SuperCluster T5-8 on two different subnets. For this option, client access to the Database Domains on all the compute servers in Oracle SuperCluster T5-8 would be on one subnet, and client access to the Application Domains on all the compute servers would be on a second, separate subnet. Two subnets on the client access network would be needed for this option.

Note – It is preferable for the IP addresses on each subnet of this network to be in sequential order. If you cannot set aside the appropriate number of sequential IP addresses on each subnet for this network, and you must break the IP addresses into non-sequential addresses, the Oracle installer can break the IP addresses on this network into non-sequential blocks. However, this will make the information in the Installation Template more complex, and will require additional communication between you and your Oracle representative to ensure that the non-sequential IP addresses are correctly assigned to the appropriate components or domains in the system.

Note that you will be asked for the total number of domains and database zones on each server. So, for example, if you have four domains in the compute server 1:

- One Database Domain without zones

- One Application Domain running the Oracle Solaris 10 OS

- One Database Domain containing zones, with the Oracle installer setting up four database zones

- One Root Domain, with the Oracle installer setting up four I/O Domains

In this situation, you would enter 11 for the total number of IP addresses for this network for this server, which covers the following:

- Three dedicated domains (note that a client access network IP address is not needed for any Root Domain)

- Four database zones

- Four I/O Domains

Note – Even though your Oracle installer can configure up to eight database zones or I/O Domains during the initial installation of your Oracle SuperCluster, keep in mind that you will be able to configure additional database zones and I/O Domains after the initial installation, so additional IP addresses might be needed for the 10GbE client access network for these future configurations you set up. Do not provide that information in these tables, but keep this in mind so that you can plan accordingly for the total number of IP addresses needed for the 10GbE client access network for the future.

Refer to the appropriate section to determine the number of IP addresses you will need, and the number of subnets, depending on the choice you made from the options listed above:

- Client Access for Entire Oracle SuperCluster T5-8 on a Single Subnet on page 80

- Client Access for Entire Oracle SuperCluster T5-8 on Two Separate Subnets on page 82

Client Access for Entire Oracle SuperCluster T5-8 on a Single Subnet

Use this worksheet to provide subnet mask and gateway IP addresses for the client access network for the entire Oracle SuperCluster T5-8.

For this option, client access network for the entire Oracle SuperCluster T5-8 (Database Domains and Application Domains) will be on a single subnet.

| Item | Entry | Description and Example |
| --- | --- | --- |
| Subnet mask |  | Subnet mask for client access network. Example: 255.255.252.0 |
| Gateway IP address |  | Gateway IP address for the client access network. Example: 172.16.8.1 |
| Use client access network gateway for default gateway? |  | For the default gateway, you can use either the management network gateway or the client access network gateway. Options for this field: - Yes if the client access network gateway will be the default gateway - No if the client access network gateway will not be the default gateway (if the management network gateway will be the default gateway) |
| Starting IP address |  | Starting IP address for the client access network. Example: 172.16.8.100 |
| VLAN tag (optional) |  | If this network belongs to a tagged VLAN, enter the VLAN tag ID. Example: 101 |

Complete the following worksheet to determine the total number of IP addresses you will need for the client access network.

| Item | Entry |
| --- | --- |
| For compute server 1, how many dedicated domains, I/O Domains and database zones are on this server? Enter the total number of dedicated domains, I/O Domains and database zones on this server so that one 10GbE client access IP address is assigned to each domain and database zone in the server. |  |
| For compute server 2, how many dedicated domains, I/O Domains and database zones are on this server? Enter the total number of dedicated domains, I/O Domains and database zones on this server so that one 10GbE client access IP address is assigned to each domain and database zone in the server. |  |
| For compute server 1, how many of the following are part of a RAC: - Database Domains (dedicated domains or Database I/O Domains) - Database zones Enter the total number of Database Domains and database zones on this server that are part of a RAC so that one RAC VIP address is assigned to each Database Domain and database zone in the server. |  |
| For compute server 2, how many of the following are part of a RAC: - Database Domains (dedicated domains or Database I/O Domains) - Database Domains zones Enter the total number of Database Domains and database zones on this server that are part of a RAC so that one RAC VIP address is assigned to each Database Domain and database zone in the server. |  |
| How many Oracle RACs will there be altogether within your Oracle SuperCluster T5-8? Enter the total number of Oracle RACs that will be in your Oracle SuperCluster T5-8 times 3 so that three SCAN IP addresses are assigned to each RAC. For example, if you have four Oracle RACs in your Oracle SuperCluster T5-8, enter 12 in this field. |  |
| Add the entries from the Entry column. This is the total number of IP addresses that you will need for the client access network for this server. |  |
| What’s Next: Go to InfiniBand Network IP Addresses on page 87. |  |

Client Access for Entire Oracle SuperCluster T5-8 on Two Separate Subnets

For this option, client access network for the entire Oracle SuperCluster T5-8 will be on two separate subnets:

- Client access to the Database Domains on all the compute servers in Oracle SuperCluster T5-8 on one subnet

- Client access to the Application Domains on all the compute servers would be on a second, separate subnet.

Client Access Information for Database Domains

Use the following worksheet to provide subnet mask and gateway IP addresses for the client access network for the Database Domains for all the compute servers in Oracle SuperCluster T5-8.

| Item | Entry | Description and Example |
| --- | --- | --- |
| Subnet mask |  | Subnet mask for client access network for the Database Domains. Example: 255.255.252.0 |
| Gateway IP address |  | Gateway IP address for the client access network for the Database Domains. Example: 172.16.8.1 |
| Use client access network gateway for default gateway? |  | For the default gateway, you can use either the management network gateway or the client access network gateway. Options for this field: - Yes if the client access network gateway will be the default gateway - No if the client access network gateway will not be the default gateway (if the management network gateway will be the default gateway) |
| Starting IP address |  | Starting IP address for the client access network for the Database Domains. Example: 172.16.8.100 |
| VLAN tag (optional) |  | If this network belongs to a tagged VLAN, enter the VLAN tag ID. Example: 101 |

Use the following worksheet to provide the client access network information for the Database Domains for all the compute servers in Oracle SuperCluster T5-8. You will need the following IP addresses:

- One 10GbE client access IP address for every Database Domain (dedicated domain or Database I/O Domain)

- One 10GbE client access IP address for each database zone within a Database Domain

- One Oracle RAC VIP address for each Database Domain (dedicated domain or Database I/O Domain) that is part of a RAC

- One Oracle RAC VIP address for every database zone that is part of a RAC

- Three SCAN IP addresses for every Oracle RAC

| Item | Entry |
| --- | --- |
| For compute server 1, how many Database Domains (dedicated domains or Database I/O Domains) and database zones are on this server? Enter the total number of Database Domains and database zones on this server so that one 10GbE client access IP address is assigned to each Database Domain and database zone in the server. |  |
| For compute server 2, how many Database Domains (dedicated domains or Database I/O Domains) and database zones are on this server? Enter the total number of Database Domains and database zones on this server so that one 10GbE client access IP address is assigned to each Database Domain and database zone in the server. |  |
| For compute server 1, how many of the following are part of a RAC: - Database Domains (dedicated domains or Database I/O Domains) - Database zones Enter the total number of Database Domains and database zones on this server that are part of a RAC so that one RAC VIP address is assigned to each Database Domain and database zone in the server. |  |
| For compute server 2, how many of the following are part of a RAC: - Database Domains (dedicated domains or Database I/O Domains) - Database zones Enter the total number of Database Domains and database zones on this server that are part of a RAC so that one RAC VIP address is assigned to each Database Domain and database zone in the server. |  |
| How many RACs will there be altogether within your Oracle SuperCluster? Enter the total number of RACs that will be in your Oracle SuperCluster T5-8 times 3 so that three SCAN addresses are assigned to each RAC. For example, if you have four RACs in your Oracle SuperCluster T5-8, enter 12 in this field. |  |
| Add the entries from the Entry column. This is the total number of IP addresses that you will need for the client access network for all the Database Domains in Oracle SuperCluster T5-8. |  |
| What’s Next: - If you have Application Domains on any of the compute servers in the system, go to Client Access Information for Application on page 86. - If you do not have Application Domains on any of the compute servers in the system, go to InfiniBand Network IP Addresses on page 87. |  |

Client Access Information for Application Domains

Use the following worksheet to provide subnet mask and gateway IP addresses for the client access network for the Application Domains (dedicated domains or Application I/O Domains) for all the compute servers in Oracle SuperCluster T5-8.

| Item | Entry | Description and Example |
| --- | --- | --- |
| Subnet mask |  | Subnet mask for client access network for the Application Domains. Example: 255.255.252.0 |
| Gateway IP address |  | Gateway IP address for the client access network for the Application Domains. Example: 172.17.8.1 |
| Starting IP address |  | Starting IP address for the client access network for the Application Domains. Example: 172.17.8.100 |

Use the following worksheet to provide the client access network information for the Application Domains for all the servers.

| Item | Entry |
| --- | --- |
| Enter the number of Application Domains (dedicated domains or Application I/O Domains) on server 1. |  |
| Enter the number of Application Domains (dedicated domains or Application I/O Domains) on server 2. |  |
| Add the entries from the Entry column. This is the total number of IP addresses needed for the client access network for all the Application Domains in Oracle SuperCluster T5-8. |  |
| What’s Next: Go to InfiniBand Network IP Addresses on page 87 to enter information for the InfiniBand network. |  |

# InfiniBand Network IP Addresses

Use the worksheets in this section to determine the total number of IP addresses that you will need for the InfiniBand network for your system.

Note the following important characteristics of the InfiniBand network:

- The InfiniBand network is a private network. The IP addresses and host names assigned to the components and domains for the InfiniBand network should not be registered in the DNS.

- The InfiniBand addresses for the components associated with the Database Domains must be on a different subnet from the InfiniBand addresses for the components associated with the Application Domains and Root Domains.

- These are the default InfiniBand IP addresses for all components in the system that should remain, if possible:

- Sequential IP addresses for the first subnet, starting with 192.168.10.1, for components associated with the Database Domains. The ending IP address for this subnet varies, depending on the number of Database Domains in the system.

- Sequential IP addresses for the second subnet, starting with 192.168.28.1, for components associated with the Application and Root Domains. The ending IP address for this subnet varies, depending on the number of Application and Root Domains in the system.

- If there are conflicts with the default IP addresses for InfiniBand network and existing IP addresses already on your network, or if this is another SuperCluster system that is being monitored through the same Enterprise Controller host, you can change the default IP addresses. The addresses for the components associated with the Database Domains must remain on a different subnet from the addresses for the components associated with the Application Domains and Root Domains.

General InfiniBand Network Configuration Worksheet

Use the following worksheet to provide the subnet mask and gateway IP address for the InfiniBand network.

| Item | Entry | Description and Example |
| --- | --- | --- |
| Subnet mask | 255.255.252.0 | Subnet mask for the InfiniBand network. Only valid entry for this field: 255.255.252.0 |

IP Addresses for the Database Domain InfiniBand Network

The default starting IP address for this subnet is 192.168.10.1, and the IP addresses for this subnet are assumed to be sequential.

Note – The instructions in this section assume the starting IP address for this subnet is 192.168.10.1 based on the assumption that this is the only SuperCluster system being monitored by the Enterprise Controller host. If this is not the case, you must use different IP addresses ranges for the InfiniBand network for each SuperCluster system. See the list of requirements in IP Addresses and Oracle Enterprise Manager Ops Center 12c Release 2 on page 70 for more information.

The number of IP addresses that you will need for this subnet will vary, depending on the type of Oracle SuperCluster T5-8 and the number of Database Domains and database zones:

- Half Rack: At least 16 IP addresses are needed for the InfiniBand network for this subnet:

- Maximum of 8 IP addresses needed for the Database Domains (if each compute server has four domains, and all domains are Database Domains)

- Maximum of 8 IP addresses needed for the four Exadata Storage Servers

- Full Rack: A minimum of 32 IP addresses are needed for the InfiniBand network for this subnet:

- Maximum of 16 IP addresses needed for the Database Domains (if each compute server has eight domains, and all domains are Database Domains)

- Maximum of 16 IP addresses needed for the eight Exadata Storage Servers

In addition to the minimum number of IP addresses listed above, you will also need to set aside IP addresses for the following:

- Database I/O Domains set up by your Oracle installer

- Database zones set up by your Oracle installerError: Reference source not found

Using the default starting IP address of 192.168.10.1, you should set aside the appropriate number of IP addresses for the InfiniBand network for this subnet.

Note – Even though your Oracle installer can configure up to eight database zones or I/O Domains during the initial installation of your Oracle SuperCluster, keep in mind that you will be able to configure additional database zones and I/O Domains after the initial installation, so additional IP addresses might be needed for the InfiniBand network for these future configurations you set up. Keep this in mind so that you can plan accordingly for the total number of IP addresses needed for the InfiniBand network for the future.

If there are conflicts on your network with sequential IP addresses starting with 192.168.10.1 for this subnet, enter an alternate starting IP address for this subnet in the table below. Note that you can choose a different subnet from 192.168.10.1, if necessary, as long as it is not the same subnet used in the section IP Addresses for the Application Domain and Root Domain Storage InfiniBand Network on page 90.

| Item | Entry |
| --- | --- |
| Enter the alternate starting IP address. |  |
| If necessary, enter the alternate subnet for the InfiniBand network for this subnet. |  |

IP Addresses for the Application Domain and Root Domain Storage InfiniBand Network

The default starting IP address for this subnet is 192.168.28.1, and the IP addresses for this subnet are assumed to be sequential.

Note – The instructions in this section assume the starting IP address for this subnet is 192.168.28.1 based on the assumption that this is the only SuperCluster system being monitored by the Enterprise Controller host. If this is not the case, you must use different IP addresses ranges for the InfiniBand network for each SuperCluster system. See the list of requirements in IP Addresses and Oracle Enterprise Manager Ops Center 12c Release 2 on page 70 for more information.

The number of IP addresses that you will need for this subnet will vary, depending on the type of Oracle SuperCluster T5-8:

- Half Rack: A minimum of 19 IP addresses are needed for the InfiniBand network for this subnet:

- 1 IP address needed for the ZFS storage controller cluster

- Maximum of 8 IP addresses needed  for the two compute servers

- Maximum of 8 IP addresses needed for the storage interconnect for Database Domains

- Maximum of 2 IP addresses needed for Oracle Enterprise Manager Ops Center VIP

- Full Rack: A minimum of 35 IP addresses are needed for the InfiniBand network for this subnet:

- 1 IP address needed for the ZFS storage controller cluster

- Maximum of 16 IP addresses needed  for the two compute servers

- Maximum of 16 IP addresses needed for the storage interconnect for Database Domains

- Maximum of 2 IP addresses needed for Oracle Enterprise Manager Ops Center VIP

In addition to the minimum number of IP addresses listed above, you will also need to set aside two IP addresses for every Application I/O Domain that is set up by your Oracle installer. See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information.

Using the default starting IP address of 192.168.28.1, you should set aside the appropriate number of IP addresses for the InfiniBand network for this subnet.

Note – Even though your Oracle installer can configure up to eight I/O Domains during the initial installation of your Oracle SuperCluster, keep in mind that you will be able to configure additional I/O Domains after the initial installation, so additional IP addresses might be needed for the InfiniBand network for these future configurations you set up. Keep this in mind so that you can plan accordingly for the total number of IP addresses needed for the InfiniBand network for the future.

If there are conflicts on your network with sequential IP addresses starting with 192.168.28.1 for this subnet, enter an alternate starting IP address for this subnet in the table below. Note that you can choose a different subnet from 192.168.28.1, if necessary, as long as it is not the same subnet as the one used in the section IP Addresses for the Database Domain InfiniBand Network on page 88.

| Item | Entry |
| --- | --- |
| Enter the alternate starting IP address. |  |
| If necessary, enter the alternate subnet for the InfiniBand network for this subnet. |  |

Chapter

6

Change Log

Use the tables in this chapter to record changes that have occurred over time with this installation.

| Date | Item That Has Changed | New Item | Notes |
| --- | --- | --- | --- |

See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information on the maximum number of database zones and I/O Domains that can be set up by your Oracle installer.

See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information on the maximum number of database zones and I/O Domains that can be set up by your Oracle installer.

Only valid if the first domain (the Control Domain) is also a Database Domain, with or without zones.

See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information on the maximum number of database zones and I/O Domains that can be set up by your Oracle installer.

Only valid if the first domain (the Control Domain) is also a Database Domain, with or without zones.

For raw capacity, 1 GB = 1 billion bytes. Capacity calculated using normal space terminology of 1 TB = 1024 * 1024 * 1024 * 1024 bytes. Actual formatted capacity is less.

Note that for the Half Rack, the DATA and RECO disk groups will be set to high redundancy, but the DBFS disk group will be set to normal redundancy.

For raw capacity, 1 GB = 1 billion bytes. Capacity calculated using normal space terminology of 1 TB = 1024 * 1024 * 1024 * 1024 bytes. Actual formatted capacity is less.

Applicable when you do not have Oracle Support Hub.

Applicable when you only have Oracle Support Hub.

See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information.

See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information.

See Oracle Setup of Database Zones and I/O Domains Overview on page 11 for more information.