# Oracle® COMMUNICATIONS

# Diameter Signaling Router 
DSR Network Impact Report

Release 8.4

F12342 Revision 02

June 2020

Oracle Diameter Signaling Router DSR Network Impact Report,

Release 8.4

Copyright © 2020 Oracle and/or its affiliates. All rights reserved.

This software and related documentation are provided under a license agreement containing restrictions on use and disclosure and are protected by intellectual property laws. Except as expressly permitted in your license agreement or allowed by law, you may not use, copy, reproduce, translate, broadcast, modify, license, transmit, distribute, exhibit, perform, publish, or display any part, in any form, or by any means. Reverse engineering, disassembly, or decompilation of this software, unless required by law for interoperability, is prohibited.

The information contained herein is subject to change without notice and is not warranted to be error-free. If you find any errors, please report them to us in writing.

If this is software or related documentation that is delivered to the U.S. Government or anyone licensing it on behalf of the U.S. Government, the following notice is applicable:

U.S. GOVERNMENT END USERS: Oracle programs, including any operating system, integrated software, any programs installed on the hardware, and/or documentation, delivered to U.S. Government end users are "commercial computer software" pursuant to the applicable Federal Acquisition Regulation and agency-specific supplemental regulations. As such, use, duplication, disclosure, modification, and adaptation of the programs, including any operating system, integrated software, any programs installed on the hardware, and/or documentation, shall be subject to license terms and license restrictions applicable to the programs. No other rights are granted to the U.S. Government.

This software or hardware is developed for general use in a variety of information management applications. It is not developed or intended for use in any inherently dangerous applications, including applications that may create a risk of personal injury. If you use this software or hardware in dangerous applications, then you shall be responsible to take all appropriate fail-safe, backup, redundancy, and other measures to ensure its safe use. Oracle Corporation and its affiliates disclaim any liability for any damages caused by use of this software or hardware in dangerous applications.

Oracle and Java are registered trademarks of Oracle and/or its affiliates. Other names may be trademarks of their respective owners.

Intel and Intel Xeon are trademarks or registered trademarks of Intel Corporation. All SPARC trademarks are used under license and are trademarks or registered trademarks of SPARC International, Inc. AMD, Opteron, the AMD logo, and the AMD Opteron logo are trademarks or registered trademarks of Advanced Micro Devices. UNIX is a registered trademark of The Open Group.

This software or hardware and documentation may provide access to or information on content, products, and services from third parties. Oracle Corporation and its affiliates are not responsible for and expressly disclaim all warranties of any kind with respect to third-party content, products, and services. Oracle Corporation and its affiliates will not be responsible for any loss, costs, or damages incurred due to your access to or use of third-party content, products, or services except as set forth in an applicable agreement between you and Oracle.

Contents

Oracle® COMMUNICATIONS	1

Diameter Signaling Router  DSR Network Impact Report	1

1	Introduction	11

1.1	Purpose/Scope	11

1.2	Compatibility	11

1.2.1	8.4.0.0.0 Product Compatibility	11

1.3	DSR 8.4.x Incompatibility Features	11

1.3.1	8.4.0.3.0 InCompatibility features	11

1.3.2	8.4.0.5.0 InCompatibility features	11

1.4	Disclaimers	12

2	Overview of DSR 8.4.x Features	13

2.1	Enhancements to DSR 8.4.0.0.0	14

2.1.1	Alarm Group Feature	15

2.1.2	Time distance CHECK ENHANCEMENT FEATURE	15

2.1.3	VNFM	16

2.1.4	MMI UPDATES	17

2.1.5	VSTP SPARE POINT CODE SUPPORT	18

2.1.6	VSTP MNP SUPPORT OVER SS7	19

2.1.7	VSTP MTP SCREENING SUPPORT	19

2.1.8	VSTP Multiple point code support	19

2.1.9	VSTP flow control ENHANCEMENTS	19

2.1.10	VSTP ANSI-ITU Conversion SUPPORT	20

2.1.11	VSTP GTT Modifications	20

2.1.12	VSTP SCCP loop DETECTION	20

2.1.13	VSTP MBR enhancement	20

2.1.14	SCEF	20

2.2	Enhancements to DSR 8.4.0.3.0	22

2.2.1	vstp GTT Throttle action	23

2.2.2	vstp GTT scpval action	23

2.2.3	vstp MTP based gtt	23

2.2.4	vstp sfapp stateful security	23

2.2.5	vstp TDm support	24

2.2.6	vstp M3UA Client support	24

2.2.7	VSTP IDPR MOSMS	24

2.2.8	vstp eir enhancements	24

2.3	Enhancements to DSR 8.4.0.5.0	25

2.3.1	VSTP sls rotation	25

2.3.2	VSTP sfapp dynamic learning	25

2.3.3	VSTP tif support	25

2.3.4	VSTP segmented xudt	26

2.3.5	VSTP duplicate point code support	26

2.3.6	vSTP GUI configuration	26

2.3.7	vstp ir21 bulk upload for ss7 security	26

2.3.8	dsa with udr	26

2.4	HARDWARE CHANGES	27

2.4.1	Hardware Supported	27

2.4.2	Hardware Upgrade	27

2.5	Software Details	27

2.5.1	Software Platform Components in 8.4.0.0.0	27

2.5.2	Software Platform Components in 8.4.0.3.0	28

2.5.3	Software Platform Components in 8.4.0.5.0	28

2.5.4	DSR Release 8.4	29

2.5.5	iDIH 8.2.1 and 8.2.2	29

2.5.6	SDS 8.4	29

2.6	FIRMWARE CHANGES	29

2.7	UPGRADE OVERVIEW	30

2.7.1	DSR Upgrade Path	30

2.7.2	The supported upgrade paths for SDS 8.4	30

2.7.3	The supported upgrade paths for iDIH 8.2.1	31

2.7.4	Upgrade Execution	31

2.7.5	Limitations	31

2.8	Migration of DSR Data	32

3	Release 8.4.0.0.0 Feature OAM Changes	33

3.1	ALARM GROUP FEATURE	33

3.1.5	Alarm Aggregation interworking with Alarm Group feature	36

3.2	TIME DISTANCE CHECK	36

3.3	VNFM	38

3.4	SCEF	42

3.5	VSTP SPARE POINT CODE SUPPORT	50

3.6	VSTP MNP GPORT	54

3.7	VSTP MNP ATI NP	63

3.8	VSTP MNP INP	66

3.9	VSTP MTP Screening support	68

3.10	VSTP MULTIPLE POINT CODE SUPPORT	74

3.11	VSTP FLOW CONTROL ENHANCEMENTS	80

3.12	VSTP ANSI-ITU CONVERSION	80

4	Release 8.4.0.3.0 Feature OAM Changes	82

4.1	GTT THROTTLE ACTION	82

4.2	GTT SCPVAL ACTION	86

4.3	MTP BASED GTT	89

4.4	SFAPP STATEFUL SECURITY	90

4.5	TDM SUPPORT	95

4.6	M3UA CLIENT SUPPORT	99

4.7	VSTP IDPR MOSMS	102

4.8	VSTP EIR ENHANCEMENTS	106

5	Release 8.4.0.5.0 Feature OAM Changes	112

5.1	SLS ROTATION	112

5.2	SFAPP DYNAMIC LEARNING	115

5.3	TIF SUPPORT	117

5.4	SEGMENTED XUDT	119

5.5	DUPLICATE POINT CODE SUPPORT	123

5.6	VSTP IR21 BULK UPLOAD FOR SS7 SECURITY	125

5.7	DSA WITH UDR	129

6	MEAL INSERTS	131

6.1	DSR/SDS 8.4.0.5.0 MEAL Snapshot	131

6.1.1	MEAL Delta between 8.4.0.0.0 and 8.4.0.5.0	131

6.1.2	MEAL Delta between 8.4.0.3.0 and 8.4.0.5.0	131

6.2	DSR/SDS 8.4.0.3.0 MEAL Snapshot	131

6.2.1	MEAL Delta between 7.0.1.0.0 and 8.4.0.3.0	131

6.3	DSR/SDS 8.4.0.0.0 MEAL Snapshot	132

6.3.1	MEAL Delta between 8.0.0.0.0 and 8.4.0.0.0	132

6.3.2	MEAL Delta between 8.1.0.0.0 and 8.4.0.0.0	132

6.3.3	MEAL Delta between 8.2.1.0.0 and 8.4.0.0.0	132

6.3.4	MEAL Delta between 8.3.0.0.0 and 8.4.0.0.0	132

7	Reference list	133

List of Figures

Figure 1 – DSR Upgrade Paths	30

Figure 2 – SDS Upgrade Paths	30

Figure 3 – IDIH Upgrade Paths	31

Figure 4 - Alarm Group Feature Design	33

Figure 5 – Peer Node Alarm Group GUI	34

Figure 6 – Connection Alarm Group GUI	35

Figure 7 – DCA Framework Screen	37

Figure 8 – VNF Scaling	39

Figure 9 – VNF LCM Operation	40

Figure 10 – VNF Forceful Termination	41

Figure 11 – VNF Graceful Termination	42

Figure 12 – LWM2M Registration Process	43

Figure 13 – Subscription and Observe	43

Figure 14 – Event Notification	44

Figure 15 – Subscription and cancel	44

Figure 16 – Action on DL data transfer	44

Figure 17 – T8 Event	45

Figure 18 – DELETE Subscription	46

Figure 19 – T8 Request and Response	46

Figure 20 – PUT Operation	47

Figure 21 – DELETE Operation	47

Figure 22 – T8 POST Operation	48

Figure 23 – API based charging for Invocation events	49

Figure 24 – Notify and Uplink Request	49

Figure 25 – SLTM Operation accept	52

Figure 26 – SLTM Operation discard	52

Figure 27 – MMI API Guide reference	53

Figure 28 – ATINP message flow	64

Figure 29 – INQP message flow	67

Figure 30 – MTP screening design diagram	69

Figure 31 – Process Flow of GTT Throttle Action	84

Figure 32 – Flow Diagram for SCPVAL	87

List of Tables

Table 1 - DSR 8.4.0.0.0 New Features/Enhancements	14

Table 2 - Alarm Group Feature Description	15

Table 3 - Time Distance Check Feature Description	15

Table 4 - VNFM Feature Description	16

Table 5 -  MMI Updates Feature Description	17

Table 6 - VSTP Spare point code Feature Description	18

Table 7 - VSTP MNP support over SS7 Feature Description	19

Table 8 - VSTP mtp screening support Feature Description	19

Table 9 - VSTP multiple point code support Feature Description	19

Table 10 - VSTP Flow Control Enhancements Feature Description	19

Table 11 - VSTP ANSI-ITU Conversion support Feature Description	20

Table 12 - VSTP GTT Modifications Feature Description	20

Table 13 - VSTP sccp loop detection Feature Description	20

Table 14 - VSDTP MBR enhancement Feature Description	20

Table 15 - SCEF Feature Description	20

Table 16 - DSR 8.4.0.3.0 New Features/Enhancements	22

Table 17 – VSTP GTT Throttle Action Feature Description	23

Table 18 – VSTP GTT SCPVAL Action Feature Description	23

Table 19 – VSTP GTT SCPVAL Action Feature Description	23

Table 20 –VSTP MTP Based GTT Feature Description	23

Table 21 – VSTP SFAPP Stateful Security Feature Description	23

Table 22 – VSTP TDM Support Feature Description	24

Table 23 – VSTP M3UA Client Support Feature Description	24

Table 24 – VSTP IDPR MOSMS Feature Description	24

Table 25 – VSTP EIR Enhancements Feature Description	24

Table 26 - DSR 8.4.0.5.0 New Features/Enhancements	25

Table 27 – VSTP SLS Rotation Feature Description	25

Table 28 – VSTP SFAPP Dynamic Learning Feature Description	25

Table 29 – VSTP TIF Support Feature Description	25

Table 30 – VSTP Segmented XUDT Feature Description	26

Table 31 – VSTP Duplicate Point Code Feature Description	26

Table 32 – VSTP GUI Configuration Feature Description	26

Table 33 – VSTP IR21 Bulk Upload for SS7 Security	26

Table 34 – VSTP GUI Configuration Feature Description	26

Table 35 - Hardware Details	27

Table 36 – Software Platform Component Details – 8.4.0.0.0	27

Table 37 – Software Platform Component Details – 8.4.0.0.0	28

Table 38 – Software Platform Component Details – 8.4.0.5.0	28

Table 39 - Release Details	29

Table 40 - IDIH Details	29

Table 41 - SDS Details	29

Table 42 – Security Countermeasure Config	36

Table 43 -System Config Options Table	36

Table 44 -TimeDistChk_Config Table	36

Table 45 – TimeDistChk Continent Config	37

Table 46 – TimeDistChk MCC Config	37

Table 47 - NI and Name	50

Table 48 - Configuration details1	51

Table 49 - Configuration details2	51

Table 50 – MO Details 1	53

Table 51 – MO Details 2	53

Table 52 – MO details 3	59

Table 53 – Measurements details1	59

Table 54 – Measurement details2	60

Table 55 – Events & Alarms 1	61

Table 56 – MO’s for ATI NP Feature	65

Table 57 – ATI NP Measurements details	65

Table 58 – Alarms & Events for ATI NP feature	66

Table 59 – Measurements for INPQ feature	67

Table 60 – Alarms & Events for INPQ feature	68

Table 61 - Message processing with mtpScrEventLog and mtpScrTestMode option when NSFI is FAIL	71

Table 62 – MO details for MTP screening feature	72

Table 63 – Alarms and Events for MTP screening feature	73

Table 64 – SCCP message routing behavior	75

Table 65 – Point Code provisioning	77

Table 66 – Isp Name with reference	78

Table 67 – Field name with Description	78

Table 68 – AppType with Ssn id	78

Table 69 – Alarms & Description	79

Table 70 – Events & Description	79

Table 71 – MO details for GTT action feature	80

Table 72 – Measurements	84

Table 73 – Alarms for GTT Throttle Action Feature	85

Table 74 – GTT Action SCPVAL – Parameters	86

Table 75 – Alarms for GTT SCPVAL Action Feature	88

Table 76 – Measurements	90

-Table 77 – Measurements	91

Table 78 – Alarms for SFAPP Stateful Security Feature	93

Table 79 – Measurements	95

Table 80 – Events for TDM Support Feature	97

Table 81 – Alarms for TDM Support Feature	97

Table 82 – Measurements	100

Table 83 – Measurements	103

Table 84 – Events for vSTP IDPR MOSMS Feature	105

Table 85 – Measurements	107

Table 86 – Alarms & Events for vSTP EIR Feature	108

Table 87 – Measurements	116

Table 88 – Alarms & Events	117

Table 89 – Measurements	118

Table 90 – Alarms & Events	119

Table 91 – Measurements	120

Table 92 – Alarms & Events	121

Table 93 – Measurements	127

GLOSSARY

| Acronym/Term | Definition |
| --- | --- |
| APIGW | API Gateway |
| ASGU | Automated Server Group Upgrade |
| AS | Application Server |
| ASU | Automated Site Upgrade |
| AVP | Attribute Value Pair |
| BSBR | Binding SBR |
| CA | Communication Agent |
| CAF | Customized Application Framework |
| CLI | Command Line Interface |
| CLR | Cancel Local Request |
| DA-MP | Diameter Agent Message Processor |
| DAL | Diameter Application Layer |
| DCA | Diameter Custom Application Framework |
| DCL | Diameter Connection Layer |
| DEA | Diameter Edge Agent |
| DPC | Destination Point Code |
| DPL | Data Processor Library |
| DRMP | Diameter Routing Message Priority |
| DPI | Diameter Plug-in |
| DSA | Diameter Security Application |
| DoS | Denial of Service |
| EXGSTACK | Eagle Next Generation Stack |
| vEIR | Virtual Equipment Identity Register |
| ECR | Mobile Equipment-Identity-Check-Request |
| ECA | Mobile Equipment-Identity-Check-Answer |
| FLOBR | Flexible Link set Optional Based Routing |
| GUI | Graphical User Interface |
| GTT | Global title translation |
| GTA | Global title Address |
| HSS | Home Subscriber Server |
| HLR | Home Location register |
| iLO | Integrated Lights Out |
| IMI | Internal Management Interface |
| IPv4 | IPv4 address of the subscriber |
| IPv6 | IPv6 address of the subscriber |
| IMSI | International Mobile Subscriber Identity |
| IMPU | IP Multimedia Public Identity |
| IMPI | IP Multimedia Private Identity |
| IOT | Interoperability Tests |
| KPI | Key Performance Indicator |
| LAI | Location Area Identity |
| LTE | Long Term Evolution |
| MAP | Mobile Application Part |
| MBR | Map Based Routing |
| MCC | Mobile Country Code |
| MEAL | Measurements, Events, Alarms, and Logging |
| MME | Mobility Management Entity |
| MMI | Man Machine Interface |
| MP | Message Processor |
| MPS | Messages per Second |
| MS | Mobile Station/Handset |
| MSU | Message signal Unit |
| MSISDN | Mobile Station International Subscriber Directory Number |
| MTC | Machine type communication |
| MTP | Message Transfer Part |
| MO | Managed Object |
| NE | Network Element |
| NGN | Next Generation Networks |
| NGN-PS | NGN Priority Services |
| NIDD | Non-IP data delivery [directly through MME/SGSN] |
| NMS | Network Management System |
| NOAM | Network Operations Administration and Maintenance |
| NF | Network Function |
| NRF | NF Repository Function |
| OAG | Oracle Accessibility Guidelines |
| OAM | Operations, Administration, Maintenance |
| OAM&P | Operations, Administration, Maintenance and Provisioning |
| OCUDR | Oracle Communications User Data Repository |
| OPC | Origin Point Code |
| PDRA | Policy Diameter Relay Agent |
| PCRF | Policy Control and Charging Rules Function |
| PCIMC | Per Connection Ingress Message Control |
| PDU | Protocol Data Unit |
| PDN | Packet Data Network |
| PM&C | Platform, Management and Control |
| POR | Plan of Record |
| PS | Priority Service (NGN-PS) |
| RAN | Radio Access Network |
| ROS | Routing Option Set |
| RSA | Reset Answer |
| RSR | Reset Request |
| SBR | Session Binding Repository |
| SSBR | Session SBR |
| SCEF | Service Capability Exposure Function |
| ScsAsId | String provided by SCS to identify itself in non-3GPP world |
| SCEF-MP | Message processing server that will run business login of SCEF/MTC-IWF. (for DSR , it is DA-MP server) |
| SCEF-DB | U-SBR (database server that stores context of SCEF calls) |
| SCS | Service Control Server |
| SOAM | Site Operations Administration and Maintenance |
| SS7 | Signaling System No. 7 |
| STP-MP | Signaling Transfer Point Message Processor |
| SV | Software Version |
| TPD | ORACLE Platform Distribution |
| TCAP | Transaction Capability Part |
| TLTRI | T8 Long Term Transaction Reference ID |
| TTRI | T8 Transaction Reference ID |
| TOBR | TCAP Opcode Based Routing |
| UE | User Equipment |
| USBR | Universal SBR |
| VIP | Virtual IP Address |
| VNF | Virtual Network Functions |
| VNFM | Virtual Network Functions Manager |
| VPLMN | Virtual Public Land Mobile Network |
| VSTP | Virtual SS7 Signal Transfer Point |
| VEDSR | Virtualized Engineered DSR |
| XMI | External Management Interface |
| XSI | External Signaling Interface |

# Introduction

## Purpose/Scope

Purpose of this document is to highlight the changes of the product that may have impact on the customer network operations, and should be considered by the customer during planning for this release.

## Compatibility

## 8.4.0.0.0 Product Compatibility

DSR 8.4.0.0.0 is compatible with IDIH 8.0, 8.1, 8.2, 8.2.1 and 8.2.2

DSR 8.4.0.0.0 is compatible with SDS 8.0.1, 8.1.2, 8.2.1, 8.3 and 8.4.0

DSR 8.4.0.0.0 is compatible with APIGW 8.4.0

DSR 8.4.0.0.0 is compatible with TPD 7.6, ComCOL 7.5, AppWorks 8.4, EXGSTACK 8.4, TVOE 3.6, PM&C 6.6, APIGW 8.4 and UDR 12.5.1

SDS 8.4 is compatible with TPD 7.6, ComCOL 7.5, AppWorks 8.4, EXGSTACK 8.4, TVOE 3.6 and PM&C 6.6.

## DSR 8.4.x Incompatibility Features

The following features has been made incompatible from DSR 8.3 and remain incompatible in 8.4

- Active/Standby DA-MP server architecture (1+1) redundancy model

- MAP-IWF

- Radius

- GLA

## 8.4.0.3.0 InCompatibility features

Virtualized Engineered DSR (VEDSR) deployment also known as TVOE based Fully Virtualized Rack Mount Server (FV RMS) Signaling node is not supported from DSR Release 8.3 onwards.
Following are the non-supported network elements of Virtualized Engineered DSR (VEDSR):
·      DSR NOAM,
·      DSR SOAM,
·      DSR Message Processors (MP),
·      SS7 MP,
·      DSR IPFE,
·      DSR SBR (Session/Binding/Universal),
·      SDS NOAM,
·      SDS SOAM,
·      SDS QS,
·      SDS DP

Note: DSR and SDS Baremetal Installations with TVOE based NOAM/SOAM will continue to be supported.

Virtualized Engineered DSR (VEDSR) networks and associated elements needs to be migrated to virtual DSR implementation based on KVM with/without Openstack or VMWARE prior to DSR 8.3 or 8.4.x upgrade or install.

## 8.4.0.5.0 InCompatibility features

The “Diameter Security Application (DSA) with Universal-SBR (USBR)" is an obsolete application. Alternatively, the "Diameter Security Application (DSA) with UDR is introduced in DSR 8.4.0.5.0. Refer to NIR document and Diameter Security Application with UDR User’s Guide for details.

Customers using this application must not upgrade DSR software to DSR 8.4.0.5.0 release and should migrate to “DSA with UDR” based application.

## Disclaimers

This document summarizes Release Diameter Signaling Router Release 8.4 new and enhancement features as compared to Release 8.3, and the operations impacts of these features, at a high level. The Feature Requirements Specification (FRS) documents remain the defining source for the expected behavior of these features.

# Overview of DSR 8.4.x Features

This section provides a high-level overview of the DSR 8.4.x release features that may impact OAM interfaces and activities.

For a list of all features, please see Release Notes for DSR 8.4.x found at the following link:

http://docs.oracle.com/en/industries/communications/diameter-signaling-router/index.html

For additional details of the various features, please refer to the “DSR 8.4 Feature Guide” found at the following link:

http://docs.oracle.com/en/industries/communications/diameter-signaling-router/index.html

## Enhancements to DSR 8.4.0.0.0

Note: For information on upgrade planning and required steps before upgrade, please refer to the DSR 8.4 Software Upgrade Guide on the public Oracle Documentation Site:

Docs.oracle.com  Industries  Oracle Communications  Diameter Signaling Router  Release 8.4.

Table 1 - DSR 8.4.0.0.0 New Features/Enhancements

DSR 8.4 Feature/Enhancement Name

Alarm Group FeatureError! Hyperlink reference not valid.

Time Distance Check Enhancement FeatureError! Hyperlink reference not valid.

VNFMError! Hyperlink reference not valid.

MMI UpdatesError! Hyperlink reference not valid.

vSTP Spare Point Code Support

vSTP MNP support over SS7 using vSTP

vSTP MTP Screening Support

vSTP Multiple Point Code Support for virtual STP

vSTP Flow control enhancements

vSTP ANSI-ITU Conversion support

vSTP GTT Modifications

vSTP SCCP loop detection

vSTP MBR enhancements

SCEF

### Alarm Group Feature

Table 2 - Alarm Group Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 27797933 | DSR displays all relevant individual and aggregated alarms for Connection/Peer as per functionality captured by Alarm Aggregation feature. However due to the large size of the network, volume of connection and peer initiating alarms can be high. This limits operator view to monitor the state of crucial managed objects. Operators have requested the Alarm Group feature that allows them to suppress similar type of alarms, after a given threshold. Alarm Group feature allows operator to do the following: - Define group of peers/connections for which alarm throttling is required - Define throttling level for each group This is named as Alarm Group feature. This feature is administratively managed by the operator through GUI, Configuration and alarm monitoring is also on Active SOAM Note: Alarm Aggregation feature and Alarm Group feature are mutually exclusive for Peer Nodes and Connections. | Alarm Group feature shall work for Peer Node Alarm Group and Connection Alarm Group, Alarm monitoring is limited to Peer Nodes and Connections that are added in Alarm Group |

### Time distance CHECK ENHANCEMENT FEATURE

Table 3 - Time Distance Check Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 28108321 28899149 27737337 | This feature provides the following enhancement: Enhance DSA logic to detect whether the IMSI is vulnerable or not when subscriber is an outbound roamer. Update DSR OAM GUI to configure MCC_Based, VPLMN_Based and Continent - Changed allowed even if diameter connection is enabled. - This feature is intended for security countermeasure which detects if subscriber is indeed physically able to move roam from one network/country to another network/country within the given transit time. Ex: If the mobile subscriber is roaming from USA to India within 1 hour which is physically impossible to commute in 1 hour. Such scenarios can be the hacker induced security attacks on the MNO's network. - This countermeasure is applicable to outbound roaming subscriber and compares the current location with previous location of the subscriber and analyze if it is physically possible to move from the previous location to this new one. This can be achieved by maintaining the minimum transit time between each of the VPLMN Id's or MCC's of VLPMN Id's. - If subscriber moves from one country to another country, and the time difference between last update location and current update location procedure (i.e. time difference between last ULR and current AIR/ULR) is not greater than the configured transit time between corresponding countries(MCC's) then ingress AIR/ULR shall be marked as vulnerable by DEA | Enhancement Request |

### VNFM

The objective of the DSR VNFM is to provide an ETSI-compliant VNF manager.

Table 4 - VNFM Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 28104363 | DSR VNFM is an application that helps to deploy virtual DSRs quickly by automating the entire deployment process and making it ready to use in the shortest possible time. The VNFM is responsible for the lifecycle management of virtual network functions (VNFs) under the control of the network function virtualization orchestrator (NFVO). The VNFM would be helpful in - Instantiate Network OAM VNFs with fixed IPs - Instantiate Signaling VNFs with Multiple XSIs for fixed IPs - Instantiating DSR DR NOAM - Instantiating SDS DR NOAM - Scale VNF -Scale VNF to Level (Scale Out C Level servers of Signaling VNF) -Scale VNF to Arbitrary size (Scale Out C Level servers of Signaling VNF) - Query Individual / All LCM Operation(s) Terminating VNF - Discover VNF - Terminate VNF | Enhancement Request |

### MMI UPDATES

DSR supports a RESTful machine-to-machine interface to support OAM requests from external clients Oracle provided or from 3rd parties.

Table 5 -  MMI Updates Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 27096415 Machine-to-Machine interface updates | MMIs have been enabled for the following: - SBR Configuration - SBR Databases - SBR Database Resizing Plans - SBR Data Migration Plans - Database Options - Dynamic Peer Discovery - Configuration - Realms - DNS Sets - Discovery Attributes - Diameter Common - MPs - DA-MP - SS7-MP - Topology Hiding – Configuration - Trusted Network Lists - Path Topology Hiding Configuration Sets - S6a/S6d HSS Topology Hiding Configuration Sets - MME/SGSN Topology Hiding Configuration Sets - S9 PCRF Topology Hiding Configuration Sets Options - S9 AF/pCSCF Topology Hiding Configuration Sets - Protected Networks - User Configuration - Appworks -Software Management - Versions - User Configuration - Appworks - Access Control - Users - Groups - Sessions - Authorized Ips - User Configuration - Appworks - Remote Server - LDAP Authentication - Data Export - User Configuration - Appworks - Alarms - Alarm History - Trap Logs - User Configuration - Appworks - Administration - General Options | Enhancement Request User Configuration – Appworks – Remote Server - Data Export User Configuration – Appworks - Alarms - Place and Place Association in Alarm History - Export in Alarm History |

### VSTP SPARE POINT CODE SUPPORT

The DSR 8.4 provides support for ITUI spare domain and ITUN spare.

Table 6 - VSTP Spare point code Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 28219409 | This feature provides support for ITUI spare domain and ITUN spare domain. | Enhancement Request |

### VSTP MNP SUPPORT OVER SS7

The DSR 8.4 vSTP provides support for MNP over SS7. It covers GPORT, ATINPQ and IDPQ.

Table 7 - VSTP MNP support over SS7 Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 27355487 & 28829325 – {28648154, 28648192} | This feature provides support for GPORT, ATINPQ and IDPQ. Also, it includes support for HEX digits for GTA. | Enhancement Request |

### VSTP MTP SCREENING SUPPORT

The DSR 8.4 vSTP provides MTP screening features, which allows screening of messages based on the parameters of MTP3 layer.

Table 8 - VSTP mtp screening support Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 25973064 | MTP screening feature provides solution to screen the messages based on MTP3 layer parameters of the messages. | Enhancement Request |

### VSTP Multiple point code support

The DSR 8.4 vSTP provides support for multiple local point codes of a particular domain. This feature provides support for SPC and CPC.

Table 9 - VSTP multiple point code support Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 28126356 | This feature supports assigning multiple point codes to VSTP i.e. SPC (Secondary Point Codes) and CPC (Capability Point Codes). Earlier, vSTP was used to support only one local signaling point for a domain, which is now known as TPC (True point code). CPC also optionally permits attaching service/application with hosted CPC. SPC in turn enables collapsing/merging of multiple STPs into one vSTP. | Enhancement Request |

### VSTP flow control ENHANCEMENTS

The DSR 8.4 vSTP supports with the egress and ingress flow control

Table 10 - VSTP Flow Control Enhancements Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 28609300 | Flow Control is essential to prevent it from sending bursty traffic in the egress path and receiving bursty traffic in ingress path. This maintains a balanced message processing rate in the ingress path based on capacity at run time. | Enhancement Request |

### VSTP ANSI-ITU Conversion SUPPORT

The DSR 8.4 vSTP supports ANSI/ITU SCCP Conversion

Table 11 - VSTP ANSI-ITU Conversion support Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 25973034 | ANSI/ITU SCCP Conversion feature provides the vSTP the ability to support a generic ANSI/ITU SCCP Conversion. ANSI/ITU SCCP Conversion is supported for UDT, UDTS, XUDT and XUDTS messages. | Enhancement Request |

### VSTP GTT Modifications

The DSR 8.4 vSTP supports GTT modifications feature

Table 12 - VSTP GTT Modifications Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 25972692 | This request is to allow the support of GTT Modification as per EAGLE code in vSTP (equivalent to Advanced GTT Modification in EAGLE World) | Enhancement Request |

### VSTP SCCP loop DETECTION

The DSR 8.4 vSTP supports SCCP looping issues.

Table 13 - VSTP sccp loop detection Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 29136281 | Loop Detection feature allows user to solve the SCCP looping issue without the need for network-wide implementation of a protocol. | Enhancement Request |

### VSTP MBR enhancement

The DSR 8.4 vSTP MBR Enhancement enabled GTT routing based on three new parameters (VLRNP, SMRPOA and SMRPDA)

Table 14 - VSDTP MBR enhancement Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 27393516 | This feature allows GTT routing based on MAP component (i.e. IMSI, MSISDN, VLRNB, SMRPOA, SMRPDA). MBR Enhancement enabled GTT routing based on three new parameters (VLRNP, SMRPOA and SMRPDA). | Enhancement Request |

### SCEF

Table 15 - SCEF Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR 28111851 LWM2M Gateway | LWM2M Gateway(LG) shall act as cross proxy converting the CoAP messages to HTTP messages and vice versa to enable IP device communication between IoT devices and SCS/AS (Note: CoAP protocol is used as transport protocol to carry the LWM2M payload.) LG shall enable LWM2M protocol based IoT device communication for following use cases: - Access Control - Device Management - Connectivity - Firmware Update - Location - Connectivity Statistics LG shall enabled SCEF to be used for following transport bindings: - TCP, UDP, TLS and DLTS ( covered by this feature) (Note: Scope of the current release will be UDP transport only) - Non IP Data Delivery ( existing SCEF functionality) LG shall provide unified T8 interface for both IP and Non IP device communication enabling MO, MT and monitoring event MTC call flows. | Enhancement Request |
| POR 28111828 MQTT Broker | MQTT Broker(MB) shall act as cross proxy converting the MQTT messages to HTTP messages and vice versa to enable IP device communication between IoT devices and SCS/AS. MB shall enable IoT device communication for following use case: - MO messages MB shall enable SCEF to be used for following transport bindings: - TCP and TLS ( covered by this feature) MB shall provide unified T8 interface for IP device communication enabling monitoring event call flows | Enhancement Request |
| POR 28629586 QoS Control | According to 3GPP specifications, there are two main Network Attach options to support IOT connectivity. - Attach with PDN (Packet Data Network) connection. - Attach without PDN connection. For IOT connectivity attach with PDN scenario, The 3rd party SCS/AS may request that a data session to a UE that is served by the 3rd party service provider is set up with a specific QoS (e.g. low latency or jitter) and priority handling. This section defines requirements for supporting the functionality where Oracle SCEF can enable PCRF setting up of the QoS via Rx Interface as requested from SCS/AS. | Enhancement Request |
| POR 28124801 API Based Charging Solution | SCEF shall support API based charging for the following procedures that operate across the T8 reference point. In addition, charging shall be implemented based on Offline event based charging mechanism. Moreover, the event are. - NIDD Events - Monitoring Events - Device Triggering Events - Enhanced coverage Restriction Events. | Enhancement Request |
| POR 28113780 SCEF T8 Compliance Changes | SCEF Compliance changes as per 3GPP TS 29.122 latest specification i.e.,v15 | Enhancement Request |

## Enhancements to DSR 8.4.0.3.0

Note: For information on upgrade planning and required steps before upgrade, please refer to the DSR 8.4 Software Upgrade Guide on the public Oracle Documentation Site:

Docs.oracle.com  Industries  Oracle Communications  Diameter Signaling Router  Release 8.4.0.3.0.

Table 16 - DSR 8.4.0.3.0 New Features/Enhancements

DSR 8.4.0.3.0 Feature/Enhancement Name

vSTP Gtt Throttle Action

vSTP _GTT_SCPvalError! Hyperlink reference not valid.Error! Hyperlink reference not valid.

vSTP MTP Based GTT

vSTP Sfapp Stateful Security

vSTP_TDM_SupportError! Hyperlink reference not valid.

vSTP M3UA Client Support

vSTP IDPR MOSMS

vSTP EIR Enhancements

### vstp GTT Throttle action

Table 17 – VSTP GTT Throttle Action Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29152322 | The GTT Throttle action is part of SS7 security firewall. It provides support for Egress throttling of GTT messages in vSTP | Enhancement Request |

### vstp GTT scpval action

Table 18 – VSTP GTT SCPVAL Action Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29152322 | The SCPVAL GTT action along with relevant parameters performs the validation on MAP parameters by comparing the SCCP and MAP digits. | Enhancement Request |

Table 19 – VSTP GTT SCPVAL Action Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29152322 | The SCPVAL GTT action along with relevant parameters performs the validation on MAP parameters by comparing the SCCP and MAP digits. | Enhancement Request |

### vstp MTP based gtt

Table 20 –VSTP MTP Based GTT Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29115539 | This feature provides the capability of performing SCCP services on MTP-routed messages. Therefore, allows the operator to perform GTT and GTT Actions on MTP Routed MSUs, similar to GTT handling for GT Routed MSUs. | Enhancement Request |

### vstp sfapp stateful security

Table 21 – VSTP SFAPP Stateful Security Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: | SS7 Firewall - Stateful Applications (SFAPP) allows vSTP to validate the messages coming in for a subscriber by validating them against the Visitor Location Register (VLR). The last seen details of the subscriber can be fetched from the Home Location Register (HLR). Once the HLR provides a validity of the new VLR, vSTP then allows the message into the network. If the message is not validated, it is handled as per configuration (either silent discard, fallback, or respond with error). | Enhancement Request |

### vstp TDm support

Table 22 – VSTP TDM Support Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: | This feature was introduced in 8.4.0.3.1 patch release. The feature provides access to E1/T1 links based ADAX HDC3 PCIe TDM Card using PCIe Pass-through. | Enhancement Request |

### vstp M3UA Client support

Table 23 – VSTP M3UA Client Support Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29113802 | The MTP3-User Adaptation (M3UA ) Client support allows vSTP to trigger the M3UA connection initiation. For information related to M3UA Protocol | Enhancement Request |

### VSTP IDPR MOSMS

Table 24 – VSTP IDPR MOSMS Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29265575, 29302872, 29302882, 29302888, 29302902, 29265587, 29152427 | The Prepaid IDP Query Relay feature (IDP Relay) provides a mechanism to ensure the correct charging for calls from prepaid subscribers in a portability environment. The Mobile Originated Short Message Service (MO SMS) features address the number portability requirements of wireless network operators for delivery of Mobile Originated SMS messages. The vSTP 5 ISS MO SMS features apply number portability database lookup to SMS messages for GSM networks, validates subscriber use of the correct Short Message Service Center, and delivers messages to Prepaid Servers if either the Calling Party Number or Called Party Number is associated with a prepaid subscriber. | Enhancement Request |

### vstp eir enhancements

Table 25 – VSTP EIR Enhancements Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29651069, 29651081 | This feature allows using EirOptoins MO to enable white listed IMSI/IMEI logging. Instead of IP, vSTP logs the OPC in the logs. Instead of IP, DEIR logs the Origin Host and Origin Realm in the logs. OPC/Origin Host and Origin Realm is logged by default. There are no options to enable/disable logging OPC/Origin Host and Origin Realm. Total 100K IMSI white listing is supported on vSTP and DEIR (Diameter EIR) | Enhancement Request |

## Enhancements to DSR 8.4.0.5.0

Note: For information on upgrade planning and required steps before upgrade, please refer to the DSR 8.4 Software Upgrade Guide on the public Oracle Documentation Site:

Docs.oracle.com  Industries  Oracle Communications  Diameter Signaling Router  Release 8.4.0.5.0.

Table 26 - DSR 8.4.0.5.0 New Features/Enhancements

DSR 8.4.0.5.0 Feature/Enhancement Name

vSTP SLS Rotation

vSTP_SFAPP_DynamicError! Hyperlink reference not valid.

vSTP Tif Support

vSTP Segmented XUDTError! Hyperlink reference not valid.

vSTP Duplicate Point Code SupportError! Hyperlink reference not valid.

vSTP GUI Configuration

vSTP IR21 Bulk Upload for SS7 Security

DSA with UDR

### VSTP sls rotation

Table 27 – VSTP SLS Rotation Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29153597 | The Signaling Link Selection(SLS) Rotation feature facilitates a proper distribution of SLS values to provide a good distribution of traffic and load sharing across links and linksets. | Enhancement Request |

### VSTP sfapp dynamic learning

Table 28 – VSTP SFAPP Dynamic Learning Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 30087663 | The Stateful Security Dynamic Learning feature enables vSTP to create and use a whitelist that is created as part of learning from the validation attempts defined in VLR Validation. | Enhancement Request |

### VSTP tif support

Table 29 – VSTP TIF Support Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 30087787 | For TIF features, TIF provides an overall structure that allows the vSTP to intercept ISUP messages that would normally be through-switched and apply special processing to them. For example, an IAM message could be intercepted and have the called number prefix replaced based on portability information. | Enhancement Request |

### VSTP segmented xudt

Table 30 – VSTP Segmented XUDT Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: 29265596 | The Segmented XUDT feature allows vSTP to perform the following operations: - Reassembly of incoming XUDT Class 1 SCCP segmented messages - Segmentation of the outgoing XUDT Class 1 SCCP reassembled messages | Enhancement Request |

### VSTP duplicate point code support

Table 31 – VSTP Duplicate Point Code Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POA: 30087756 | The Duplicate Point Code support functionality allows vSTP to route traffic for two or more countries that may have overlapping point code values. | Enhancement Request |

### vSTP GUI configuration

Table 32 – VSTP GUI Configuration Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: | The vSTP configurations and Maintenance operations can be performed using VSTP GUI on Active System OAM (SOAM). vSTP configuration GUI allows you to manage vSTP configuration. You can perform different tasks on an Active System OAM (SOAM). The VSTP > Configuration folder contains the tables used in vSTP operations. To configure a specific table, select the table name from the list to display the table details. vSTP maintenance pages display status information for Links, RSPs, Connections, Linksets, and SCCP applications. | Enhancement Request |

### vstp ir21 bulk upload for ss7 security

Table 33 – VSTP IR21 Bulk Upload for SS7 Security

| Name | Description | Scope |
| --- | --- | --- |
| POR: | This feature allows a vSTP to provides security to detect anomalies on inbound packets through bulk upload of customer IR.21 documents. | Enhancement Request |

### dsa with udr

Table 34 – VSTP GUI Configuration Feature Description

| Name | Description | Scope |
| --- | --- | --- |
| POR: | Diameter Security Application (DSA) has implemented various Countermeasures to detect vulnerability in an ingress diameter message from a foreign network. The Countermeasures can be divided into two categories. Stateful Countermeasure Stateless Countermeasure | Enhancement Request |

## HARDWARE CHANGES

### Hardware Supported

Table 35 - Hardware Details

| Hardware | Comment |
| --- | --- |
| HP BL460c Gen8, Gen8_v2 | c-Class |
| HP BL460c Gen9, Gen9_v2 | c-Class |
| HP DL360/380 Gen8, Gen8_v2 | Rack Mount Server |
| HP DL380 Gen9, Gen9_v2 | Rack Mount Server |
| Oracle Server X5-2 | Rack Mount Server |
| Oracle Server X6-2 | Rack Mount Server |
| Oracle Server X7-2 | Rack Mount Server |
| Netra X5-2 | Rack Mount Server |
| HP 6125XLG, 6125G, 6120XG | Enclosure Switch |
| Cisco 3020 | Enclosure Switch |
| Cisco 4948E-F | Rack Switch |
| Cisco 4948E | Rack Switch |

Note:

Gen9, Gen9 v2 and Gen 8 v2 hardware are also supported, when purchased by a customer.

Mixed Sun/HP deployments are not generally supported.

### Hardware Upgrade

The VEDSR 8.4 release builds on top of the RMS and provides the support for the newer and higher capacity X7-2 RMS hardware.

## Software Details

### Software Platform Components in 8.4.0.0.0

Software changes include a new release of the software Platform components, and new DSR release.

Table 36 – Software Platform Component Details – 8.4.0.0.0

| Component | Release |
| --- | --- |
| TPD 64 Bit | 7.6.1.0.0-88.55.0 |
| COMCOL | 7.5.0.14.0-14027 |
| APIGW | 8.4.0.0.0_84.15.0 |
| PM&C | 6.6.1.0.0-66.9.0 |
| TVOE | 3.6.1.0.0-88.55.0 |
| AppWorks | 8.4.0-84.11.0 |
| EXGSTACK | 8.4.0-84.12.0 |
| HP Firmware FUP | 2.2.13 (Minimum) |
| Oracle Firmware | 3.1.8 (Minimum) |

### Software Platform Components in 8.4.0.3.0

Software changes include a new release of the software Platform components, and new DSR release.

Table 37 – Software Platform Component Details – 8.4.0.0.0

| Component | Release |
| --- | --- |
| TPD 64 Bit |  |
| COMCOL |  |
| APIGW |  |
| PM&C |  |
| TVOE |  |
| AppWorks |  |
| EXGSTACK |  |
| HP Firmware FUP | 2.2.13 (Minimum) |
| Oracle Firmware | 3.1.8 (Minimum) |

### Software Platform Components in 8.4.0.5.0

Software changes include a new release of the software Platform components, and new DSR release.

Table 38 – Software Platform Component Details – 8.4.0.5.0

| Component | Release |
| --- | --- |
| TPD 64 Bit |  |
| COMCOL |  |
| APIGW |  |
| PM&C |  |
| TVOE |  |
| AppWorks |  |
| EXGSTACK |  |
| HP Firmware FUP | 2.2.13 (Minimum) |
| Oracle Firmware | 3.1.8 (Minimum) |

### DSR Release 8.4

DSR Release 8.4 inherits all functionality from DSR 8.3

Table 39 - Release Details

| Component | Release |
| --- | --- |
| DSR Release | 8.4 |

### iDIH 8.2.1 and 8.2.2

Table 40 - IDIH Details

| Component | Release |
| --- | --- |
| IDH Release | 8.2.1, 8.2.2 |

DSR 8.4 is compatible with IDIH 7.3, 8.0, 8.1, 8.2, 8.2.1 and 8.2.2

### SDS 8.4

Table 41 - SDS Details

| Component | Release |
| --- | --- |
| SDS Release | 8.4 |

DSR 8.4 is compatible with SDS 8.0.1, 8.2.1, 8.2.1, 8.3 and 8.4

NOTE: It is recommended for SDS to be upgraded before the DSR. SDS release 8.4 is compatible with DSR releases 8.0.1, 8.1.1, 8.2.1 and 8.3

## FIRMWARE CHANGES

Firmware release guidance is provided via DSR 8.4 Release Notice which can be found at the following link:

http://docs.oracle.com/en/industries/communications/diameter-signaling-router/index.html

and then navigating to the Release 8.4.x link.

## UPGRADE OVERVIEW

This section provides an overview of the Upgrade activities for Release 8.4.

### DSR Upgrade Path

The supported upgrade paths for DSR 8.4 are:

All in the figure above refers to the available releases and all of its maintenance releases

Figure 1 – DSR Upgrade Paths

### The supported upgrade paths for SDS 8.4

The supported upgrade paths for SDS 8.4 are

All in the figure above refers to the available releases and all of its maintenance releases

Figure 2 – SDS Upgrade Paths

|  | !!Caution!! | SDS Upgrade If the customer deployment has only FABR features enabled, it is recommended to upgrade the SDS nodes first before upgrading the DSR nodes. If the customer deployment has both the FABR and PCA features enabled, then upgrade the DSR nodes first before upgrading the SDS nodes. |
| --- | --- | --- |

.

### The supported upgrade paths for iDIH 8.2.1

The supported upgrade paths for iDIH 8.2.1 are

All in the figure above refers to the available releases and all of its maintenance releases

Figure 3 – IDIH Upgrade Paths

iDIH upgrade can be scheduled prior to or following the DSR upgrade. If iDIH upgrade is deferred until after DSR upgrades then any newly captured elements existing within the upgraded DSR will not be decoded by iDIH until after the iDIH upgrade.

### Upgrade Execution

With DSR 8.4, there are multiple methods available for upgrading a site. The newest and most efficient way to upgrade a site is the Automated Site Upgrade feature. As the name implies, this feature will upgrade an entire site (SOAMs and all C-level servers) with a minimum of user interaction. Once the upgrade is initiated, the upgrade will automatically prepare the server(s), perform the upgrade, and then sequence to the next server or group of servers until all servers in the site are upgraded. The server upgrades are sequenced in a manner that preserves data integrity and processing capacity.

Automated Site Upgrade can be used to upgrade the DSR/SDS servers. However, Auto Site Upgrade cannot be used to upgrade PMAC, TVOE, or IDIH servers at a site.

Additionally, there are separate procedures described in the upgrade procedures to support either a manual or automated approach to upgrading any particular server group. When planning upgrades the “Site Upgrade Methodology Selection” section of the upgrade procedure should be carefully reviewed.  The use of the automated methods (Auto Site or Auto Server Group) for DA-MP server groups should be carefully considered regarding potential negative traffic impacts. The ASU enhancement in DSR 8.4 resolves this issue. The user is now instructed to rearrange/add cycles to create a suitable upgrade plan.

### Limitations

When AppEventLog file is full then SOAM/NOAM becomes unstable and shown undefined behavior like:

1. Replication and merging stopped.

2. GUI access stops working.

Also please note that upgrade will fail if utilization of /var/TKLC/rundb partition is more than 70% which may be true in case of larger AppEventLog file size (~5.5 GB in size). To prevent the above listed issues, we need to assign/allocate /var/TKLC/rundb size and AppEventLog file size in sync i.e. AppEventLog file size (plus some delta for other files like MeasStat) should be always less than 70 % of /var/TKLC/rundb partition size.

## Migration of DSR Data

As in prior releases, the existing DSR Data will be preserved during the upgrade.

# Release 8.4.0.0.0 Feature OAM Changes

At the time of upgrade to DSR 8.4.0.0.0 a number of features and enhancements will become visible on the interfaces to the DSR and may change certain existing OAM behaviors of the system.

OAM changes includes: User Interfaces (NO GUI, SO GUI), Measurements Reports, Alarms, and KPIs.

Note: this section covers OAM changes that will be visible after upgrade to the 8.4.0.0.0 release and does not include changes that will be seen only as new Optional Features are activated on the system (post-upgrade activity, and customer specific).

## ALARM GROUP FEATURE

#### 3.1.1 Problem Statement

DSR displays all relevant individual and aggregated alarms for Connection/Peer as per functionality captured by Alarm Aggregation feature. However due to the large size of the network, volume of connection and peer initiating alarms can be high. This limits operator view to monitor the state of crucial managed objects. Operators have requested the feature that allows them to suppress similar type of alarms, after a given threshold.

#### 3.1.2 Overview

This feature allows operator to do the following:

- Define group of peers/connections for which alarm throttling is required

- Define throttling level for each group

This is named as Alarm Group feature. This feature is administratively managed by the operator through GUI.

Note: Alarm Aggregation feature and Alarm Group feature are mutually exclusive for Peer Nodes and Connections.

Overall working design on this feature is as follows:

Figure 4 - Alarm Group Feature Design

#### 3.1.3 Peer Node Alarm Group

A new GUI screen “Peer Node Alarm Groups” allows user to create Peer Node Alarm Groups, associate the peer nodes to a Peer Node Alarm Group and define throttle and abatement threshold (Minor, Major and Critical) level for each Alarm Group. Threshold level of a Peer Node Alarm Group has no impact on the thresholds of other Peer Node Alarm Group,

Figure 5 – Peer Node Alarm Group GUI

Given below is the high level working of Peer Node Alarm Group:

- New task “PeerNodeAlarmGroup” is added in dsroam process. This task is running on Active SOAM.

- This task checks the administrative state of Alarm Group feature in every 10 seconds.

- If Alarm Group feature is enabled, then the task proceeds further for the processing. If Alarm Group feature is disabled, there is no alarm reporting for peer node failures or threshold condition by “PeerNodeAlarmGroup” task.

- Underlying steps are performed if Alarm Group feature is enabled:

  - Read the configuration data of Peer Node Alarm Group(s). This includes peer nodes in a Peer Node Alarm Group and the threshold level of that alarm group. It also reads the Peer Node status table to see if peer node alarm is set by DA-MP.

Please note that peer node status is only reported by MP Leader. In case of no MP leader detection or multi-MP leaders condition, (No / Multi) MP leader alarm is raised. There is no reporting of individual peer alarms or group threshold alarm.

  - Raise the alarm for every failed peer node in an alarm group until throttling level of that Peer Node Alarm Group is not reached.

  - Once throttle condition is met for a Peer Node Alarm Group, peer node failure alarms for the peers in that alarm group are cleared. This also raises a group alarm to signify that throttle threshold condition for the Peer Node Alarm Group has been reached. Threshold level of a Peer Node Alarm Group is unaffected by threshold of other Peer Node Alarm Group.

  - There are no alarms raised for individual peer node failures in a Peer Node Alarm Group until group threshold alarm of Minor or Major or Critical severity is present.

  - When number of failed peer nodes in a Peer Node Alarm Group drops below the abatement minor threshold, then threshold alarm for a Peer Node Alarm Group is cleared. Further, individual alarms for peer node failures in that Peer Node Alarm Group are asserted.

  - DSROAM process does not have alarm reporting for a peer not added in any Peer Node Alarm Group.

  - If peer node is associated to a Peer Node Alarm Group, then Peer Node Status screen displays its associated Peer Node Alarm Group Name and the id of the alarm raised by DA-MP against the peer node.

- Underlying steps are performed if Alarm Group feature is disabled:

  - Clear the Peer Node Alarm Group threshold alarm if it had been raised before.

  - Individual peer node failure alarms remain unaffected.

#### 3.1.4 Connection Alarm Group

A new GUI screen “Connection Alarm Groups” allows user to create Connection Alarm Groups, associate the connections to a Connection Alarm Group and define throttle and abatement threshold (Minor, Major and Critical) level for each Alarm Group. Threshold level of a Connection Alarm Group has no impact on the thresholds of other Connection Alarm Group,

Figure 6 – Connection Alarm Group GUI

Given below is the high level working of Connection Alarm Group:

- New task “ConnectionAlarmGroup” is added in dsroam process. This task is running on Active SOAM.

- This task checks the administrative state of Alarm Group feature in every 10 seconds.

- If Alarm Group feature is enabled, then the task proceeds further for the processing. If Alarm Group feature is disabled, there is no alarm reporting for connection failures or threshold condition by “ConnectionAlarmGroup” task.

- Underlying steps are performed if Alarm Group feature is enabled:

  - Read the configuration data of Connection Alarm Group(s). This includes connections in a Connection Alarm Group and the threshold level of that alarm group. It also reads the Connection status table to see if connection alarm is set by DA-MP.

Please note that floating connection status is only reported by MP Leader. In case of no MP leader detection or multi-MP leaders condition, (No / Multi) MP leader alarm is raised. Consequently, there is no reporting of floating connections.

Fixed connection status is reported by the MP owning the connection. Fixed connection status is not affected by No MP leader or multi-MP leader conditions. Consequently, there is reporting of fixed connection status.

  - Raise the alarm for every failed connection in an alarm group until throttling level of that Connection Alarm Group is not reached.

  - Once throttle condition is met for a Connection Alarm Group, connection failure alarms for the connections in that alarm group are cleared. This also raises a group alarm to signify that throttle threshold condition for the Connection Alarm Group has been reached. Threshold level of a Connection Alarm Group is unaffected by threshold of other Connection Alarm Group.

  - There are no alarms raised for individual connection failures in a Connection Alarm Group until group threshold alarm of Minor or Major or Critical severity is present.

  - When number of failed connections in a Connection Alarm Group drops below the abatement minor threshold, then threshold alarm for a Connection Alarm Group is cleared. Further, individual alarms for connection failures in that Connection Alarm Group are asserted.

  - DSROAM process does not have alarm reporting for a connection not added in any Connection Alarm Group.

  - If connection is associated to a Connection Alarm Group, then Connection Status screen displays its associated Connection Alarm Group Name and the id of the alarm raised by DA-MP against the connection.

- Underlying steps are performed if Alarm Group feature is disabled:

  - Clear the Connection Alarm Group threshold alarm if it had been raised before.

  - Individual connection failure alarms remain unaffected.

### Alarm Aggregation interworking with Alarm Group feature

## TIME DISTANCE CHECK

#### Description

if a S6a/d ULR/AIR is received for an Outbound Roamer and an earlier ULR has already been received for this Roamer and the time difference between the previously received ULR’s VPLMN-Id/MCC and the current ULR/AIR’s VPLMN-Id/MCC is less than the minimum transition time configured in Time Distance Check Config Table then current received ULR/AIR will be considered as Vulnerable by Time-Distance-Check CounterMeasure and configured action will be performed as defined

#### Overview

The feature enhance DSA logic to find whether Outbound Roamer is Vulnerable or not. Following tables has to be configured.

Table 42 – Security Countermeasure Config

| Admin Status | Admin_Status defines the current Admin State of the countermeasure. If the Admin_Status is configured as Enable, then only the countermeasure business logic is executed. If the Admin_Status is configured as Disable, then the countermeasure business logic is not executed. |
| --- | --- |
| Operating Mode | Defines the action taken if a message is found to be vulnerable by the countermeasure. If the Operating_Mode is configured as Detection_Only, then the countermeasure works on monitoring mode. The vulnerable message is only reported to the user. DSA further processes the message (depending upon Continue If vulnerable configuration) for executing the next available countermeasure. If the Operating_Mode is configured as Detection_And_Correction_By_Drop, then the vulnerable diameter message is discarded at DSR and is not processed/relayed any further. If the Operating_Mode is configured as Detection_And_Correction_By_Send_Answer, then the vulnerable diameter message is rejected by DSR by sending an Error Answer and is not processed/relayed any further. |

Table 43 -System Config Options Table

| MCC or VPLMN-ID | Indicates the source and destination node IDs configured in Error! Reference source not found. are MCCs or VPLMN-IDs. If MCC_Or_VPLMNID is configured as MCC_Based, then the source and destination node IDs are treated as MCC values. If MCC_Or_VPLMNID is configured as VPLMNID_Based, then the source and destination node IDs are treated as VPLMN-ID values. |
| --- | --- |

Table 44 -TimeDistChk_Config Table

| Source and Destination Node-IDs | Defines the two Node-ID values. Node-ID can be MCC or VPLMN-ID of any given network. Value of MCC_Or_VPLMNID configured in Error! Reference source not found. determines that the configured Node-IDs is MCCs or VPLMN-IDs. If MCC_Or_VPLMNID is configured as MCC_Based, then the Node_ID_1 and Node_ID_2 are treated as MCC values. If MCC_Or_VPLMNID is configured as VPLMNID_Based, then the Node_ID_1 and Node_ID_2 are treated as VPLMN-ID values. |
| --- | --- |
| Minimum Transition Time | Defines the minimum transition time (in minutes) required to move between Node_ID_1 and Node_ID_2. |

Table 45 – TimeDistChk Continent Config

| Continent_1 and Continent_2 | List of Various supported continents. |
| --- | --- |
| Minimum Transition Time | Minimum Transition time [in Minutes] between the Continent_1 and Continent_2.[Range = 0 - 720] |

Table 46 – TimeDistChk MCC Config

| Node_Id_1 and Node_Id_2 | Node_Id is MCC .[Range = 3digits Integer] |
| --- | --- |
| Minimum Transition Time | Minimum Transition time [in Minutes] between the Node_Id_1 and Node_Id_2.[Range = 1 - 4320] |

#### GUI Changes

DCA Framework Screen (Main Menu > DCA Framework > Application Control > Version > [Click on “Config”])

Figure 7 – DCA Framework Screen

#### Behavior

Minimum Transit Time between country X and Y is preconfigured as ‘d’ time units at DSA into Minimum Transit Time Table. This configuration shall be used to perform time distance check.

- Mobile subscriber is in home network X and performs updated location procedure with home network HSS

- Mobile subscribers now roams into visited network Y outside home network. Visited network MME/SGSN initiates the AIR/ULR as the part of authentication and update location procedure.

- DSA receives the AIR/ULR from the visited network MME/SGSN and saves the timestamp Y = a for later timestamp comparison. DEA shall initiate the Send Routing Information Request for LCS on SLh interface towards home network HSS to know the last know serving node ( i.e. MME/SGSN) information back in the home network.

HSS responds back with last serving node i.e. MME/SGSN identity of home network. Here DEA acts as an GLMC node for communicating with HSS.

- DEA sends the S6a/d interface IDR message to last serving MME/SGSN network element of home network to retrieve the last update location timestamp. Home network serving node identity is retrieved in Step 3 from HSS. DEA requests the location information( which also includes the last unknown attach timestamp) from last home network MME/SGSN by setting ‘EPS Location Information Request’ IDR Flag bit in IDR request sent to MME/SGSN. Home network MME/SGSN shall include the last unknown update location timestamp using Age-Of-Location-Information AVP embedded into MME-Location-Information/SGSN-Location-Information   AVP of IDA response. DEA shall store the home network last update location timestamp X = b.

## VNFM

#### Description

The DSR VNFM 3.0 provides support for following operations:

- Instantiate Network OAM VNFs with fixed IPs

- Instantiate Signaling VNFs with Multiple XSIs for fixed IPs

- Instantiating DSR DR NOAM

- Instantiating SDS DR NOAM

- Scale VNF

-Scale VNF to Level (Scale Out C Level servers of Signaling VNF)

-Scale VNF to Arbitrary size (Scale Out C Level servers of Signaling VNF)

- Query Individual / All LCM Operation(s) Terminating VNF

- Discover VNF

- Terminate VNF

#### Instantiate Network OAM VNFs with fixed IPs

VNFM supports both the fixed and dynamic IP support. In order to bring up the new VNFM with the same IP as the existing one, the user can use FIXED IP deployment model.

#### Instantiate Signaling VNFs with Multiple XSIs for fixed IPs

Signaling VNF supports both dynamic and fixed IP deployment with multiple XSIs support for both.

###### DSR Signaling VNF with Multiple XSI support (1,2 & 4 xsi interface only)

- Multiple XSI support only DSR Signaling node.

- DAMP vnf will support 1 ,2 & 4 xsi interface.

- STPMP vnf will support 1, 2, & 4 xsi interface.

- IPFE vnf will support 1, 2, & 4 xsi interface.

- UDR vnf will support only 1 & 2 xsi interface.

- While passing the xsiNetwork through request body. Add list of network in the xsiNetwork.

#### Scale VNF

It is a N/B LCM scale_to_level Rest I/F which helps in Scaling already created VNF's .

Following are the two options while scaling using "scale to VNF level"  N/B Interface.

1. Scale VNF to Level based on pre-defined sizes (using Instantiation level Id) .

2. Scale VNF to Level with arbitrary sizes ( using scaleInfo).

Note:

- This feature is only supported for Scaling out C-level servers of Signaling Stack.

- The stack must have been instantiated prior to performing scale to level operation.

- Before  Scaling the VNF to level, VnfInstance Id of the stack must be available.

- The instantiation level for Signaling stack is available under Instantiating the first signaling VNF section.

- Currently, we do not support the cloud-init (appworks configurations) for the scaled out VMs. It will supported shortly.

- Scale To Level Request accepts either instantiationLevelId or scaleInfo

- Cross deployment scaling is not supported by VNFM - if the user instantiated the VNF in fixed IP deployment model, then he must scale to level using FIXED IP deployment model only and vice versa.

The following image illustrates the VNF Scaling:

Figure 8 – VNF Scaling

#### Query Individual / All LCM Operation(s) Terminating VNF

This resource represents VNF lifecycle management operation occurrences. This resource can be used to query status information about multiple VNF lifecycle management operation occurrences.

The diagram describes a sequence for querying/reading information about a VNF LCM Operation.

Figure 9 – VNF LCM Operation

Below are the two ways to query LCM Operation

1) Query individual LCM Operation

2) Query All LCM Operation

#### Discover VNF

1. It is an LCM Discover Rest I/F.

2. It is used to discover the created stack in OpenStack and save the stack information (parameter file and VNF instance) in the VNFM persistent directory. This information can be used for further requests by the orchestrator. For example, to scale out the stack. This information can be used to further request by the orchestrator . e.x :- To scale out the stack.

3. Before Discovering the Stack, the following information must be available:

- The Stack ID for a previously created stack.

- Information about the OpenStack instance on which the Stack must be discovered:

  - OpenStack Controller URI

  - Domain name

  - Username

  - Password

  - Tenant name

4. The Interface discovers the stack and performs below operations:

- Download the Parameter file of discover stack

- Create the Instance file of discover stack.

- These two files will be save in "/var/vnfm/instances/<autoDiscovery InstanceId>/" directory

#### Terminating VNF

This resource represents the "Terminate VNF" operation. The client can use this resource to terminate a VNF instance. The POST method terminates a VNF instance.

Two types of TerminateVnfRequest that represents request parameters for the "Terminate VNF" operation:

- FORCEFUL: The VNFM will delete the VNF and release the resources immediately after accepting the request.

- GRACEFUL: After accepting the request, VNFM will first validate if the VNF configuration is cleaned up . Once the validation is successful, VNFM will delete the VNF and release the resources.

###### Forceful Termination

The VNFM will delete the VNF immediately after accepting the request. The instance file is updated with VNF Operational State set to "STOPPED",

Note: If the VNF is still in service, requesting forceful termination can adversely impact the network service.

Figure 10 – VNF Forceful Termination

###### Graceful Termination

The VNFM will first validate if the VNF configuration is cleaned up after accepting the request. If the configuration is cleaned up , the VNFM will delete the VNF. Then the instance file is updated with VNF Operational State set to "STOPPED".

If appworks configurations not cleaned up manually and orchestrator tries to do gracefull termination for that VNF, then termination of VNF will fail.

Note: User must manually clean up the AppWorks configurations before doing Graceful Termination.

Steps for cleaning up the AppWorks Configuration for Signaling Stack of DSR and SDS:

- Open corresponding Active NOAM GUI of the Signaling instance.

- In Status & Manage Tab, under HA - edit the Max Allowed HA Role of instances of the Signaling stack as OOS.

- In Configuration Tab, under Server Groups, edit the corresponding server groups of the instances and uncheck SG Inclusion for the Server, and press OK. After this step, the excluded Servers must disappear in Status & Manage -> Server section.

- Finally, go to Configuration -> Servers section, select the servers that are to be deleted and press the delete button.
Note: 'GRACEFUL' termination is not supported for DR NOAM and SDS DR NOAM. Although, tried to perform then this scenario will be treated as 'FORCEFUL' termination and the stack will be deleted.

Figure 11 – VNF Graceful Termination

## SCEF

LWM2M Gateway:

LwM2M server is intended to handle IP device connectivity of IoT network via LwM2M protocol over CoAP. The server provides a unified interface T8 towards SCS A/S to handle communication with LwM2M devices.Lwm2M server is developed using communication service framework in OCSG.

LWM2M Device registration process

When LwM2M device is registered, LwM2MServer will send a notification to configured SCS/AS Url.

SCS/AS Url can be configured to receive registration notification for a specified device id or based on domain.

If no SCS/AS Url is configured for the device registration messages received then the device will be registered with LWM2MServer and will not be notified to SCS/AS.

Figure 12 – LWM2M Registration Process

LwM2M device registration process

Monitoring Event

Create event subscription to monitor a LwM2M device resource (using T8 Monitoring Event API)

SCS/AS can send monitoring event subscription message to API Gateway to monitor a device resource.

If the device is not registered for which the subscription request is received, HTTP 404 Device not found error will be returned.

If the device is registered in LwM2M server, then LwM2M observe message will be created sent to Device. A unique subscription identifier will be generated by server and returned in response.

The subscription will be active in LwM2M server until SCS/AS delete the request.

Figure 13 – Subscription and Observe

LwM2M event subscription process

Subscription event notification:

When device send notification to the observe request, the stored subscription request will be fetched and notification request will be generated and sent to the callback Url sent in subscription request.

Figure 14 – Event Notification

LwM2M event subscription notification process

Delete subscription request (using T8 Monitoring event API)

SCS/AS can send HTTP Delete message to stop monitoring of the device resource.

If the subscription id sent is wrong then HTTP 404 error code should be returned.

If the subscription id sent is correct then LwM2M cancel request should be sent to Device to stop the observation.

Figure 15 – Subscription and cancel

LwM2M event subscription delete process

NIDD Downlink Data Transfer

Sending data to device (MT Data using T8 Nidd API)

SCS/AS can send data to LwM2M device using Nidd Downlink Data transfer request.

When the Downlink data transfer request is sent LwM2M server will send corresponding LwM2M message to the device.

If the device is not registered with the server then HTTP 404 message should be returned.

Figure 16 – Action on DL data transfer

LwM2M MT Data delivery process

MQTT Broker:

High-level functional requirements of SCEF MQTT Broker includes MQTT IP Device Monitoring event procedure that includes Unified T8 interface for IP devices.

MQTT Broker is intended to handle IP device connectivity of IoT network via MQTT protocol. The broker provides a unified interface T8 towards SCS A/S to handle communication with MQTT devices.

MQTT Broker is developed using communication service framework in OCSG.

Create event subscription to monitor a MQTT device (using T8 Monitoring Event API)

If the device is not connected for which the subscription request is received, HTTP 404 Device not found error would be returned.

If the device is connected to MQTT Broker, then a unique subscription identifier will be generated by broker and returned in response.

The subscription will be active in MQTT Broker until delete request is sent.

Subscription event notification:

When SCS/AS publish the data, the stored subscription request will be fetched and notification request will be generated and sent to the callback Url sent in subscription request.

Figure 17 – T8 Event

MQTT Monitoring data call flow

Delete subscription request (using T8 Monitoring event API):

MQTT Device can send HTTP Delete message to MQTT Broker to delete subscription of the device.

If the subscription id sent is wrong then HTTP 404 error code should be returned.

If the subscription id sent is correct then subscription of the MQTT device will be deleted and response ok is returned.

Figure 18 – DELETE Subscription

MQTT Event subscription delete process

QoS Control

The QoS Control feature in SCEF allows SCS/AS to setup an AS session with required QoS and priority handling. In order to achieve this SCEF will act as AF to PCRF and setup the session over Rx interface.

AS Session Setup

Below diagram illustrates the procedure of AS session setup initiated by SCS/AS to a given user as identified using the IPv4 or IPv6 address.

SCS/AS sends AS Session setup request with required QoS subscription to SCEF over T8 interface. SCEF processes the request and sends a Rx AAR message towards PCRF to create AS session with requested QoS and priority signaling.

When PCRF sends a success response in Rx-AAA. Then a session record is created in the database and a resource location (URI) is sent to SCS/AS in the response.

The SCS/AS shall use the URI received in the Location header in subsequent requests to the SCEF to refer to this AS session. Otherwise, the SCEF shall send an HTTP response to the SCS/AS with a corresponding status code and include the result in the body of the HTTP response.

PCRF

SCEF

SCS/AS

T8 POST request

Rx AAR (Initial)

Rx AAA

T8 Response

PCRF

SCEF

SCS/AS

T8 POST request

Rx AAR (Initial)

Rx AAA

T8 Response

Figure 19 – T8 Request and Response

AS Session Setup

#### AS Session Modify

The below figure illustrates AS Session modify procedure to replace existing QoS properties. In order to update the established AS session, the SCS/AS sends an HTTP PUT message to the resource "Individual AS Session with Required QoS Subscription" requesting to replace all properties in the existing resource, addressed by the URI received in the response to the request that has created the resource. The UE IP address has to remain unchanged from previously provided values. After receiving such message, SCEF will make the change and interact with the PCRF to modify the Rx session by triggering an Rx-AAR.

PCRF

SCEF

SCS/AS

T8 PUT request

Rx AAR (Update)

Rx AAA

T8 Response

PCRF

SCEF

SCS/AS

T8 PUT request

Rx AAR (Update)

Rx AAA

T8 Response

Figure 20 – PUT Operation

AS Session modify

#### AS Session Remove

Figure illustrates AS Session remove initiated by SCS/AS. To remove the AS session SCS/AS sends an HTTP DELETE message to the resource "Individual AS Session with Required QoS Subscription". After receiving the HTTP DELETE message, the SCEF interacts with the PCRF to terminate the Rx session by sending an Rx-STR.

After success response from PCRF, the AS session data stored in database is cleared. After this a success response is sent back to SCS/AS stating that the AS session is removed successfully.

PCRF

SCEF

SCS/AS

T8 DELETE request

Rx STR

Rx STA

T8 Response

PCRF

SCEF

SCS/AS

T8 DELETE request

Rx STR

Rx STA

T8 Response

Figure 21 – DELETE Operation

#### AS Session QoS notification

When SCEF receives Rx ASR from PCRF, it gets informed that the Rx session is terminated (e.g. due to a release of PDN connection). SCEF sends STR towards PCRF and then it will send an HTTP POST message including the notified event (session terminated) and the accumulated usage (if received from the PCRF) to the callback URI "notificationUri" provided by the SCS/AS during the creation of individual AS Session with Required QoS Subscription. The SCS/AS shall respond with an HTTP response to confirm the received notification.

The event notification generated when PCRF terminates the Rx session. In this case PCRF sends Rx-ASR to SCEF, it verifies if the session exists in the database. Once the session data is fetched from the database a notification is sent to SCS/AS with details of the event. After a response is received from SCS/AS the session data in the database will be cleared.

PCRF

PCRF

SCEF

SCEF

SCS/AS

SCS/AS

Rx ASR

Rx ASR

Rx RAA

Rx RAA

T8 POST (notify) request

T8 POST (notify) request

Rx STR

Rx STR

Rx STA

Rx STA

T8 Response

T8 Response

Figure 22 – T8 POST Operation

AS Session termination event notification

#### API Based Charging

SCEF shall support API based charging for the following procedures that operate across the T8 reference point.

In addition, charging shall be implemented based on Offline event based charging mechanism. Moreover, the event are.

  - NIDD Events

  - Monitoring Events

  - Device Triggering Events

  - Enhanced coverage Restriction Events.

The SCEF shall support Offline charging for API based charging. On the other hand, in offline charging, an user is charged for the network resources that is already used. That is, the network reports the resource usage by the particular user by forwarding the CDR (Charging Data Record) to its billing domain.

SCEF shall support Event based charging function (EBCF). EBCF is based on their occurrence rather than their duration or volume used in the event. Typical SCEF events are Nidd/Monitor/Device Trigger/ECR.

SCEF shall create an exposure function API CDR in real time (up to 1 second) for each event when the API invocation or notification encountered.

Invocation:

SCS/AS shall send T8 request (Nidd/Monitor/Device Trigger/ECR) to OCSG, which is part of SCEF.

OCSG forwards the incoming T8 request to serving SCEF-MP.

SCEF-MP will act on the message and send back response message to OCSG.

Now OCSG will generate a CDR with available data and write into a file in binary format. Then forward that response to SCS/AS.

SCEF-MP

OCSG

SCS/AS

SCEF

CDR will be generated for either Success or failure response of Nidd/Monitor /DT/ECR

Nidd/Monitor/DT/ECR Request

Nidd/Monitor/DT/ECR Request

Response

Response

SCEF-MP

OCSG

SCS/AS

SCEF

CDR will be generated for either Success or failure response of Nidd/Monitor /DT/ECR

Nidd/Monitor/DT/ECR Request

Nidd/Monitor/DT/ECR Request

Response

Response

Figure 23 – API based charging for Invocation events

Notification:

SCEF-MP may receive notification message from MME/HSS if SCEF subscribed for certain events.

SCEF-MP shall send that notification request (Nidd/Monitor/Device Trigger) to OCSG.

Now OCSG will generate a CDR with available data and write into a file in binary format. Then forwards the request to SCS/AS.

Moreover, the subsequent SCS/AS response will be forward to SCES-MP via OCSG.

SCEF-MP

OCSG

SCS/AS

SCEF

CDR will be generated for either Success or failure response of Nidd/Monitor /DT/ECR

Notify/Uplink Request

Nidd/Monitor/DT Notify

Response

Response

SCEF-MP

OCSG

SCS/AS

SCEF

CDR will be generated for either Success or failure response of Nidd/Monitor /DT/ECR

Notify/Uplink Request

Nidd/Monitor/DT Notify

Response

Response

Figure 26

Figure 24 – Notify and Uplink Request

SCEF T8 Compliance Changes

Changes were made to the existing APIs (NIDD (Non-IP Data Delivery), Device Triggering, Monitoring Events, Enhanced Coverage Restriction (ECR) Control) to comply with specifications 3GPP TS 29.122 T8 Reference Point for Northbound APIs

## VSTP SPARE POINT CODE SUPPORT

#### Purpose and Solution

Purpose

- At present vSTP supports only 4 domain PCs of ANSI/ITU-I/ITU-N/ITU-N24.

- This limits operators from hosting more no. of point codes for ITUI and ITUN domain which triggers                                  the need for more no of nodes in the network and associated provisioning & maintenance.

Solution

Spare PC solution resolves the above limitations by enabling vSTP to new domain PC ITUN Spare and ITUI Spare in addition to the point codes used by the vSTP domains  ITU-N (14-bit or 24-bit) and ITU-I.

Feature Overview

- The vSTP to fully support ITU National Spare and ITU International Spare by providing a new PC sub type named Spare.

- ITUN_S and ITUI_S indicates a Spare point code.  Spare point codes only apply to ITU-I and ITU-N point code types.

- The subservice field contains the network indicator and two spare bits. The network indicator is used by signaling message handling functions.

Table 47 - NI and Name

| NI | NAME |
| --- | --- |
| 00(03) | International |
| 01(43) | International-Spare |
| 10(83) | National |
| 11(c3) | National-Spare |

- The vSTP currently provides full support for four types of point codes (PC)  – ANSI, ITU-National (NI=10binary ), ITU-National 24-bit, and ITU-International (NI=00 binary ).  ITU National Spare PCs (NI=11 binary ) and ITU International Spare(NI= 01 binary) PCs can be primarily supported :

- If the Spare PC feature is enabled then vSTP will validate the NI value of incoming message on designated linkset . If NI value matches then link will be mark available otherwise message will be discarded.

- For example, any incoming message with DPC = 1-1-1 (NI=11binary ) on link set equal to configured ITUN spare DPC = 1-1-1 (NI=11binary ) shall be accepted.

- If the Spare PC feature is disabled then vSTP will allow to accept the In coming messages with NI value ITUI spare on configured ITUI linkset.

- Incoming messages with the Spare bit set will be treated as Non-Spare point codes.

- For example, vSTP will accept a message with DPC = 1-1-1 (NI=11binary ) the same way as a message with DPC = 1-1-1 (NI=10binary ).

MTP Message Routing

- Message destined for DPC When Spare PC feature turned on and No MTP conversion.

  - The NI Value of an outgoing MTP-routed MSU must match the domain type of the APC/SAPC on the corresponding link set over which the MSU is received otherwise error is thrown “MTP3 Routing Error - Invalid NI”.

  - During MTP routing with no MTP conversion, the MSU is simply routed without modification, so the outgoing NI will be the same as the incoming NI.

  - All route selection and MSU processing shall be done using a combination of the DPC and the Network Indicator fields in incoming messages.

Table 48 - Configuration details1

| Spare Feature | Spare Bit on | Configured Domain/NI for APC | Domain /Received NI in MSU | Message Routing |
| --- | --- | --- | --- | --- |
| Yes | Yes | ITUN_S(11) | ITUN_S(11) | MSU routed |
| Yes | Yes | ITUN(10) | ITUN_S(11) | MSU Discarded |
| Yes | Yes | ITUI_S(01) | ITUI_S(01) | MSU routed |
| Yes | Yes | ITUI(00) | ITUI_S(01) | MSU Discarded |

  - If a message is MTP-routed with conversion, the outgoing NI value will be set to the provisioned value of the outgoing DPC.  The incoming NI will only be used to define the incoming DPC as Spare or Non-Spare, in order to determine the correct conversion.

Table 49 - Configuration details2

| Spare Feature | Spare Bit on | Configured Domain/NI for APC | Domain /Received NI in MSU | Message Routing |
| --- | --- | --- | --- | --- |
| Off | Yes | ITUN_S(11) | ITUN_S(11) | MSU Discarded |
| Off | Yes | ITUN(10) | ITUN_S(11) | MSU routed |
| Off | Yes | ITUI_S(01) | ITUI_S(01) | MSU Discarded |
| Off | Yes | ITUI(00) | ITUI_S(01) | MSU routed |

SCCP Message GTT Routing:

- During SCCP Translation via GTT, the outgoing NI will match the GTT-database value for the DPC associated with the CdPA digits to be translated.

- The incoming NI does not affect the digits used for SCCP translation, and thus does not affect the outgoing NI.

  - NOTE: Based on GTT configuration, the Network Conversion feature may convert spare point code into other supported domain.

Signaling Link Test Messages (SLTM/SLTA)

- SLTM/SLTA messages will be performed with the linkset APC, and the NI will match the provisioned value of the APC.

- vSTP shall validate that the DPC of the message matches with the DPC provisioned against the OPC  of the message.

- When Spare PC feature is on, The NI Value of an received message must match the domain type of the APC on the corresponding link set over which the MSU is transmitted.

- When Spare feature is off, Incoming messages with the Spare bit set will be treated as Non-Spare point codes.

- For example , vSTP will route a message with DPC = 1-1-1 (NI=11binary ) the same way as a message with DPC = 1-1-1 (NI=10binary ).

Signaling Link Test Messages (SLTM/SLTA) Spare PC Off

Figure 25 – SLTM Operation accept

Signaling Link Test Messages (SLTM/SLTA) Spare PC On

Figure 26 – SLTM Operation discard

MO’s and operations supported:

- Following is the list of MO’s and operation supported for Spare PC feature :

- ss7DomainType supports ITUN_S and ITUI_S domain.

Table 50 – MO Details 1

| MO Name | Operations supported | URI |
| --- | --- | --- |
| VstpLocalSP | Insert, Update, Delete | /vstp/localsignalingpoints |
| VstpRemoteSP | Insert, Update, Delete | /vstp/remotesignalingpoints |
| VstpSccpGTTSel | Insert, Update, Delete | /vstp/gttselectors |

- SparePCSupportEnabled flag should be Enable to support ITUN_S and ITUI_S domain.

Table 51 – MO Details 2

| MO Name | Operations supported | URI |
| --- | --- | --- |
| m3rloptions | update | /vstp/m3rloptions |

Refer MMI API Guide on Active NOAM/SOAM: “Main Menu ->MMI API Guide” on any DSR GA release setup for details about the URI, example and parameters about each MO.

Figure 27 – MMI API Guide reference

## VSTP MNP GPORT

#### Purpose and Solution

  - Purpose

  - Support for GSM Mobile Number Portability (a.k.a. G-Port) feature in vSTP.

  - Mobile Number Portability (MNP) allows mobile subscribers to retaining their original MSISDN when changing the network to which they subscribe.

  - This feature is an optional feature and can be turned on/off via configurable option in the VstpMnpOptions.

  - Solution

  - G-Port shall perform the following actions based on the message received and number status:

    - If the number is ported-out or not known to be ported and the message received is call-related SRI (not-SOR), G-Port shall send the SRI Ack message to the MSC with the Routing Number information in the MAP portion of the message.

    - If the number is ported-out and the message received is non-call related (non-SRI), G-Port shall perform message relay and forward the translated message based on the Routing Number information.

    - If the number is non-ported or ported-in then G-Port shall perform HLR translation and forward the translated message to the HLR.

  - An additional user option will allow the user to configure the G-Port to modify the above processing in the following way:

    - If the number is not found in the UDR NPDB (individual or range) then the G-Port shall return a negative acknowledgement in response to an SRI.

#### G-Port involves following main functions:

Message discrimination: Since G-Port is currently only used for translation of ported numbers, it provides a method to identify which messages should receive G-Port vs. GTT. This is provided via a service selector table where one can define G-Port service for a combination of selectors. If a selector match is not found then G-Port shall fall-through to GTT.

Number conditioning: Since the UDR NPDB stores International MSISDNs only, G-Port provides the capability to condition incoming numbers to be international MSISDNs (i.e. Delete Routing Number Prefix or Insert CC or/and NDC ) for the database look up. Also, messages with ported-in number in SCCP CdPA or MAP MSISDN might have RN prefix. G-Port shall strip off the RN prefix and then condition the non-international numbers to international numbers, if needed, before performing any database lookup.

SRI Response : When the incoming SRI message does not already contain a RN in the SCCP portion, or if the VstpMnpOption:MNPCRP is off, G-Port shall generate a SRI Ack message for a SRI message when the number is foreign number, and a RN is associated with the DN in the database.  When neither a RN nor a SP is associated with a MSISDN in the database, G-Port shall formulate a SRI Ack message ONLY for “null” PT or “foreign number” PT. For all other cases the SRI shall fall through to GTT.  When formulating a SRI Ack message, G-Port shall use the RN prefix associated with the MSISDN entry to build the MSRN number or based on the VstpMnpOption:MSRNDIG to not prefix shall return the RN only. G-Port shall generate negative SRI Ack message upon encountering any MAP SRI problems.  If VstpMnpOption:MNPCRP is on and the SRI message already contains a RN in the SCCP portion or MAP MSISDN, then G-Port will issue Event # 70304 and the message will fall through to GTT.  Since the CdPA / MAP MSISDN contains RN+DN, this should result in a GTT failure, which will cause a UDTS to be returned to the originator if the Return Message on Error flag was set in the incoming UDT. If MSISDN is provisioned in UDR NPDB with PT values of 0/1/2 (foreign number), the SRI response will have NPS value of 0/1/2. When PT value of “null” is defined, it will be mapped to NPS value of (0) in the SRI response.

G-Port Message Relay: G-Port shall perform message relay on non-SRI or SRI-SOR (SRI messages with OR Interrogation Indicator present) messages when the MSISDN number is ported. Message relay shall provide an ability to prefix the Routing number to the CdPA digits or replace the CdPA digits by RN prefix based on the Digit Actions.  If VstpMnpOptions:SRISMGTTRTG  is ON, then the SRI_SM and ReportSMDeliveryStatus messages shall not be relayed. Instead the CdPA GTA in the message shall be modified in CC+RN+DN format (or RN+IDN format if CC match is not found in leading digits). The NAI of CdPA GTA shall be set to International and the message shall Fall-Through to GTT.

Message relay on ported-in numbers : G-Port shall automatically perform SCCP relay on SRI and non-SRI messages for Own Numbers (i.e. a SP is associated with the DN in the database).  However, the non-ported (PT=4) or ported-in (PT=5) entries should be present in the UDR NPDB and SP entity should have been defined for this entry.  This applies even in the case when VstpMnpOptions:MNPCRP  is on and the message contained an RN in the incoming SCCP.  If GRN is also associated (along with SP entity) with the DN and the GRN is not present in the HomeRN table and  VstpMnpOptions:SRISMGTTRTG  is ON, then the SRI_SM and ReportSMDeliveryStatus messages shall not be relayed. Instead the CdPA GTA in the message shall be modified in CC+GRN+DN format (or GRN+IDN format if CC match is not found in leading digits). The NAI of CdPA GTA shall be set to International and the message shall Fall-Through to GTT.

- G-Port Message Handling:

- The message arrives at vSTP route-on-gt. vSTP decodes the SCCP portion and uses the data to perform G-Port Selection based on the CdPA GT fields other than the ES and GTAI.  The result of this selection provides service indicator. The service indicator will be G-Port if MNP-SRF is required.  If a service selector does not match the incoming GT fields, then GTT selection is attempted.

- If step 1 indicates MNP-SRF is required and the message is not a UDTS generated by vSTP, vSTP then performs SSN-based discrimination. If the message is a UDTS generated by the vSTP then regular GTT is performed on the message.

- MNP-SRF will first decode the Operation Code of the MAP message to distinguish SRI/SRI_SM message with the rest. If the Operation Code is SRI and the OR Interrogation indicator is absent, and the VstpMnpOptions:SRIDN = “TCAP”, then the MSISDN parameter is decoded from the MAP portion of the message. If the Operation Code is SRI_SM, and the VstpMnpOptions parameter SRISMDN = “TCAP”, then the MSISDN parameter is decoded from the MAP portion of the message. If the VstpMnpOptions parameter SRIDN (for SRI message) / SRISMDN (for SRI_SM message) = “SCCP”, or if the message is not SRI or not SRI_SM, then digits available in the SCCP CdPA GTAI are used for database lookup.

- The decoded DN (either from MAP MSISDN or from SCCP CdPA) is then conditioned to an international number before performing the UDR NPDB lookup. The conditioning is different based on whether the digits are obtained from TCAP or MAP part of the message.

- If the digits are from the SCCP part then number conditioning is based on SNAI value. RN prefix deletion is performed first and then conversion to International based on its value.   Conversion to international format is based on DefCC and DefNDC, as required.  If the incoming number is CCRNDN, DefCC and MultCC will be used to determine the Best Match CC to locate the RN digits for RN prefix deletion.

- If the digits are from the MAP part then number conditioning is based on NAI of MSISDN Parameter.  Home RN Prefix deletion is performed if VstpMnpOptions:MNPCRP  is on.  The number is converted to International if needed.  Conversion to international format is based on DefCC and DefNDC, as required.  If the incoming number is international, DefCC and MultCC will be used to determine if the format is CCRNDN or RNIDN. If a Best Match CC is located, then it will be used to locate the RN digits for RN prefix deletion.

- The UDR NPDB database lookup involves 2 steps:

- First the exception or individual number database is searched for a match. If the match is found then the data associated with this entry is considered.

- If the conditioned number is absent in the exception (individual) database, then the number range database is searched. If the match is found then the data associated with this range entry is considered. If the search is unsuccessful then the result is no match.

- If the number is found, and a RN prefix is present for this entry, then:

- If the message is SRI, and VstpMnpOptions:MNPCRP  is off, or if VstpMnpOptions:MNPCRP  is on and a HomeRN was not present in the incoming DN (i.e. a HomeRN was not deleted from the SCCP CdPA/MAP MSISDN), then G-Port shall generate a SRI Ack message with the RN prefix in the Routing Number parameter.

- If the message is non-SRI, and VstpMnpOptions:MNPCRP  is off, or if VstpMnpOptions:MNPCRP  is on and a HomeRN was not present in the incoming DN (i.e. a HomeRN was not deleted from the SCCP CdPA), then G-Port shall use the translation data for the number to alter the CdPA digits and route the message to the destination.

- If the message is SRI or non-SRI, and VstpMnpOptions:MNPCRP  is on, and a HomeRN was present in the incoming DN (i.e. a HomeRN was deleted from the SCCP CdPA/MAP MSISDN), then G-Port shall generate Event #70304, and the message shall fall through to GTT.  In most network implementations, since the message contains RN+DN, this should cause a GTT failure, which will result in the vSTP sending a UDTS to the originator if the Return Message on Error flag was set in the incoming UDT.

- If VstpMnpOptions:SRISMGTTRTG  is ON, then the SRI_SM and ReportSMDeliveryStatus messages shall not be relayed. Instead the CdPA GTA in the message shall be modified in CC+RN+DN format (or RN+IDN format if CC match is not found in leading digits). The NAI of CdPA GTA shall be set to International and the message shall Fall-Through to GTT.

- If the number is found and a SP entity is present for this entry, then G-Port shall use the SP translation data for the number to route the message to the destination.  This is true whether or not VstpMnpOptions:MNPCRP  option is on.  However, if GRN is also associated (along with SP entity) with the DN and the GRN is not present in the HomeRN table and VstpMnpOptions:SRISMGTTRTG  is ON, then the SRI_SM and ReportSMDeliveryStatus messages shall not be relayed. Instead the CdPA GTA in the message shall be modified in CC+GRN+DN format (or GRN+IDN format if CC match is not found in leading digits). The NAI of CdPA GTA shall be set to International and the message shall Fall-Through to GTT.

- If the number is found and neither SP nor RN data is associated with it (this is a direct routing case with number not known to be ported or not identified to be ported), then

- If the message is is SRI, and VstpMnpOptions:MNPCRP  is off, or if VstpMnpOptions:MNPCRP  is on and no HomeRN was present in the incoming DN (i.e. a HomeRN was not deleted from the SCCP CdPA/MAP MSISDN), and if the portability type associated with the DN entry is other than 3-35 , then G-Port shall generate a SRI Ack message with the MSISDN in the Routing Number parameter. If the message is SRI, and VstpMnpOptions:MNPCRP  is off, or if VstpMnpOptions:MNPCRP  is on and no HomeRN was present in the incoming DN (i.e. a HomeRN was not deleted from the SCCP CdPA/MAP MSISDN), and the portability type associated with the DN entry is 3-35, then the SRI shall fall through to GTT (i.e. no SRI Ack message is generated).

- If NPS parameter will be encoded in the SRI Ack message if :

VstpMnpOptions : ENCODENPS=ON and DN is associated with PT =0, 1, 2 (foreign number)

VstpMnpOptions : ENCDNPSPTNONE=ON and DN is associated with PT = null (deemed foreign number)

- If the message is non-SRI, and VstpMnpOptions:MNPCRP  is off, or if VstpMnpOptions:MNPCRP  is on and no HomeRN was present in the incoming DN (i.e. a HomeRN was not deleted from the SCCP CdPA), then the message shall fall through to GTT.

- If the message is SRI or non-SRI, and VstpMnpOptions:MNPCRP  is on, and a HomeRN was present in the incoming DN (i.e. a HomeRN was deleted from the SCCP CdPA/MAP MSISDN), then G-Port shall generate Event #70304, and the message shall fall through to GTT.  In most network implementations, since the message contains RN+DN, this should cause a GTT failure, which will result in the vSTP sending a UDTS to the originator if the Return Message on Error flag was set in the incoming UDT.

- If the number is not found in the UDR NPDB then the VstpMnpOptions:SRIDNNOTFOUND option is consulted if query is not SRI prepaid. If the query is identified to be an SRI prepaid then SRI Ack message shall be sent back. NPS will be encoded in the SRI Ack message if VstpMnpOptions:ENCDNPSDNNOTFOUND is ON.

- If the VstpMnpOptions:SRIDNNOTFOUND option is set to SRINACK then a negative acknowledgement is generated in response to the given message.

- If the VstpMnpOptions:SRIDNNOTFOUND option is set to GTT then GTT is performed on the message.

- Message Verification/Decode:

  - MTP/SCCP Verification: vSTP shall not perform any additional MTP/SCCP verification for G-Port. G-Port shall use the information decoded by SCRC.

  - General TCAP/MAP verification: TCAP/MAP verification is performed on all messages.

Any error found in the message verification process will not generate any error responses. G-Port will abort verification and perform message relay on the message using the decoded SCCP information. The Event information shall be printed to report the error.

  - MAP Verification: G-Port performs no MAP verification like validation of ACN or decoding of User Information. G-Port shall look at the operation code of the message to distinguish SRI messages with all other messages. After determining the operation code to be SRI, G-Port shall look for the presence of OR Interrogation Parameter to further distinguish SRI from SRI for Optimal routing (SRI-SOR) message. If the OR Interrogation is present or if operation code is not SRI then G-Port message relay is performed. Otherwise, SRI Specific verification is performed.

  - SRI specific Verification: This verification shall be performed only for SRI messages. G-Port shall only look for MSISDN parameter only. It shall not look for the existence of any other parameter even if they are mandatory.

Any error found in this part of the verification process would cause the following to occur:

    - SRI message will be discarded, appropriate SRI negative response message is sent back.

- Message Handling:

  - RN Prefix deletion:

    - SRIDN = ‘SCCP’

The decoded SCCP CdPA digits may have a RN concatenated with the MSISDN number in two forms 1). RN+DN 2). CC+RN+DN.. So when the SNAI is either RNIDN or RNNDN or RNLDN, G-Port compares the decoded MSISDN number with the list of provisioned home RN prefixes defined in the NPDB. If a match is found, then G-Port shall strip off the RN digits from the number. Number conditioning (if required) is performed after deleting the RN. When SNAI is CCRNDN, G-Port shall first compare the CC to DEFCC/MultCC list.  If CC DEFCC/MultCC then no prefix deletion is performed and G-Port processing continues.  If CC=DEFCC/MultCC then, G-Port shall compare the digits after CC with the list of provisioned home RN prefixes defined in the NPDB. If a match is found, then G-Port shall strip off the RN digits from the number. If no match then the no prefix deletion is performed and G-Port processing continues.

    - SRIDN = ‘TCAP’

The decoded MAP MSISDN digits may have a RN concatenated with the MSISDN number in two forms. 1) RN + DN 2) CC+RN+DN.  The MAP NAI will be used to determine the type: International, National or Subscriber.  If VstpMnpOptions:MNPCRP  is off, RN prefix deletion will not be attempted.  If VstpMnpOptions:MNPCRP  is on, then RN prefix deletion will be attempted on all MSISDNs.  If the MAP NAI indicates International, then a check shall be performed for DEFCC/MultCC prefix on the MSISDN.  If DEFCC/MultCC is detected, then HomeRN deletion shall be attempted using the CC+RN+DN format.  All other MSISDNs will use the RN+DN format.  G-Port shall compare the decoded MSISDN number with the list of provisioned home RN prefixes defined in the NPDB.  If a match is found, the G-Port shall strip off the RN digits from the number.  Number conditioning (if required) is performed after deleting the RN.  If CC+RN+DN search is performed, G-Port shall compare the digits after CC with the list of provisioned home RN prefixes defined in the NPDB.  If a match is found the G-Port shall strip off the RN digits from the number.  If no match is found, then no prefix deletion is performed and G-Port processing continues.

The RN Prefix deletion for SRI_SM, when SRISMDN= SCCP or TCAP, will work in the same manner as it works for SRI message when SRIDN=SCCP or TCAP respectively.

  - Number Conditioning:

UDR NPDB shall store international MSISDNs only. The received MSISDN number or SCCP CdPA digits may need to be converted to an international number in order to do a database lookup. When G-Port is required to be performed on a message and the number is not international (i.e. NAI of MSISDN number is “National (Significant) Number” or “Subscriber Number” or SNAI is NATL or SUB or RNNDN or RNLDN), then the National/Local to International number is triggered. For a National (Significant) Number, the received CdPA/MAP MSISDN digits will be prepended with the default country code and for a Subscriber number the CdPA/MAP MSISDN digits will be prepended with the default country code and the default network code.

  - Database Lookup:

G-Port performs the UDR NPDB database lookup using the international MSISDN. The individual number database is searched first and if the number is not found, then the number range database is searched. If a match is not found in individual and range based database then GTT is performed on the message. In case of MSISDN numbers in the UDR NPDB database being odd and CdPA GTI of the incoming being 2 and the last digit of the number is ‘zero’, G-Port shall first perform database lookup once using the even number. If no match is found then G-Port shall again perform the database lookup now using the odd number (without last digit).

- Mobile terminated GSM SMS NP:

MT-SMS messaging involves the SMSC or MMSC querying the HLR for destination subscriber for SMS delivery. For GSM network, these query messages are called SRI_SM. The HLR response to these messages includes routing information that can be used by the query generator (SMSC) to deliver the SMS message. The G-Port service intercepts these MT-SMS messages destined to the HLR and replies with routing information for out of network destination subscribers.

The MT-SMS NP feature will:

  - Intercept SMS routing information request from SMSC/MMSC before it reaches the HLR.

  - Extract message destination address (MAP MSISDN or SCCP Called Party GTA based on SRISMDN parameter value in VstpMnpOptions table), condition the digits and perform lookup in NPDB.

  - For destination address/subscribers belonging to foreign networks, sends reply message to the SMSC/MMSC with routing information. This information can be used by the SMSC to route the message to their recipient networks using protocols like SMPP.

  - For in-network destination addresses, the SMS routing information request is relayed to the HLR.

- MT-SMS NP Processing :

The SMSC (or MMSC) will send the SRI_SM message to the vSTP (with a destination of the HLR) with SCCP CdPA GTA (or MAP MSISDN based on SRISMDN parameter value in VstpMnpOptions table) as the DN of the destination subscriber to be GT routed to the HLR.

The service selector configured to channel MSUs to the G-Port service has an “SNAI” or service NAI parameter as described earlier.

  - Processing Step:

  - Existing handling of SRI_SM for GT-routed, ANSI/ITU MTP/SCCP, ITU TCAP/MAP, encapsulated in either non-segmented XUDT or UDT SCCP message type, matching G-Port service Selector involves the following steps (detailed MSU decode/encode information provided later).

- Limitations

  - MNP G-Port feature is designed with a basic concept: Own-network subscribers are provisioned with an SP Entity type, and other network subscribers are provisioned with RN Entity type or No Entity type

MOs and Operation Supported:

- Following is the list of MO’s and operation supported for vSTP MNP G-Port features

Table 52 – MO details 3

| MO Name | Operations Supported |
| --- | --- |
| VstpSccpMnpOptions | Update |
| VstpSccpSrvcSel | Insert, Update, Delete |
| VstpSccpHomeEntity | Insert, Delete |

Table 3

- Refer MMI API Guide on Active NOAM/SOAM: “Main Menu ->MMI API Guide” on any DSR 8.4 GA release setup for details about the URI, example and parameters about each MO.

#### MEALS

      - Measurements:

Following measurements are added by G-Port feature and all are B scoped

Table 53 – Measurements details1

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| vstpMnpCrd | Single | Number of times circuler route detected by MNP CRP | 5 | Performance | Simple |
| vstpGportSriRecv | Single | Number of call related SRI message received. | 5 | Performance | Simple |
| vstpGportSriReply | Single | Number of call related SRI messages that fell through to GPORT service. | 5 | Performance | Simple |
| vstpGportSriGtt | Single | Number of call related SRI message that fell through to GTT due to no match. | 5 | Performance | Simple |
| vstpGportSriErr | Single | Number of call related messages that cause an error response message. | 5 | Performance | Simple |
| vstpGportSriSmRcv | Single | Number of SRI_SM message received. | 5 | Performance | Simple |
| vstpGportSriSmRep | Single | Number of SRI_SM messages resulting in SRI_SM_ACK or SRI_SM_NACK. | 5 | Performance | Simple |
| vstpGportSriSmErr | Single | Number of SRI_SM messages resulting in error. | 5 | Performance | Simple |
| vstpGportNonCallRelay | Single | Number of non-call related messages relayed by G-Port. | 5 | Exception | Simple |
| vstpGportNonCallGtt | Single | Number of on-call related messages that fell through to GTT. | 5 | Performance | Simple |
| VstpUdrDbDiscCATxFail | Single | Number of messages discarded by Lss because of send fail to CA layer. | 5 | Exception | Simple |
| VstpMnpCAQueryProcessMax | Single | Peak time by CA to send query and receive the response from UDR | 5 | Performance | Peak |
| VstpMnpCAQueryProcessAvg | Single | Average time by CA to send query and receive the response from UDR | 5 | Performance | Average |
| VstpMnpCAQueryProcesTime | Array | Time required by CA to send query and receive the response from UDR | 5 | Performance | Simple |
| VstpMnpCATimeOut | Single | Number of messages for which CA query to UDR timed out | 5 | Exception | Simple |

Table 54 – Measurement details2

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| VstpUdrDbDiscCADcdFail | Single | Number of messages discarded by LSS due to decode failed of CA response message | 5 | Exception | Simple |
| VstpUdrDbDiscPduFul | Single | Number of messages discarded when PDU pool is exhausted | 5 | Exception | Simple |
| VstpUdrDbDiscIntErr | Single | Number of messages discarded due to internal processing error | 5 | Exception | Simple |
| VstpMnpRxRatePeak | Single | The peak Rx messages by MNP Application | 5 | Performance | Peak |
| VstpMnpRxRateAvg | Single | The average Rx messages by MNP Application | 5 | Performance | Average |
| VstpMnpMsgRecv | Single | Number of messages received MNP Application | 5 | Performance | Simple |
| VstpUdrDbSubsNotFound | Single | Number of subscriber record not in UDR DB | 5 | Exception | Simple |
| VstpUdrDbQueryFailUDRConnDown | Single | Number of UDR DB Queries not initiated due to UDR connectivity down | 5 | Exception | Simple |

      - Events & Alarms

Following new events & alarms are added by GPORT feature:

Table 55 – Events & Alarms 1

| Alarm/Event Name | Type | Description | Raise condition | Clear condition | Throttle sec | Instance | Additional Information | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Invalid length of conditioned digits (70301) | Event | MNP length of the conditioned digit is invalid | 1. If the length of the international MSISDN is less than 5 or greater than 15 digits. | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| Conv to intl num - Dflt CC not found (70302) | Event | Dflt CC not defined | 1. If the default CC is not found | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| Conv to intl num - Dflt NC not found (70303) | Event | Default NC not defined | 1. If the NDC is not found during the conversion of the MSISDN to an international MSISDN | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| MNP Circular Route detected (70304) | Event | NP Circular Route detected | 1. If a RN is found, or if neither a RN nor a SP are found, in the database for a DN when a HomeRN was present in the incoming DN of the message | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| Translation PC type is ANSI (70305) | Event | MNP translated PC type is ANSI |  | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| Invalid digits in MAP MSISDN parameter (70306) | Event | GPORT inv map msisdn for sri/srism | 1. If number of digit (MSISDN address digits) is ‘0’ or greater than ‘21’ | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| Invalid prefix/suffix digit length (70307) | Event | Too many digits | 1. If SRI_SM or ReportSMDeliveryStatus Digit modification fails for modified DN > 21 digits | NA | 1 | None | SIO, OPC, DPC, SCCP MSG Type | NA |

## VSTP MNP ATI NP

#### Purpose and Solution

Purpose

- Support for ATI Number Portability (a.k.a. ATI NP) feature in vSTP.

- Number Portability (MNP) allows mobile subscribers to retaining their original MSISDN when changing the network to which they subscribe.

- This feature is an optional feature and can be turned on/off via configurable option in the VstpMnpOptions.

Solution

- ATI NP shall perform the following actions based on the message received  :

- If the incoming ATI query message requested MNP information, ATI NP shall send the ATI Ack message to the MSC with the Routing Number and number portability status information in the MAP portion of the message.

- If the incoming ATI query message requested Location information, ATI NP shall send the ATI Ack message to the MSC with the Location information of the VLR.

Message flow for ATINP Solution on vSTP

  - MSC will send ATI  request to vSTP-MP over SS7 links.

- vSTP-MP will decode and verify the ATI Message

- Check whether ATI message has valid request (the requestedInfo parameter must be MNPRequestedInfo and/or Location Information).

- Decode the MSISDN parameter from the Subscriber Identity parameter.

- Condition the MSISDN to the international format

  - vSTP-MP will query the UDR NOAM for conditional MSISDN DB.

  - UDR NOAM will  look up MSISDN DB and will send response to the vSTP-MP.

  - Determine whether the lookup is considered to be successful based on provisioned options. If yes, use entity information to encode ATI ACK response and route the response to the originator. If no, send ATI NACK response with appropriate error code.

- Message flow for ATINP Solution on vSTP

Figure 28 – ATINP message flow

- ATI NP Message Handling:

- The ATI query message arrives at vSTP   with routing determined by either route-on-gt or route-on-SSN.

- If the message is route-on-SSN and the SSN number in Called Party Subsystem field in the matches to the ATINP SSN provisioned on vSTP, the message is sent to ATINP subsystem for further processing.

- If the message is route-on-GT, vSTP decodes the SCCP portion and uses the data to perform ATINP Selection based on the CdPA GT fields.  The result of this selection provides service indicator. If the service selector is ATINP, the message is sent to ATINP subsystem for further processing.

- The message is decoded to verify the opcode and mandatory parameters.  fF the MAP message opcode is ATI, the MSISDN and requested info parameters are decoded. If MSISDN is valid and either MNPinfo or LocationInfo or both parameters are present, further number conditioning is performed. Otherwise, error message is sent back to originator.

- The decoded DN is conditioned to international number before performing UDR NPDB lookup.

The DN is considered to be in international format if either of the following conditions are true.

- VstpMnpOptions:ATISNAI = INTL OR

- VstpMnpOptions:ATISNAI = NAI   and NAI field from MSISDN is INTL (0x1) or Network Specific Number (0x3).

If the incoming DN is in national format, VstpMnpOptions:DefCC will be prepended to the DN to condition it in international format.

- The UDR NPDB database lookup involves 2 steps:

- First the exception or individual number database is searched for a match. If the match is found then the data associated with this entry is considered.

- If the conditioned number is absent in the exception (individual) database, then the number range database is searched. If the match is found then the data associated with this range entry is considered. If the search is unsuccessful then the result is no match.

- UDR NPDB lookup is considered successful if

- VstpMnpOptions:ATINPTYPE = ANY and MSISDN is found in the NPDB with entity type = RN, SP or GRN. or

- VstpMnpOptions:ATINPTYPE = ALWAYS and MSISDN is not found in individual or range entries.

- If UDR NPDB lookup is successful and if the ATI query contains MNP request info parameter ATINP will send ATI ACK response message with the following fields in the message

- Routing Number : Formatting will be determined by VstpMnpOptions:ATIACKRN option

- IMSI : Formatting will be determined by VstpMnpOptions:ATIACKIMSI option

- MSISDN : Formatting will be determined by VstpMnpOptions:ATIACKMSISDN option

- Number portability Status

Not Known To Be Ported (0)

Own Number Ported Out (1)

Foreign Number Ported To Foreign Network (2)

Own Number Not Ported Out (4)

Foreign Number Ported In (5)

- If UDR NPDB lookup is successful and if the ATI query contains Location Information request info parameter ATINP will send ATI ACK response message with the following fields in the message

- VLR Number: Formatting will be determined by VstpMnpOptions:ATIACKVLRNUM option

- If UDR NPDB lookup is unsuccessful  ATINP will send ATI NACK response message with the error code = “UnknownSubscriber”

MOs and Operation Supported:

Table 56 – MO’s for ATI NP Feature

| MO Name | Operations Supported |
| --- | --- |
| VstpSccpMnpOptions | Update |
| VstpSccpSrvcSel | Insert, Update, Delete |

Refer MMI API Guide on Active NOAM/SOAM: “Main Menu ->MMI API Guide” on any DSR 8.4 GA release setup for details about the URI, example and parameters about each MO.

#### MEALS

      - Measurements

Table 57 – ATI NP Measurements details

| Meas Id # | Measurement Name | Description | Dimension | Type | Interval Mins | Measurement Group |
| --- | --- | --- | --- | --- | --- | --- |
| 21678 | vSTPAtiNpRcv | Total MSUs received by SCCP with opcode of ATI. | Single | Simple | 5 | vSTP MNP Performance |
| 21679 | vSTPAtiNpAck | Total ATI-ACK response messages sent by SCCP | Single | Simple | 5 | vSTP MNP Performance |
| 21680 | vSTPAtiNpErr | Total MSUs received by SCCP with opcode of ATI that did not result in either ATI-ACK or ATI-NACK response message. | Single | Simple | 5 | vSTP MNP Exception |
| 21689 | VstpAtiNpRxRatePeak | The peak Rx messages by ATINP Application | Single | Max | 5 | vSTP MNP Performance |
| 21690 | VstpAtiNpRxRateAvg | The average Rx messages by ATINP Application | Single | Average | 5 | vSTP MNP Performance |

      - Alarms & Events

Table 58 – Alarms & Events for ATI NP feature

| Event Id # | Event Text | Event Description | Throttle (Sec) | Additional Info |
| --- | --- | --- | --- | --- |
| 70091 | Missing Mandatory Parameter | Subscriber Identity parameter or Requested Info parameter is missing in the received message | 10 | SIO, OPC, DPC, CgPA, CdPA, SSN |
| 70092 | Malformed Subscriber Id | The subscriber Identity parameter in ATI NP query was found to be mistyped. | 10 | SIO, OPC, DPC, CgPA, CdPA, SSN |
| 70093 | Unexpected value for Subscriber Id | The Choice for Subscriber Identity in ATI NP query is not MSISDN. | 10 | SIO, OPC, DPC, CgPA, CdPA, SSN |
| 70094 | Invalid MSISDN length | The MSISDN length in Subscriber Information was 0, or the MSISDN length was 1 (byte) and the MSISDN had only one 0xF (filler) digit. | 10 | SIO, OPC, DPC, CgPA, CdPA, SSN |
| 70095 | ATINP Invalid Requested Info | The Requested Info parameter in incoming ATI NP query was invalid. Either, length of Requested Info parameter < 2, or the Requested Info parameter does not contain MNP Requested Info and/or Location Information , or the parameter is badly formatted | 10 | SIO, OPC, DPC, CgPA, CdPA, SSN |
| 70096 | Digits truncated in encoded parameter | One or more encoded digits parameters in ATI ACK response had to be truncated to fit maximum allowed encoded digits. | 10 | SIO, OPC, DPC, CgPA, CdPA, SSN |

## VSTP MNP INP

#### Purpose and Solution

Purpose

- Support for INPQ Number Portability feature in vSTP.

- Number Portability (MNP) allows mobile subscribers to retaining their original MSISDN when changing the network to which they subscribe.

- This feature is an optional feature and can be turned on/off via configurable option in the VstpMnpOptions.

Solution

- INPQ shall perform the following actions based on the message received  :

- If the incoming INPQ query message requested MNP information,  INPQ shall send the INPQ Ack message to the MSC with the Routing Number and number portability status information in the MAP portion of the message.

- If the incoming INPQ query message requested Location information,  INPQ shall send the INPQ Ack message to the MSC with the Location information of the VLR.

Message flow for INPQ Solution on vSTP

- MSC will send INPQ  request to vSTP-MP over SS7 links.

- vSTP-MP will decode and verify the INPQ Message

- Check whether INPQ message has valid request (the requestedInfo parameter must be MNPRequestedInfo and/or Location Information).

- Decode the MSISDN parameter from the Subscriber Identity parameter.

- Condition the MSISDN to the international format

- vSTP-MP will query the UDR NOAM for conditional MSISDN DB.

- UDR NOAM will look up MSISDN DB and will send response to the vSTP-MP.

- Determine whether the lookup is considered to be successful based on provisioned options. If yes, use entity information to encode INPQ ACK response and route the response to the originator. If no, send INPQ NACK response with appropriate error code.

Message flow for INQP Solution on vSTP

Figure 29 – INQP message flow

#### MEALS

      - Measurements

Table 59 – Measurements for INPQ feature

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| VstpInpCirrouteDetected | Single | No. of circular route detected by INPQS | 5 | Performance | Simple |
| VstpInpSuccessReply | Single | Number of INP successful replies | 5 | Performance | Simple |
| VstpInpErrReplies | Single | Number of INP error replies with TCAP error code | 5 | Performance | Simple |
| VstpInpDiscardedQuerieNoReply | Single | Number of INP discarded queries as no reply is generated | 5 | Performance | Simple |
| VstpInpQueryReceived | Single | Inp Query received |  |  |  |

      - Alarms & Events

Table 60 – Alarms & Events for INPQ feature

| Alarm/Event Name | Type | Description | Raise condition | Clear condition | Throttle sec | Instance | Additional Information | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Unsupported ACN object ID length.(70420) | Event | Unsupported ACN object ID length. | ACN object Identifier length > 32 | NA | 10 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| Failed to Decode TCAP parameters.(70421) | Event | Failed to Decode TCAP parameters. | Invalid INAP CalledPartyNumber len,No parameter sequence | NA | 10 | None | SIO, OPC, DPC, SCCP MSG Type | NA |
| INAP Called Party Number is missing.(70422) | Event | INAP Called Party Number is missing. | No INAP CalledPartyNumber parameter | NA | 10 | None | SIO, OPC, DPC, SCCP MSG Type | NA |

## VSTP MTP Screening support

#### Purpose and Solution

Purpose

At present VSTP allows all MSUs to enter for processing which will be received on a link set. This will be a security issue for VSTP because any message can enter into VSTP for processing and can be part of crashing the software or interrupt the processing of other messages.

Solution

MTP screening feature provides solution to screen the message based on MTP3 layer parameters of the messages. The feature achieves it by :

- Creating screening rule and keeping rules of same type in a group.

- Creating a screen set and referring rule group created above in that screen set.

- Attaching that screen set to incoming linkset.

Feature Overview

- MTP Screening feature provides a first level of security check for VSTP.

- The MTP Screening feature examines the contents of a Message Signaling Unit (MSU) attempting to enter the VSTP against predefined criteria in the VSTP database to determine if the MSU should be allowed to enter.

- When a message comes on a linkset, the associated screen set will be looked up. Based on the NSFI and rule group name associated with screen set, the rule in the corresponding rule group will be looked up.

- If rule lookup is successful, then message will go for further screening level based on NSFI and the next screen rule group name associated with it.

- If rule lookup does not find a match, then

- In case of BLKOPC/BLKDPC rule type, default rule for that rule group will be looked up and based on NSFI and next screen rule group name associated with default rule further screening performed.

- In case of OPC/SIO/DPC/AFTDSTN rule type, FAIL NSFI will be performed.

- The final result of MTP screening should always be either FAIL or STOP.

- In case of FAIL, the message will be discarded.

- In case of STOP, the message will go for further processing in VSTP.

MTP screening high level design diagram

Figure 30 – MTP screening design diagram

Types of MTP Screening “NSFI”:

  - OPC

  - BLKOPC

  - SIO

  - DPC

  - BLKDPC

  - AFTDSTN

Types of MTP Screening “NSFI” - OPC:

  - OPC NSFI and next OPC rule group name defines rules that have all the Originating Point Codes that are allowed to send any message to the recipient network.

  - The possible values for rule type OPC are BLKOPC, SIO, DPC, BLKDPC, STOP, FAIL.

  - When a message comes on a linkset, the corresponding screen set will be looked up. Based on the NSFI and next screening rule group name associated with screen set (i.e. OPC as first NSFI and next OPC rule group name in that case), the rule lookup will be performed.

  - If OPC screening rule lookup is successful, then message should go for further screening level.

  - If OPC screening rule lookup does not find a match, then message should be discarded if mtpScrTestMode is OFF in VstpLinkset table.

Types of MTP Screening “NSFI” - BLKOPC:

  - BLKOPC NSFI and next BLKOPC rule group name defines rules that have all the Originating Point Codes that are prohibited to send any message to the recipient network.

  - The possible values for rule type BLKOPC are SIO, DPC, BLKDPC, STOP, FAIL.

  - When a message comes on a linkset, the screen set associated with that linkset will be looked up and based on the NSFI and next screening rule group name associated with screen set (i.e. BLKOPC as first NSFI and next BLKOPC rule group name in that case), the rule lookup will be performed.

  - If BLKOPC screening rule lookup is successful, then message should be discarded if mtpScrTestMode is OFF in VstpLinkset table.

  - If BLKOPC rule lookup does not find a match in BLKOPC rule group, then message should go for further screening level based on default rule in BLKOPC rule group.

Types of MTP Screening “NSFI” - SIO:

  - SIO NSFI and next SIO rule group name defines rules that have all the SIO values that are allowed/prohibited to send any message to the recipient network.

  - The possible values for rule type SIO are DPC, BLKDPC, AFTDSTN, STOP, FAIL.

  - When a message comes on a linkset, the corresponding screen set will be looked up. Based on the NSFI and next screening rule gr screening rule group name associated with screen set (i.e. SIO as first NSFI and next SIO rule group name in that case), the rule lookup will be performed.

  - If SIO screening rule lookup is successful, then message should go for further screening level.

  - If SIO rule lookup does not find a match, then message should be discarded if mtpScrTestMode is OFF in VstpLinkset table.

  - The NSFI “AFTDSTN” only applicable for messages which has service indicator value < 3.

Types of MTP Screening “NSFI” - DPC:

  - DPC NSFI and next DPC screening rule group name defines rules that have all the Destination Point Codes that are allowed to send any message to the recipient network.

  - The possible values for rule type DPC are BLKDPC, AFTDSTN, STOP, FAIL.

  - When a message comes on a linkset, the corresponding screen set will be looked up. Based on the NSFI and next screening rule gr screening rule group name associated with screen set (i.e. DPC as first NSFI and next DPC rule group name in that case), the rule lookup will be performed.

  - If DPC screening rule lookup is successful, then message should go for further screening level.

  - If DPC rule lookup does not find a match, then message should be discarded if mtpScrTestMode is OFF in VstpLinkset table.

Types of MTP Screening “NSFI” - BLKDPC:

  - BLKDPC NSFI and next BLKDPC rule group name defines rules that have all the Destination Point Codes that are prohibited to send any message to the recipient network.

  - The possible values for rule type BLKDPC are AFTDSTN, STOP, FAIL.

  - When a message comes on a linkset, the corresponding screen set will be looked up. Based on the NSFI and next screening rule gr screening rule group name associated with screen set (i.e. BLKDPC as first NSFI and next BLKDPC rule group name in that case), the rule lookup will be performed.

  - If BLKDPC screening rule lookup is successful, then message should be discarded if mtpScrTestMode is OFF in VstpLinkset table.

  - If BLKDPC rule lookup does not find a match in BLKDPC rule group, then message should go for further screening level based on default rule in BLKDPC rule group.

Types of MTP Screening “NSFI” - AFTDSTN:

  - AFTDSTN NSFI and next AFTDSTN rule group name defines rules that have all the Affected Destination values for network management messages that are allowed to send any message to the recipient network.

  - The possible values for rule type AFTDSTN are STOP and FAIL.

  - During screening if NSFI is AFTDSTN then either STOP or FAIL action will be performed based of AFTDSTN screening success or failure.

  - The AFTDSTN NSFI can’t be configured as first NSFI in Screen Set.

  - Only messages which have Affected Destination (i.e. SNM message) will be screened.

  - The message which doesn’t have Affected Destination  will be continue processed and not discarded. Event 70393 (Invalid MSU received for AFTDSTN NSFI) will be generated if mtpScrEventLog flag is ON for that incoming linkset.

Table 61 - Message processing with mtpScrEventLog and mtpScrTestMode option when NSFI is FAIL

| mtpScrEventLog (ON/OFF) | mtpScrTestMode (ON/OFF) | Message discarded/Message continue processed for further routing | Event 70392 (MSU discarded due to MTP Screening) Generated (Yes/No) |
| --- | --- | --- | --- |
| OFF | OFF | Message will be discarded | No |
| ON | OFF | Message will be discarded | Yes |
| OFF | ON | Message will be continue processed for further routing | No |
| ON | ON | Message will be continue processed for further routing | Yes |

NOTE:

When mtpScrEventLog is OFF and mtpScrTestMode is ON then screening rule processing will not take place.

NSFI STOP doesn’t have any effect with these options.

Table 62 – MO details for MTP screening feature

| MO Name | Operations supported | URI |
| --- | --- | --- |
| MTP Screening Rule | POST/PUT/DELETE/GET | /vstp/mtpscreeningrules |
| MTP Screen Set | POST/PUT/DELETE/GET | /vstp/mtpscreensets |
| Linkset | POST/PUT/DELETE/GET | /vstp/linksets |

- Refer MMI API Guide on Active NOAM/SOAM: “Main Menu ->MMI API Guide” on any DSR 8.4 GA release setup for details about the URI, example and parameters about each MO.

Screening Examples:

- If a message is coming with the values:

  - Service Indicator = 5

  - Network Indicator Code = 3

  - Priority = 0

- In order to discard this message, please configure the SIO screening rule through MMI as shown in Slide #29 with NSFI = FAIL.

- If a message is coming with the values:

  - Service Indicator = 0

  - Priority = Any Value (0-3)

  - Network Indicator Code = Any Value (0-3)

  - H0 code = 4

  - H1 Code = 1

- In order for the message to be continued to process, please configure the SIO screening rule through MMI as shown in Slide #29 with NSFI = STOP.

#### MEALS

      - Measurements

Following is the List of measurements supported by MTP Screening Feature:

  - VstpRxScrPerformed: The total number of MSUs on which MTP Screening is performed.

  - VstpRxLnksetScrPerformed: The total number of MSUs on which MTP screening performed on Linkset.

  - VstpRxMSUScrDiscard: The total number of MSUs discarded due to MTP Screening.

  - VstpRxLnksetMSUScrDiscard: The total number of received MSUs discarded due to MTP screening on Linkset.

      - Alarms & Events

Following is the list of events supported by MTP Screening Feature:

Table 63 – Alarms and Events for MTP screening feature

| Alarm/Event Name | Type | Description | Raise Condition |
| --- | --- | --- | --- |
| MSU Failed MTP Screening (70392) | Event | MSU Failed MTP Screening | When incoming MSU is discarded on a linkset due to MTP screening and mtpEventLogging flag is turned on for that linkset. |
| Invalid MSU received for AFTDSTN NSFI (70393) | Event | Invalid MSU received for AFTDSTN NSFI | When NSFI is AFTDSTN and a non-network management message received which does not have affected destination and mtpEventLogging flag is turned on for that linkset. |

Limitations:

- This feature does not implement screening based on SCCP layer parameters. The feature scope is limited to screening based on MTP3 layer parameters.

- Point code support for the following is not tested in this release and advised not to configure the same:

  - ITUN16

  - ITUN24

  - ITUN_S

  - ITUI_S

Troubleshooting Steps

- If MTP Screening executes successfully on an incoming MSU, then VstpRxScrPerformed measurement will be pegged on a system basis.

- If MTP Screening executes successfully on an incoming MSU, then VstpRxLnksetScrPerformed measurement will be pegged on a per Linkset basis.

- If incoming MSU is discarded due to MTP Screening, then VstpRxMSUScrDiscard measurement will be pegged on a system basis.

- If incoming MSU is discarded due to MTP Screening, then VstpRxLnksetMSUScrDiscard measurement will be pegged on a per Linkset basis.

- When incoming MSU is discarded on a linkset due to MTP screening and mtpEventLogging flag is turned on for that linkset, then event 70392 MSU Failed MTP Screening will be generated and contains reason with MTP3 layer parameters (i.e. OPC, DPC, SIO) information and incoming Linkset Name.

- When NSFI is AFTDSTN and a non-network management message received which does not have affected destination and mtpEventLogging flag is turned on for that linkset, then event 70393 Invalid MSU received for AFTDSTN NSFI will be generated and contains reason with MTP3 layer parameters (i.e. OPC, DPC, SIO) information and incoming Linkset Name.

If any of the above statement fails as per given scenarios, then verify configuration.

If issue still exists, then contact Oracle for support.

## VSTP MULTIPLE POINT CODE SUPPORT

#### Purpose and Solution

Purpose:

- At present vSTP supports only 4 True Point Codes(TPCs) of ANSI/ITU-I/ITU-N/ITU-N24 domain and 1 Capability Point Code(CPC).

  - This limits operators from hosting more no. of point codes which triggers the need for more no of nodes in the network and associated provisioning & maintenance.

  - Since only for each domain only one TPC can be configured, it limits operators from using same Signaling Transfer Point(STP) to handle multiple national networks.

  - It also limits provisioning of additional links between two nodes beyond the number of links permitted by the protocol.

  - Moreover, vSTP does not support associating CPC with a hosted application/service.

- MPC solution resolves the above limitations by enabling vSTP to host Secondary Point Codes (SPCs) in addition to the true point codes used by the vSTP in any of the three domains ANSI, ITU-N (14-bit or 24-bit) and ITU-I.

- It also optionally permits attaching service/application with hosted CPC.

- This in turn enables collapsing/merging of multiple STPs into one vSTP.

Solution:

- VstpLocalSP table stores all the self point codes hosted  by vSTP.  This table schema shall be changed to allow provisioning of SPC and CPC.

- Support shall be provided to associate CPC with a service/application hosted  by vSTP.

- Adjacent Remote Signaling Point(RSP) can be associated with LocalSP (configured as TPC or SPC) via Linkset configuration (in VstpLinkset table).

- SRM message encoding shall be modified to set TPC/SPC as the Origin Point Code(OPC) as configured for the recipient RSP.

- Upgrade changes required to ensure old entries of VstpLocalSP table is copied properly into the new VstpLocalSP table.

Feature Overview:

- This feature proposes to start support for Secondary Point Codes and increase the number of supported Capability point codes.

- Multiple Point Code (MPC) feature shall allow the vSTP to add the support of Secondary Point Codes (SPCs) in addition to the true point codes used by the vSTP in any of the three domains ANSI, ITUN/ITUN-Spare (14-bit or 24-bit) and ITUI/ITUI-Spare.

- This is a different concept from capability point codes. The provisioning and routing will use secondary point codes as if they were the actual point code of the vSTP. SPCs are supported for any type of link (A, B, C, D, etc.).

- In addition to the one True Point Code (TPC) already supported for each of the ANSI, ITU-N (14-bit or 24-bit) and ITU-I domains, the vSTP support a pool of Secondary Point Codes (SPC), each of which may be assigned as either ANSI, ITUI/ITUI-Spare, 14-bit ITUN/ITUN-Spare, or 24-bit ITUN.

- In addition to the SPCs this feature also recommends increasing the number of CPCs supported by the vSTP.

- Currently only 1 CPC is supported. This number shall be increased to 100. The increase CPC would allow the vSTP to support more applications on the same node and allow route-on-gt functionality for routing to various applications.

- MPC feature creates the framework to allow provisioning and usage of SPCs in vSTP. This change shall primary affect the provisioning and routing algorithms. In addition to the SPC’s the CPC shall also be increased.

Sccp Message Routing:

Message destined for TPC

- Existing vSTP SCCP layer signaling data message processing order can route the message to either of the following components:

- SCMG

- LSS

- GTT

- SCMG

- Message with CDPA SSN as SCMG and CDPA RI as "Route On SSN“ is sent to SCMG for processing.

- LSS

- CDPA SSN matches with SSN configured in VstpSccpApplications table.

- Application’s Admin state is Enabled.

- GTT

- When above 2 criteria fails.

- With MPC support

    - Handling of message destined for TPC remains unchanged.

    - Message destined for SPC is treated the same way as it is done for TPC.

    - Message destined for CPC handled in the following way :

      - VstpLocalSP table must have this CPC for the message to reach SCCP.

      - Check for routing Indicator whether it is GT or SSN.

      - If RI is GT, message goes for GTT. Otherwise message goes for application.

      - Check VstpSccpApplications table and verify whether the CDPA SSN value is provisioned & admin state is enabled.

      - If above condition satisfies message goes to LSS for further processing. Otherwise message is discarded and an event is raised.

Table 64 – SCCP message routing behavior

| Routing Change |  | Rt-On-Gt | Rt-On-Ssn |
| --- | --- | --- | --- |
| DPC=TPC |  | No Change | No Change |
| DPC=SPC |  | Same handling as TPC |  |
| DPC=CPC | cpcType = STP | Handling takes usual path of DPC being set to TPC/SPC |  |
|  | cpcType != STP | Application specific handling triggers |  |

| Application Handling : (DPC = CPC) | cpcType = EIR | cpcType = Others (Future) |
| --- | --- | --- |
|  | • Check whether application is provisioned in VstpSccpApplication • If yes, check whether admin state is set to Enabled • Send to LSS EIR | • Implementation specific • As new applications gets added message routing path shall be decided accordingly to send the message respective application |

- OPC of the message generated by vSTP

  - Any message generated by vSTP shall set the OPC to the TPC/SPC used by the message’s destination. Following is a list of messages generated by vSTP

    - Signaling Link Test Messages

    - MTP Network Management Messages

    - SCCP Messages post GTT

    - Message generated by local subsystem

- DPC of the message generated by vSTP

  - There is no change.

- Affected Point Codes

  - TFx and TCx messages may include vSTP’s secondary point code depending on scenario(s)

- Message Specific Handling

  - Signaling Link Test Messages (SLTM/SLTA)

    - vSTP shall validate that the DPC of the message matches with the TPC/SPC provisioned against the OPC of the message.

    - Even if DPC matches with any other provisioned TPC/SPC, the message shall be rejected.

    - This check is enforced to detect provisioning errors which may interfere with network management.

  - Transfer Prohibited/Restricted/Allowed Messages

    - vSTP shall never generate a TFP, TFR with a Affected Point Code set to vSTP’s True Point Code or one of vSTP’s Secondary Point Codes.

    - If vSTP receives a Route Set Test message (RST or RSR) concerning a Secondary Point Code, vSTP shall respond with a TFA concerning the Secondary Point Code.

  - Route Set Test Messages

    - Affected Point Code in vSTP generated routeset test messages shall always be a Remote Point Code

  - Transfer Control Messages

    - For TFC messages, the Affected Point Code is always the DPC of the message that encountered congestion.  For vSTP, the Affected Point Code of the TFC is always a remote point code.

  - Subsystem Test (SST)

    - When vSTP receives a Subsystem Test (SST) message for a local subsystem that is online, vSTP sends an SSA back.

    - If the SST’s Affected Point Code is either the True Point Code or a Secondary Point Code, vSTP shall respond with an SSA with the Affected Point Code set to the SST’s Affected Point Code.

  - GTT Messages

    - In present implementation, when vSTP performs GTT, and the result of translation is a remote point code, vSTP replaces the OPC of the message with it’s True Point Code.

    - With this feature, vSTP shall replace the OPC of the message with the TPC/SPC used by the translated point code.

  - Local Subsystem Messages

    - When vSTP receives a Subsystem Test (SST) message on TPC/SPC for a local subsystem that is online, vSTP sends an SSA back.

    - In case the subsystem is not online, no response is sent back.

Limitations:

- This feature does not implement Multiple Linkset towards same RSP.

- This feature does not implement Local subsystem routing i.e. “messages coming with routing indicator as route on GT to be routed locally to application hosted on the vSTP” , wont be supported.

MO’s and operations supported

Point Code Provisioning

- VstpLocalSP Table schema is changed to accommodate provisioning of SPC(s) and CPC(s).

  - Upto 40 SPCs is supported.

  - Upto 100 CPCs is supported.

  - Capability Point Code can be associated with Service/Application.

Table 65 – Point Code provisioning

| Field Name | Description | Mandatory (M)/Optional(O) | Default Value | Value Range | Modification Supported |
| --- | --- | --- | --- | --- | --- |
| lspName | LSP Name | M | NA | U N C H A N G E D | No |
| lspId | LSP ID |  |  |  | No |
| Domain | PC Domain |  |  |  | No |
| mtpPc | Point Code (Integer) |  |  |  | Yes only if point code type is "CPC" |
| mtpPcStr | Point Code String |  |  |  |  |
| pcType | Defines Point Code Type | M | NA | TPC/ SPC/ CPC | No |
| cpcType | Defines Service/Application | O | STP | STP/ EIR | No |

- “pcType” field is used to identify the type of point code whether it is TPC or SPC or CPC.

- “cpcType” field is used to identify the type of service/application hosted by that point code.

- If “pcType” field is set to "CPC", user has to supply "cpcType" value, in other cases default value "STP" shall be applied.

- Below is a sample entry with the new table structure.

Table 66 – Isp Name with reference

| lspName | LspID | Domain | mtpPC | mtpPcStr | pcType | cpcType |
| --- | --- | --- | --- | --- | --- | --- |
| LSP1 | 1 | ITUI | 14503 | 7-20-7 | TPC | STP |
| LSP2 | 2 | ITUI | 4114 | 2-2-2 | CPC | EIR |

- VstpSccpApplications Table schema is changed to include “appType” field which indicates application/service associated with the SSN.

- The only value supported for now would be EIR, INPQ, ATINP.

- When new application / service is added , this type  shall include the same.

Table 67 – Field name with Description

| Field Name | Description | Default Value | Value Range | Modification Supported |
| --- | --- | --- | --- | --- |
| appId | Application ID No | NA | 0-65535 | No |
| appType | Application or Service Type | NA | EIR (For Now) | No |
| Ssn | Subsystem Number | NA | 0-255 | No |
| appAdminState | Application admin state | Disabled | Enabled/Disabled | Yes |

- ed in place of “appName” to indicate a service / application  provided by vSTP

Table 68 – AppType with Ssn id

| appId | appType | Ssn | appAdminState |
| --- | --- | --- | --- |
| 1 | EIR | 9 | Disabled |

Provisioning Dependencies/Restrictions

  - VstpLocalSP

    - While provisioning CPC, if service provided in "cpcType" field is other than "STP", ensure it is provisioned in VstpSccpApplications table.

    - If same service type is not present in VstpSccpApplications table, first an entry has to be made there.

    - Allow deletion of SPC only if it is not referenced in any other tables like VstpLinkset.

  - VstpLinkset

    - Allow SPC to be selected as LSP ID while configuring Linkset.

    - CPC ID is not allowed to configure as LSP ID in VstpLinkset MO.

  - VstpSccpApplications

    - Restrict deletion of tuple if matching "appType" entry exists in VstpLocalSP MO.

    - Allow deletion only if no matching entry exists in VstpLocalSP MO.

#### MEALS

      - Measurements

#### No measurement added/modified for this feature.

      - Alarms & Events

Following Event is reused by this feature, with a new reason code.

Table 69 – Alarms & Description

| Alarm/Event Name | Type | Description | Raise Condition | Clear Condition | Throttle Sec | Instance | Additional Information | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SLTC failure (70378) | Event | Signaling Link Test Failure | While handling SLTM/SLTA message if DPC of the message doesn’t match with the TPC/SPC provisioned against the OPC | NA | NA | None | Message Type, DPC & OPC of the message | NA |

Table 70 – Events & Description

| Alarm/Event Name |  |  | Type | Description | Raise Condition | Clear Condition | Throttle Sec | Instance | Additional Information | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SCCP Application MSU Discarded (70416) |  |  | Event | SCCP Application MSU Discarded | Data message having DPC=CPC with cpctype as “EIR” but message SSN does not match with the one provisioned in VstpSccpApplications table or the application admin state is “disabled” | NA | 10 | None | Contains rejection reason which is either SSN mismatch or App admin state being “disabled” | NA |

## VSTP FLOW CONTROL ENHANCEMENTS

#### Details of feature are not applicable as, it is an engineering feature.

## VSTP ANSI-ITU CONVERSION

#### Purpose and Solution

Purpose:

  - During cross domain signaling some ANSI and ITU SCCP and/or MTP3 user message parameters are incompatible in format.

  - Hence there is  a need  for conversion capability that will correctly format and decode/encode  SCCP parameters.

  - There is a need for conversion capability to support UDT(S) and XUDT(S) across inter-network traffic.

Solution:

vSTP ANSI ITU Conversion Feature have been developed to perform cross domain conversion over inter-network traffic( UDT(S) and XUDT(S) messages, MTP routed, GT routed).

Feature Overview:

- Cross domain (ANSI/ITU) signaling can be MTP routed or GT routed.

- Cross domain conversion combination supported across domains are

- ANSI, ITU-I(S) ,ITU-N(S)

- ITU-N24, ITU-I(S)

- Based on message type, ANSI/ITU Conversion supports UDT, XUDT, UDTS and XUDTS messages.

- The feature also provides SCCP management (SCMG) across network type boundaries

- This feature also provide support of China Point Code SCCP conversions to ITU-International.

SCCP Conversion Overview:

      - MTP Routed UDT(S)/XUDT(S) Conversion

      - MTP Routed UDT(S)/XUDT(S) SCMG Conversion

      - GT Routed UDT(S)/XUDT(S)  Conversion

    - SCCP Conversion Details

      - Conversion of MTP Routing labels

      - Conversion of SCCP parameters

      - SCCP Conversion through GTT Action

- MTP routed SCCP messages (UDT(S)/XUDT(S)) are converted in the following steps:

  - MTP conversion

  - SCCP Conversion

- GT routed SCCP messages (UDT(S)/XUDT(S)) are converted in the following steps:

  - MTP conversion

  - SCCP Conversion

Table 71 – MO details for GTT action feature

| MO Name | Operations supported | URI |
| --- | --- | --- |
| Remote Signaling Points | Insert, Update, Delete | /vstp/remotesignalingpoints |
| GTT Sets | Insert, Update, Delete | /vstp/gttsets |
| GTT Selectors | Insert, Update, Delete | /vstp/gttselectors |
| Global Title Addresses | Insert, Update, Delete | /vstp/globaltitleaddresses |
| GTT Modifications | Insert, Update, Delete | /vstp/gttmods |
| Default Conversions | Insert, Update, Delete | /vstp/defaultconversions |

- Refer MMI API Guide on Active NOAM/SOAM: “Main Menu ->MMI API Guide” on any DSR 8.4 GA release setup for details about the URI, example and parameters about each MO.

# Release 8.4.0.3.0 Feature OAM Changes

At the time of upgrade to DSR 8.4.0.3.0 a number of features and enhancements will become visible on the interfaces to the DSR and may change certain existing OAM behaviors of the system.

OAM changes includes: User Interfaces (NO GUI, SO GUI), Measurements Reports, Alarms, and KPIs.

## GTT THROTTLE ACTION

#### Purpose and Solution

Purpose

- This feature is a part of SS7 Security Firewall.

- More attention is being given to the security aspects in SS7 networks.

- Mobile Interconnects is generally used for SS7 network security attacks. Specifically spam and DoS attacks.

- To prevent these attacks Interconnect level (SCCP level) traffic flow control is needed.

- On Sigtran linkset, vSTP provides ingress throttling by using the per link data rate.

- However, there is no support for Egress throttling (traffic control) of GTT messages in vSTP.

- GTT Throttle feature provides the support for Egress throttling of GTT messages in vSTP.

Solution

- The GTT Throttle Feature design is both simple and flexible by adding a new type of GTT Action SFTHROT.

- For each GTT Action, user will provision threshold as maximum number of MSUs hitting the GTT action per second.

- SMS framework is used to accumulate the total number of MSU count per SFTHROT action.

- When a MSU hits a GTT action of the type SFTHROT, the MSU count of that action is updated.

- SMS framework accumulates the total number of messages per SFTHROT action on MP Leader and sends cumulative count to all MPs across site.

- If the cumulative count of messages has crossed the provisioned threshold, MPs will start throttling that messages.

- Any MSU hitting that GTT action will get discarded and the MSU count of that messages is not increased due to throttling, due to which cumulative value will decreased in next sliding window. Once the cumulative value drops below configured threshold it will allow messages.

Feature Overview

The GTT Throttle action works based on the following rules:

- When an MSU hits a GTT action of the type SFTHROT, the MSU count of that action gets updated.

Note: The Shared Metric Service (SMS) framework is used to accumulate the total number of MSU count per SFTHROT action.

- The MSU count is updated only on the Message Processor (MP) on which the message is received for that action. On the other hand, the Threshold configuration for SFTHROT action is across the MPs.

Note: For each GTT Action, user provisions a threshold value that is the maximum number of MSUs hitting the GTT action per second.

- Two sysmetrics are registered. The first is for MSU count per MP and second one for cumulative MSU count across the site.

- Aggregation of the MSU count from all the MPs is done by the MP Leader. There is only one MP leader across the site. It performs the aggregation of MSU counts. Rest of the MPs across the site are known as followers.

- Whenever a message comes to any MP, it will increment the sysmetric count of that MP known as local sysmetric count. All the follower MPs will send the local sysmetric count data to the MP Leader to get the aggregated value of that action.

- The MP Leader will receive the data from all the other MPs including it's own local sysmetric count. It will do the aggregation and broadcast the cumulative count to all the MPs.

- The SMS framework is used to send local sysmetric count to MP leader and receive the aggregated sysmetric count from it. The aggregation of the count is taken care by SMS framework hence, any degradation in SMS service will impact the feature.

- When GTT message is received for SFTHROT action, then the aggregated sysmetric count is compared with the configured threshold value for that action:

If the aggregated sysmetric count value is lesser than the configured threshold value, then the message is allowed and the local sysmetric count value is increased by 1.

If the aggregated sysmetric count value is more than the configured threshold value, then the local sysmetric count value does not get increased due to throttling. The GTT message is discarded, discard measurement is pegged for that action, and an alarm is raised.

- The alarm will get cleared once the aggregated sysmetric count drops below 90% of the configured threshold value.

- As there is no local sysmetric is pegged, the aggregated count will be decreased in next sliding window. Convergence time is 2 sec.

- Once the cumulative value drops below the configured threshold, it will allow the GTT messages for that action and the local sysmetric count will be increased.

Note: For GTT Throttle action, an error margin of +2% to -2% of the provisioned threshold value must be considered. The error margin depends on the cloud infrastructure load & burst pattern of incoming traffic.

The following figure shows the process flow for GTT Throttle action:

Figure 31 – Process Flow of GTT Throttle Action

#### MEALS

      - Measurements

Table 72 – Measurements

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| VstpThrottleActionMsgRatePeak | Arrayed | The peak number of messages Aggregated per GTT Throttle Action. | 5 | Performance | Max |
| VstpThrottleActionMsgRateAvg | Arrayed | The Average number of messages Aggregated per GTT Throttle Action. | 5 | Performance | Average |
| VstpThrottleActionMsgDiscard | Arrayed | The number of messages discarded per GTT Throttle Action. | 5 | Exception | Simple |

      - Alarms & Events

No event is added/modified for this feature.

Following is the alarm supported by GTT Throttle Action Feature:

Table 73 – Alarms for GTT Throttle Action Feature

| Alarm/Event Name | Type | Description | Raise Condition | Clear Condition | Throttle Sec | Instance | Additional information | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sccp Egress Tps Threshold Crossed(70418) | Alarm | Sccp Egress Tps Threshold Crossed. | When incoming message rate for actid SFTHROT is 100% of the provisioned threshold. | When incoming message rate for actid SFTHROT is reduced to 90% or less of the provisioned threshold. | 86400 | GTTAction | Ta index | Major |

Limitation

There is error margin of +2% to -2% of the provisioned threshold value depending on the cloud infrastructure load & burst pattern of incoming traffic.

Dependencies

The GTT Throttle action support for vSTP has no dependency on any other vSTP operation.

Troubleshooting Steps

In case of error scenario, check the incoming traffic. The incoming traffic must be 100% or above the provisioned threshold value for respective actid with SFTHROT action.

## GTT SCPVAL ACTION

#### Purpose and Solution

Purpose

In certain MAP operations, some of the MAP parameters are expected to be same as either the SCCP CdPA or CgPA. At present, vSTP has no validation checks for the same.

Solution

With SS7 Firewall, a new GTT Action will be added to perform validation on MAP parameters. It makes comparison of the SCCP and MAP digits. The feature achieves it by adding a new GTT Action, SCPVAL, along with relevant parameters.

Feature Overview

- In certain MAP operations, some of the MAP parameters are expected to be same as either the SCCP CdPA or CgPA.

- A new GTT Action will be added to do such validation.

- This validation will be done only on MO-FSM and MT-FSM messages coming to vSTP.

- A new GTT Action “SCPVAL” is introduced for this task, which will have the following parameters:

- SPRM

- TPRM

- NDGT

- USEICMSG

- UIMREQD

- DEFACTID

Table 74 – GTT Action SCPVAL – Parameters

| Field Name | Description | Default Value |
| --- | --- | --- |
| ACT | GTT Action Existing Values: Disc, Dup, Fwd, Srvc, Tcaperr, Udts New Value: SCPVAL | Mandatory Enum |
| SPRM | SCCP Parameter | Mandatory Values: CGGTA/CDGTA |
| TPRM | TCAP Parameter | Mandatory Values: SMRPOA/SMRPDA |
| NDGT | Number of digits to be matched. Specifies the number of digits that needs to be matched between SCCP parameter and MAP parameter. | Optional Values: 1-21, All Default value: All |
| USEICMSG | Use Incoming Message. Specifies whether to retrieve the data for comparison from the original or post GTT message. | Values: ON/OFF OFF: Use original, i.e. as the message was received by SCCP ON: Use post-GTT, i.e. after possible EPAP/GTT translation/modification data has been applied |
| UIMREQD | UIM Required. Specifies whether to generate event in case GTT Action failure. | Values: ON/OFF |
| DEFACTID | Default Action ID. The default action that is performed when SCPVAL GTT Action fails. | String |

The flowchart depicts the implementation of MAP SCCP validation.

Figure 32 – Flow Diagram for SCPVAL

#### MEALS

      - Measurements

Following is the list of measurements supported by SCPVAL Feature:

- VstpCdpaGttActScpvalTotal: The number of messages that successfully pass SCPVAL CdPA GTT action.

- VstpCdpaGttActScpvalDiscard: The number of messages discarded by SCPVAL CdPA GTT action.

- VstpCdpaGttActScpvalNotApplied: The number of messages where validation was not applied by SCPVAL CdPA GTT action.

- VstpCgpaGttActScpvalTotal: The number of messages that successfully pass SCPVAL CgPA GTT action.

- VstpCgpaGttActScpvalDiscard: The number of messages discarded by SCPVAL CgPA GTT action.

- VstpCgpaGttActScpvalNotApplied: The number of messages where validation was not applied by SCPVAL CgPA GTT action.

      - Alarms & Events

Following is the list of events supported by SCPVAL Feature:

Table 75 – Alarms for GTT SCPVAL Action Feature

| Alarm/Event Name | Type | Description | Raise Condition |
| --- | --- | --- | --- |
| GTT Action Failed (70278) | Event | GTT Action Failed | When any one of the GTT Action (i.e. DUPLICATE, FORWARD, TCAP ERROR, SCPVAL) fails and UIMREQD is set to ON. |

Limitation

This feature supports the validation only on the following messages coming to the vSTP:

- MO-FSM (MAP version 2 or 3)

- MT-FSM (MAP version 3)

Dependencies

The SCPVAL action has no dependency on any other vSTP operation.

Troubleshooting Steps

The following are the troubleshooting scenarios for SCPVAL action:

- If an incoming MSU successfully passes SCPVAL CdPA GTT action, then VstpCdpaGttActScpvalTotal measurement will be pegged on a per Linkset basis.

- If validation was not applied by SCPVAL CdPA GTT action on an incoming message, VstpCdpaGttActScpvalNotApplied will be pegged on a per Linkset basis.

- If incoming MSU is discarded by SCPVAL CdPA GTT action, then VstpCdpaGttActScpvalDiscard measurement will be pegged on a per Linkset basis.

- If validation was not applied by SCPVAL CgPA GTT action on an incoming message, VstpCgpaGttActScpvalNotApplied will be pegged on a per Linkset basis.

- If an incoming MSU successfully passes SCPVAL CgPA GTT action, then VstpCgpaGttActScpvalTotal measurement will be pegged on a per Linkset basis.

- If incoming MSU is discarded by SCPVAL CgPA GTT action, then VstpCgpaGttActScpvalDiscard measurement will be pegged on a per Linkset basis.

- When anyone of the GTT Action (i.e. DUPLICATE, FORWARD, TCAP ERROR, SCPVAL) fails and UIMREQD is set to ON, then event 70278 GTT Action Failed will be generated. It contains error cause with SCCP and TCAP details, GTT Action set name and linkset ID.

If any of the above statement fails as per given scenarios, then verify configuration.

If issue still exists, then contact Oracle for support.

## MTP BASED GTT

#### Purpose and Solution

Purpose

- Support for MTP based GTT with Screening Action feature in vSTP.

- GTT and GTT Actions were not performed on MTP Routed MSU’s prior to implementation of this feature in vSTP.

Solution

- The MTP based GTT with Screening Action feature enhanced vSTP’s capability of performing SCCP services on MTP-routed messages.

- This vSTP feature allows the operator to perform GTT and GTT Actions on MTP Routed MSUs similar to existing GTT handling for GT Routed MSU’s.

Feature Overview

The MTP based GTT with Screening Action is performed if the service handling results in Fall through to GTT or if GTT Required option is ON for Service Relayed MSU.

The following system-wide options are used to configure this functionality:

- MTP Routed GTT

The MTP Routed GTT (mtprgtt) option is used for MTP Routed GTT functionality as follows:

- If option = OFF, then GTT shall not be performed on MTP Routed MSUs.

- If option = Use MTP Point codes, then GTT shall be performed on MTP Routed MSU, SCCP Portion shall be updated based on translation entry but MSU shall be sent to Original DPC (and not to translated DPC).

- If option = Full GTT, then GTT shall be performed on MTP Routed MSU, SCCP Portion as well as MTP Portion shall be updated based on translation results.

- MTP Routed GTT fallback

The MTP Routed GTT fallback (mtprgttfallbk) option is used for error handling to be performed in case of GTT failure for MTP routed MSUs. It has the following values:

- If option = GTT failure, then MSU will be discarded with appropriate UIM. UDTS will be sent to originator and measurements shall be pegged as done for GT routed messages.

- If option = Fall back to MTP routing, then MSU (with translation/modification/ routing data from UDR-related service) shall be MTP routed.

The support for the following features is required for the functionality of MTP based GTT:

- SCCP Stop Action: provide a means for the operator to specify SCCP Stop Action in the MTP Screening Rules, to allow the MTP processing to fall through to GTT on non-discarded MSUs.

- XLAT = NONE: provide a means for the operator to specify GTT Translation Type

- = NONE.

- GTT SET = DPC: A new GTT set, DPC (with set type dpc) shall be supported. The provisioning and behavior of the DPC Translations shall be same as OPC Translations. However, DPC GTT set cannot be used as secondary optional set (i.e. DPC GTT set cannot be assigned to OPCSN parameter in translation entry). The DPC GTT set type can be searched only when the GTT hierarchy is FLOBR specific.

#### MEALS

      - Measurements

Table 76 – Measurements

| Meas ID | Name | Report Group | Collection Interval | Dimension |
| --- | --- | --- | --- | --- |
| 21304 | VstpRxMSUMtpRoutedSccp | VSTP MTP3 Performance | 5 min | Single |

      - Alarms & Events

No specific Alarms and Events are generated for MTP based GTT.

Limitation

- This feature does not implement screening based on SCCP layer parameters. The feature scope is limited to screening based on MTP3 layer parameters.

Dependencies

The MTP based GTT support for vSTP has no dependency on any other vSTP operation.

Troubleshooting Steps

In case of the error scenarios, the measurements specific to MTP based GTT feature are pegged. For information related to MTP based GTT measurements, see MTP Based GTT Alarms and Measurements.

## SFAPP STATEFUL SECURITY

#### Purpose and Solution

Purpose

- Categories of messages that need to be monitored and handled to provide security to home networks from messages coming in from spurious sources with identification information of the subscribers roaming out. The messages coming in for a subscriber, roaming out of the network are in a sequence and to truly validate the veracity of the subscriber, a state information is required. This state information can be about the last message, last location, last activity time.

- Following category of messages has been identified to be protected against in the SS7 networks

Category 3: Outbound roaming MAP messages.

Category 3.1: VLR Check MAP messages

Category 3.2: Time, Location Check messages

The following Category 3 packets are worthy of monitoring and analysis on this basis:
•    UpdateLocation
•    UpdateGPRSLocation
•    PurgeMS
•    RegisterSS
•    EraseSS5
•    ActivateSS5
•    DeactivateSS5
•    InterrogateSS5
•    ProcessUnstructuredSSRequest5
•    SendAuthenticationInfo
•    RestoreData
•    NoteMMEvent
•    SendParameters5

Solution

This feature would allow the vSTP to validate the messages coming in for a subscriber roaming out by validating them against the VLR the subscriber was last seen by the HLR. If the HLR provides a validity of the new VLR the vSTP would let the message into the network if not, the message would be handled per configuration (either silent discard or respond with error).

Feature Overview

Stateful Applications (SFAPP) allows vSTP to validate the messages coming in for a subscriber by validating them against the Visitor Location Register (VLR). The last seen details of the subscriber can be fetched from the Home Location Register (HLR). Once the HLR provides a validity of the new VLR, vSTP then allows the message into the network. If the message is not validated, it is handled as per configuration (either silent discard, fallback, or respond with error).

#### MEALS

      - Measurements

-Table 77 – Measurements

| Meas ID | Name | Description | Group | Interval | Dimension |
| --- | --- | --- | --- | --- | --- |
| 21702 | VstpSfappMsgSuccess | Total number of messages that passed VLR validation. | Vstp SFAPP Performance | 5 min | Array |
| 21703 | VstpSfappMsgFailed | Total number of messages failed VLR validation | Vstp SFAPP Performance | 5 min | Array |
| 21704 | VstpSfappMsgError1 | Total number of Vstp generated SFAPP messages with validation errors | Vstp SFAPP Performance | 5 min | Array |
| 21705 | VstpSfappMsgError2 | Total number of Vstp generated SFAPP messages with validation errors | Vstp SFAPP Performance | 5 min | Array |
| 21706 | VstpRxSfappMsg | The total number of messages received to SFAPP. | Vstp SFAPP Performance | 5 min | Single |
| 21707 | VstpRxSfappMsgDiscard | The number of SFAPP messages that have been received and discarded | Vstp SFAPP Exception | 5 min | Single |
| 21708 | VstpSfappInternalError | Number of messages discarded due to internal processing error | Vstp SFAPP Exception | 5 min | Single |
| 21709 | VstpSfappCADecodeFail | Number of SfApp CA response discarded due to decode failed | Vstp SFAPP Exception | 5 min | Single |
| 21710 | VstpSfappCATimeOut | Number of messages for which CA query to UDR timed out | Vstp SFAPP Exception | 5 min | Single |
| 21711 | VstpSfappCAAvgProcessTime | Average Sfapp CA query response time from UDR. | Vstp SFAPP Performance | 5 min | Single |
| 21712 | VstpSfappCAMaxProcessTime | Peak time by CA to send query and receive the response from UDR for Sfapp Messages | Vstp SFAPP Performance | 5 min | Single |
| 21713 | VstpSfappSubsNotFound | Number of subscriber record not in UDR DB | Vstp SFAPP Exception | 5 mn | Single |
| 21714 | VstpSfappCATx | Number of DB request sent by vSTP | Vstp SFAPP Performance | 5 min | Single |
| 21715 | VstpSfappCATxFail | Number of messages discarded by SFAPP because of send fail to CA layer. | Vstp SFAPP Exception | 5 min | Single |
| 21716 | VstpSfappPduFull | Number of messages discarded when PDU pool is exhausted | Vstp SFAPP Exception | 5 min | Single |
| 21717 | VstpSfappCAProcesTime | Time required by CA to send query and receive the response from UDR | Vstp SFAPP Performance | 5 min | Single |
| 21718 | VstpSFAPPStackQueuePeak | The peak VSTP SFAPP Stack Event Queue utilization measured during the collection interval. | VSTP SFAPP Performance | 30 min | Arrayed |
| 21719 | VstpSFAPPStackQueueAvg | The average VSTP SFAPP Stack Event Queue utilization measured during the collection interval. | VSTP SFAPP Performance | 30 min | Arrayed |
| 21720 | VstpSFAPPStackQueueFull | The number of ingress SFAPP messages that were discarded because the VSTP SFAPP Stack Event Queue was full. | VSTP SFAPP Exception | 30 min | Arrayed |
| 21782 | VstpTxSfappMsg | The total number of messages transmitted from SFAPP. | VSTP SFAPP Performance | 5 miin | Single |
| 21783 | VstpTxSfappMsgPeak | The peak number of messages transmitted from SFAPP | VSTP SFAPP Performance | 5 min | Single |
| 21784 | VstpTxSfappMsgAvg | The average number of messages transmitted from SFAPP | VSTP SFAPP Performance | 5 min | Single |

      - Alarms & Events

Following is the list of events/alarms supported by MTP Screening Feature:

Table 78 – Alarms for SFAPP Stateful Security Feature

| Alarm/Event Name | Type | Description | Raise Condition | Clear Condition | Throttle sec | Instance | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- |
| SFAPP Validation Error | Event | When SFAPP decode fails | When SFAPP message decodeing fails |  | 10 | None | Info |
| SFAPP Validation Matching State not found | Event | SFAPP Validation Matching State not found | When SFAPP Validation Matching State not found. |  | 10 | None | info |
| SFAPP Validation Encoding Error | Event | SFAPP Validation Encoding Error. | When Sfapp message encoding fails. |  | 10 | None | info |
| SFAPP Validation Response Timeout Error. | Event | SFAPP Validation Response Timeout Error. | When SFAPP Validation Response Timeout Error. |  | 10 | None | info |
| SFAPP Validation Velocity Chk Failed. | Event | SFAPP Validation Velocity Chk Failed. | When SFAPP Validation Velocity Chk Failed. |  | 10 | None | info |
| SFAPP Validation Failed | Event | SFAPP Validation Failed | When SFAPP message Validation Failed |  | 10 | None | info |
| SFAPP Invalid CC/NDC received | Event | SFAPP Invalid CC/NDC received | When invalid CC/NDC received in VLR of Sfapp message |  | 10 | None | info |
| Updation failed in UDR | Event | Updation failed in UDR | When updation is failed on UDR for IMSI record. |  | 10 | None | Info |
| VSTP SFAPP Stack Event Queue Utilization | Alarm | The percent utilization of the VSTP MP's SFAPP Event Queue is approaching its maximum capacity | When the percent utilization of the VSTP MP's SFAPP Event Queue is approaching its maximum capacity | The percent utilization of the VSTP MP's SFAPP Event Queue comes back to normal | 10 | None | Major |

Troubleshooting Steps

vSTP Sfapp messages will be discarded in following scenarios :

Check whether Sfapp Thread CPU utilization exceeded Congestion Level 2

  - This check is performed at the beginning of message processing cycle and if set vSTP immediately responds with default response

Check whether Sfapp Application operational state is “Unavailable”

  - Check whether Sfapp operational state in VstpSccpAppStatus table is set to “Unavailable”

  - vSTP performs this check before sending the message to UDR and if the state “Unavailable” it sends default response; the query is not sent to UDR anymore

  - VstpSfappCATimeOut meas is pegged in this case

## TDM SUPPORT

#### Purpose and Solution

Purpose

At present, VSTP provides IP connectivity for SS7 network. However, the legacy TDM links are still running on the legacy platform. The replacement of legacy (typically old) systems with VSTP becomes infeasible for transition because of lack of TDM support in VSTP.

Solution

vSTP TDM Support feature provides access to E1/T1 links based ADAX HDC3 PCIe TDM Card using PCIe Pass Through. This solution involves following components :

- TDM Hardware: ADAX HDC3 PCIe card with physical TDM connectivity supporting Virtual IO. This card shall have built-in processor to process the MTP2 layer on hardware itself.

- VSTP MP with MTP NIF functionality: An additional (MTP Network Interworking Function - NIF) layer will be added to existing VSTP MP so that the MTP3 Layer can communicate with the MTP2 layer running on the TDM PCIe Card.

- The MTP2 Adapter (NIF) layer on VSTP MP shall communicate with MTP2 layer using Virtual-IO calls.

- The Host machine shall allow PCI Pass-through Access to the VSTP MP virtual machines.

Feature Overview

The TDM support functionality includes the following components

- TDM Hardware: The hardware involves Adax HDC3 PCIe card with physical TDM connectivity supporting Virtual IO. This card contains built-in processor to process the MTP2 layer on hardware itself.

Adax HDC3 PCIe card supports direct access using PCIe Pass-through. Therefore, a single Adax 4-port or 8-Port HDC3 PCIe card can be accessed only from a single VM at a time.

- MTP Network Interworking Function (NIF): An additional MTP NIF layer is added to existing vSTP MP so that the MTP3 Layer can communicate with the MTP2 layer running on the TDM PCIe Card.

The M3RL layer in vSTP MP VM communicates with the MTP2 layer running on the Adax HDC3 card via the MTP2 Adapter layer.

- MTP2 Adapter: The MTP2 Adapter NIF layer on vSTP MP communicates with MTP2 layer using Virtual-IO calls. It uses the libraries and APIs provided by Adax to communicate with Adax HDC3 Card.

- Host machine: The Host machine allows PCI Pass-through access to the vSTP MP virtual machines.

#### MEALS

      - Measurements

Table 79 – Measurements

| Meas ID | Name | Report Group | Collection Interval | Dimension |
| --- | --- | --- | --- | --- |
| 21800 | VstpMtp2LnkOutageDuration | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21804 | VstpMtp2LnkAvailableDuration | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21805 | VstpMtp2RxLnkMSUOctets | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21806 | VstpMtp2RxLnkMSUOctetsForGTT | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21807 | VstpMtp2TxLnkMSUOctets | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21808 | VstpMtp2Priority0MsuDiscarded | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21809 | VstpMtp2Priority1MsuDiscarded | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21810 | VstpMtp2Priority2MsuDiscarded | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21811 | VstpMtp2Priority3MsuDiscarded | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21812 | VstpMtp2RxLnkMSU | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21813 | VstpMtp2RxLnkMSUForGTT | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21814 | VstpMtp2TxLnkMSU | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21816 | VstpMtp2LnkMaintUsage | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21821 | VstpMtp2LnkCO | VSTP MTP2 Performance | 5 min | Arrayed (Link Id) |
| 21823 | VstpMtp2OOSDuration | VSTP MTP2 Exception | 5 min | Single |
| 21826 | VstpMtp2LnkCumlInhibitDuration | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21827 | VstpMtp2LnkRemoteInhibitDuration | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21828 | VstpMtp2RxLnkMSUError | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21833 | VstpMtp2LnkLocalInhibit | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21835 | VstpMtp2LnkTotalOutage | VSTP MTP2 Exception | 5 min | Single |
| 21839 | VstpMtp2RxLnkMSUInError | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21840 | VstpMtp2LnkTotalActiveDuration | VSTP MTP2 Exception | 5 min | Arrayed (Link Id) |
| 21841 | VstpMtp2LnkTotalUnAvailableDuration | VSTP MTP2 Exception | 5 min | Single |

      - Alarms & Events

Following is the list of events supported by MTP2 Links:

Table 80 – Events for TDM Support Feature

| Event Name | ID | Raise Condition |
| --- | --- | --- |
| MTP2 Link admin state change | 70220 | The MTP2 link administrative state is manually changed from one administrative state to another (e.g. Disabled to Enabled and vice versa) |
| Failed to send message to TDM driver | 70221 | This event is generated when sending message to TDM driver fails. |
| Failed to receive message from TDM driver | 70222 | This event is generated when read message from TDM driver fails. |
| MTP2 link operational state changed | 70223 | This event is generated when MTP2 link operational state is changed |
| MTP2 link failed | 70224 | This event is generated when MTP2 link is failed due to Link Out Of Service Message Received from peer or MTP2 Link Stop Request Received |
| MTP2 Ingress message discarded | 70225 | This event is generated when MTP2 Ingress message is discarded |
| MTP2 Egress message discarded | 70226 | This event is generated when MTP2 Egress message is discarded |
| Received Remote Out Of Service on MTP2 link | 70227 | This event is generated when Remote Out Of Service is received from peer on MTP2 link. |

Following is the list of alarms supported by MTP2 Links:

Table 81 – Alarms for TDM Support Feature

| Alarm ID | Alarm Name | Raise Condition |
| --- | --- | --- |
| 70001 | Link Down | New Link Added or Link manually Disabled. |
| 70005 | Link Unavailable | MTP2 has reported to MTP3 that a link is out of service. |
| 70009 | Link Congested | Link Congestion alarm is derived from link buffer utilization. Minor (>= 60%), Major (>=80% ), Critical (>=90%), % level of link buffer capacity. |
| 70102 | MTP3 Ingress Link MSU TPS Crossed | Ingress MTP3 signaling traffic rate is above Minor (>= 60%), Major (>=80% ), Critical (>=90%), % level of Max = VstpLinkset table: linkTps |
| 70103 | MTP3 Egress Link MSU TPS Crossed | Egress MTP3 signaling traffic rate is above Minor (>= 60%), Major (>=80% ), Critical (>=90%), % level of Max = VstpLinkset table: linkTps |
| 70104 | MTP3 Ingress Link Management TPS Crossed | Ingress MTP3 SNM rate is above Critical (>=20%), % level of Max = VstpLinkset table: linkTps |
| 70084 | VSTP MTP2 Transmission and Retransmission Buffer Utilization | MTP2 Link Buffer Utilization is above Minor (>= 60%), Major (>=80% ), Critical (>=95%) |

Limitation

- Current implementation doesn’t support J1 and ATM interfaces.

- One VSTP MP VM can support only one 4-Port ADAX HDC3 Card.

- Single ADAX HDC3 card cannot be accessed from Multiple VSTP MP VMs.

- The ADAX HDC3 driver and required components are not packaged with Standard DSR ISO in current release. These components and RPMs have to be installed separately.

Dependencies

The TDM support for vSTP has no dependency on any other vSTP operation.

Troubleshooting Steps

The following are the troubleshooting scenarios for TDM support:

- The E1/T1 links do not align properly

Do the following to troubleshoot:

- Verify that the cable is not faulty.

- Verify the cable connections.

- Verify that the Adax HDC3 card configuration (in QCXfile) is as per the Interface Mapping configuration.

- Ensure that the Adax HDC3 card timing source configuration is correct. In case of SUERM errors, modify the timing source.

- Frequent toggling of the E1/T1 Links

Do the following to troubleshoot:

- Verify that the point codes associated with the linkset are correct.

- Verify that the link alignment and SLTM timers are correct.

- Adax HDC3 Card is not detected on a vSTP MP VM

Do the following to troubleshoot:

- Check that the vSTP MP VM and the Adax HDC3 card are co-located on same host machine.

- Check the Adax HDC3 RPMs.
The following RPMs are required on vSTP MP VM for configuring Adax HDC3 Card:

- Adax-LiS-2.21.8-1-RedHat-6.10-x86-64bit.rpm

- Adax-hdc-1.79-1-RedHat-6.10-x86-64bit-LiS2.21.8-MAJ234.rpm

- Adax-qcx-1.25-1-Linux-x86-64bit.rpm

Points to Consider

The following points must be considered while configuring TDM:

•	The J1 and ATM interfaces are not supported.

•	Single vSTP MP VM can support only one 4-Port Adax HDC3 Card.

•	An Adax HDC3 card cannot be accessed from Multiple VSTP MP VMs .

•	The Adax HDC3 driver components and RPMs needs to be installed separately.

## M3UA CLIENT SUPPORT

#### Purpose and Solution

Purpose

- Initially vSTP can work only as M3UA SGP but with introduction of this feature now vSTP can work as M3UA ASP/AS.

- Support for M3UA Client mode feature in vSTP.

- M3UA client support allow the vSTP to trigger the M3UA connection initiation.

Solution

M3UA client support provide following functionality  :

- Configuration of M3UA as client through connections mmi configuration.

- Initiation of  ASP State Maintenance messages (ASP-UP, ASP-Active …) .

- Receiving and processing of SS7 Signaling Network Management messages (DAVA,DUNA ,DUPU, DRST, DAUD and SCON). messages.

Feature Overview

The MTP3-User Adaptation (M3UA ) Client support allows vSTP to trigger the M3UA connection initiation. For information related to M3UA Protocol, refer to RFC 4666.

The M3UA client support over vSTP enables a user to achieve the following functionalities:

- Initiation of SCTP connection to send INIT message to the server.

- Initiation of ASP state maintenance messages such as, ASP-UP, ASP-Active etc.

- Receiving and processing of SS7 Signaling Network Management messages such as, DAVA, DUNA, DUPU, DRST, DAUD and SCON.

- Receiving and processing of M3UA notify messages (NTFY).

- M3UA peer receiving the DATA message sends an MTP-TRANSFER indication primitive to the upper layer.

- On receiving an MTP-TRANSFER request primitive from an upper layer at an ASP the M3UA layer sends a corresponding DATA message to its M3UA peer.

- The M3UA message distribution function determines the Application Server (AS) by comparing the information in the MTP-TRANSFER request primitive with a provisioned Routing Key.

Message Flow

The following figure shows the message flow for M3UA client server functionality, where, SGP acts as the M3UA server and ASP is the M3UA client:

Figure 2-22    Message Flow for ASP - M3UA Client

#### MEALS

      - Measurements

Table 82 – Measurements

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| VstpTxM3uaDataMsg | Single | M3UA User DATA messages sent by VSTP server | 30 | Performance | Simple |
| VstpRxM3uaDataMsg | Single | M3UA User DATA messages received by VSTP server | 30 | Performance | Simple |
| VstpTxM3uaDataOctets | Single | M3UA DATA octets sent by the VSTP server | 30 | Performance | Simple |
| VstpRxM3uaDataOctets | Single | M3UA DATA octets received by the VSTP server | 30 | Performance | Simple |
| vSTPTxAsnOctets | Single | The number of octets sent on an association | 30 | Performance | Simple |
| vSTPRxAsnOctets | Single | The number of octets received on an association | 30 | Performance | Simple |
| VstpTxASPUp | Single | The number of ASP Up messages sent by the VSTP M3ua client. | 30 | Performance | Simple |
| VstpTxASPDown | Single | The number of ASP Down messages sent by the VSTP M3ua client. | 30 | Performance | Simple |
| VstpTxHeartbeat | Single | The number of Heartbeat messages sent by the VSTP M3ua client. | 30 | Performance | Simple |
| VstpTxASPActive | Single | The number of ASP Active messages sent by the VSTP M3ua client. | 30 | Performance | Simple |
| VstpTxASPInactive | Single | The number of ASP Inactive messages sent by the VSTP M3ua client. | 30 | Performance | Simple |
| VstpRxDUNA | Single | The number of DUNA messages received by the VSTP M3ua Client. | 30 | Performance | Simple |
| VstpRxDAVA | Single | The number of DAVA messages received by the VSTP M3ua Client. | 30 | Performance | Simple |
| VstpRxDAVA | Single | The number of DAVA messages received by the VSTP M3ua Client. | 30 | Performance | Simple |
| VstpRxDUPU | Single | The number of DUPU messages received by the VSTP M3ua Client. | 30 | Performance | Simple |
| VstpRxDRST | Single | The number of DRST messages received by the VSTP M3ua Client. | 30 | Performance | Simple |
| VstpTxDAUD | Single | The number of DAUD messages sent by the VSTP M3ua Client. | 30 | Performance | Simple |
| VstpRxASPUpAck | Single | The number of ASP Up Ack messages received by the VSTP client. | 30 | Performance | Simple |
| VstpRxASPDownAck | Single | The number of ASP Down Ack messages received by the VSTP client. | 30 | Performance | Simple |
| VstpRxASPActiveAck | Single | The number of ASP Active Ack messages received by the VSTP client. | 30 | Performance | Simple |
| VstpRxASPInactiveAck | Single | The number of ASP Inactive Ack messages received by the VSTP client. | 30 | Performance | Simple |
| VstpRxM3uaNotify | Single | The number of M3UA NOTIFY messages recevied by the VSTP client. | 30 | Performance | Simple |

      - Alarms & Events

No new event or alarm added for this feature.

Dependencies

The M3UA Client support for vSTP has no dependency on any other vSTP operation.

Troubleshooting Steps

In case of the error scenarios, the measurements specific to M3UA client support feature are pegged. For information related to M3UA measurements, see M3UA Client Support Alarms and Measurements.

## VSTP IDPR MOSMS

#### Purpose and Solution

Purpose

- Support for Prepaid IDP Query Relay (IDPR) feature in vSTP.

- Support for Mobile Originated Short Message Service (MOSMS) feature in vSTP.

Solution

- Mobile Switching Centers (MSCs) in the network are configured to send IDP prepaid query messages through vSTP, the vSTP intercepts the IDP query based on a set of configurable criteria, performs a number portability (UDR) lookup on the called number, inserts the portability information (Routing Number or HLR Address), and forwards the IDP query to a prepaid SCP for processing. When a respective entry is found in the UDR, any processing is controlled by NPP Service Actions and configuration option provisioning in the IDPROPTS table. The CdPN can be modified with the portability information (Routing Number or HLR address) and the CgPN.

- The Mobile Originated Short Message Service (MOSMS) feature address the number portability requirements of wireless network operators for delivery of Mobile Originated SMS messages. The vSTP MOSMS features apply number portability database lookup to SMS messages for GSM networks, validates subscriber use of the correct Short Message Service Center, and delivers messages to Prepaid Servers if either the Calling Party Number or Called Party Number is associated with a prepaid subscriber.

Feature Overview

The Prepaid IDP Query Relay feature (IDP Relay) provides a mechanism to ensure the correct charging for calls from prepaid subscribers in a portability environment.

IDP Relay processes GT-routed INAP or CAP messages with ITU MTP/SCCP/TCAP parts and Opcode=IDP.

IDP Relay provides functions that handle complex numbering schemes and number conditioning, such as the following examples:

- The Nature of Address Indicator (NAI) could be used in a non-compliant manner (the NAI is set to International and the number format is not international).

- The Local Area Code (LAC) 2- byte field of the Local Area Identification (LAI) information element is used in one of the following ways: – As the Area Code in cases where the AC is needed but not provided in the CdPN – To determine how to format the outgoing CdPN in the IDP query.

- The collect call Escape Codes 90 and 90909 might need to be stripped and re-inserted after the RN.

- The Carrier Selection Point (CSP) can be removed from the incoming number and sometimes re-inserted (as when the LAC is not equal to the AC).

- The RN for the CgPN might be needed when the call is identified as a collect call.

- Service Key selection could vary, and could require a change in the number of bytes present the Service Key.

- Unsegmented XUDT messages might be required.

- Sometimes the RN but not the SP, or the SP but not the RN, or both the RN and SP are required in the outgoing number format.

The Mobile Originated Short Message Service (MO SMS) features address the number portability requirements of wireless network operators for delivery of Mobile Originated SMS messages. The vSTP 5 ISS MO SMS features apply number portability database lookup to SMS messages for GSM networks, validates subscriber use of the correct Short Message Service Center, and delivers messages to Prepaid Servers if either the Calling Party Number or Called Party Number is associated with a prepaid subscriber.

These features include:

- Mobile Originated Based GSM SMS Number Portability (MO-Based GSM SMS NP)

- Portability Check for Mobile Originated SMS (MNP SMS)

- Prepaid Short Message Service Intercept (PPSMS)

- Mobile Originated SMS Additional Subscriber Data (MO SMS ASD)

- Mobile Originated SMS Generic Routing Number (MO SMS GRN)

The MO SMS features are based on the vSTP platform with UDR. Numbering Plan Processor (NPP) is used by the MO SMS features for number conditioning and service logic execution.

#### MEALS

      - Measurements

Table 83 – Measurements

| Meas ID | Name | Report Group | Collection Interval | Dimension |
| --- | --- | --- | --- | --- |
| 21739 | VstpSccpIdprCdpn | Vstp IDPR Performance | 30 min | Single |
| 21740 | VstpSccpIdprCdpn2 | Vstp IDPR Performance | 30 min | Single |
| 21741 | VstpSccpIdprCdpn3 | Vstp IDPR Performance | 30 min | Single |
| 21742 | VstpSccpIdprCdpn4 | Vstp IDPR Performance | 30 min | Single |
| 21743 | VstpSccpIdprMsrcv | Vstp IDPR Performance | 30 min | Single |
| 21744 | VstpSccpIdprMsErr | Vstp IDPR Performance | 5 min | Single |
| 21745 | VstpSccpIdpSkgtart | Vstp IDPR Performance | 30 min | Single |
| 21746 | VstpSccpIdpSkgtart2 | Vstp IDPR Performance | 30 min | Single |
| 21747 | VstpSccpIdpSkgtart3 | Vstp IDPR Performance | 30 min | Single |
| 21748 | VstpSccpIdpSkgtart4 | Vstp IDPR Performance | 30 min | Single |
| 21749 | VstpSccpIdpInpConn | Vstp IDPR Performance | 30 min | Single |
| 21750 | VstpSccpIdpInpConn2 | Vstp IDPR Performance | 30 min | Single |
| 21751 | VstpSccpIdpInpConn3 | Vstp IDPR Performance | 30 min | Single |
| 21752 | VstpSccpIdpInpConn4 | Vstp IDPR Performance | 30 min | Single |
| 21753 | VstpSccpIdpInpCont | Vstp IDPR Performance | 30 min | Single |
| 21754 | VstpSccpIdpInpCont2 | Vstp IDPR Performance | 30 min | Single |
| 21755 | VstpSccpIdpInpCont3 | Vstp IDPR Performance | 30 min | Single |
| 21756 | VstpSccpIdpInpCont4 | Vstp IDPR Performance | 30 min | Single |
| 21757 | VstpSccpIdpAPtyRtd | Vstp IDPR Performance | 30 min | Single |
| 21758 | VstpSccpIdpAPtyGtt | Vstp IDPR Performance | 30 min | Single |
| 21759 | VstpSccpIdpSkrtd | Vstp IDPR Performance | 30 min | Single |
| 21760 | VstpSccpIdpAPtySkr | Vstp IDPR Performance | 30 min | Single |
| 21761 | VstpSccpIdpInpRlc | Vstp IDPR Performance | 30 min | Single |
| 21762 | VstpSccpIdpInpRlc2 | Vstp IDPR Performance | 30 min | Single |
| 21763 | VstpSccpIdpInpRlc3 | Vstp IDPR Performance | 30 min | Single |
| 21764 | VstpSccpIdpInpRlc4 | Vstp IDPR Performance | 30 min | Single |
| 21765 | VstpSccpIdpInpRtg | Vstp IDPR Performance | 30 min | Single |
| 21766 | VstpSccpIdpInpRtg2 | Vstp IDPR Performance | 30 min | Single |
| 21767 | VstpSccpIdpInpRtg3 | Vstp IDPR Performance | 30 min | Single |
| 21768 | VstpSccpIdpInpRtg4 | Vstp IDPR Performance | 30 min | Single |
| 21769 | VstpSccpMsGwsAGt | Vstp IDPR Performance | 5 min | Single |
| 21770 | VstpSccpIdprMsSucc | Vstp IDPR Performance | 30 min | Single |
| 21771 | VstpSccpIdprMsFail | Vstp IDPR Performance | 5 min | Single |
| 21772 | VstpSmsMogRecv | Vstp MOSMS Performance | 5 min | Single |
| 21773 | VstpSmsMogErr | Vstp MOSMS Performance | 5 min | Single |
| 21774 | VstpSccpMoSmsSegOk | Vstp MOSMS Performance | 30 min | Single |
| 21775 | VstpSccpMoSmsSegErr | Vstp MOSMS Performance | 30 min | Single |
| 21788 | VstpSccpIdpBlkConn | Vstp IDPR Performance | 30 min | Single |
| 21789 | VstpSccpIdpBlkCont | Vstp IDPR Performance | 30 min | Single |
| 21659 | vstpGportNonCallRelay | Vstp MNP Exception | 5 min | Single |
| 21660 | vstpGportNonCallGtt | Vstp MNP Performance | 5 min | Single |

      - Alarms & Events

Table 84 – Events for vSTP IDPR MOSMS Feature

| ID | Event Name | Raise Condition |
| --- | --- | --- |
| 70310 | VstpTooManyDigitDRA | DRA digits have exceeded INAP_MAX_CDPN_DIGITS (32) |
| 70311 | VstpIdprCgpnEcdError | IDPR CGPN encoding failed |
| 70312 | VstpIdprCdpnEcdError | IDPR CDPN encoding failed |
| 70313 | VstpIdprCdpnNppServiceOff | IDPRCDPN(X) NPP SERVICE is OFF |
| 70314 | VstpIdprCgpnNppServiceOff | IDPRCGPN NPP SERVICE is OFF |
| 70315 | VstpDestAddrDcdFail | DESTINATION ADDRESS DECODING is FAIL |
| 70316 | VstpTcapEncFail | TCAP ENCODING is FAIL |
| 70317 | VstpOutBoundDigit | OUT OF BOUND DIGIT |
| 70318 | VstpSMSMandParamMiss | SMS MANDATORY PARAMETER MISSING |
| 70319 | VstpAddrDcdFail | ADDRESS DECODING is FAIL |
| 70320 | VstpMnpCdpaMatchHomeSmsc | MNPCDPA MATCHES HOME SMSC |

Limitation

Flow control and Congestion control mechanisms are not fully implemented in DSR release 8.4.0.3.0.

## VSTP EIR ENHANCEMENTS

#### Purpose and Solution

Purpose

- Support Equipment Identity Register functionality in vSTP

- Help network operators to reduce GSM mobile handset thefts by providing a mechanism that allows the network operators to prevent stolen or disallowed handsets from accessing the network.

Solution

- vEIR IMSI whitelist expansion: Total 100K IMSI white listing are now supported on vSTP and DEIR (Diameter EIR)

- vEIR Logging enhancements:
White listed IMEI/IMSI logging is already available. Use the Eir Options MO to enable white listed IMSI/IMEI logging.

- Instead of IP, vSTP will log the OPC in the logs.

- Instead of IP, DEIR will log the Origin Host and Origin Realm in the logs

- OPC/Origin Host and Origin Realm will be logged by default. There will not be any options to enable/disable logging OPC/Origin Host and Origin Realm.

Feature Overview

EIR provides vSTP the ability to query Mobile Station’s Identity from a designated repository provisioned by the network operator.  This enables the operator to block stolen Mobile Stations from accessing the network. Querying the Mobile Station Identity and methods to decide final equipment status are configurable.

#### MEALS

      - Measurements

Table 85 – Measurements

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| VstpEirMsgRecv | Single | Number of messages successfully received by EIR application | 5 | Performance | Simple |
| VstpEirMsgTrans | Single | Number of messages successfully transmitted by EIR application to SCCP layer | 5 | Performance | Simple |
| VstpEirBlackImei | Single | Number of IMEI that are black listed | 5 | Performance | Simple |
| VstpEirGrayImei | Single | Number of IMEI that are Gray listed | 5 | Performance | Simple |
| VstpEirWhiteImei | Single | Number of IMEI that are White listed | 5 | Performance | Simple |
| VstpEirUnkImei | Single | Number of IMEI that are Unknown | 5 | Performance | Simple |
| VstpEirImeiNotFound | Single | Number IMEI that are not found in DB | 5 | Performance | Simple |
| VstpEirBlackAllwImei | Single | Number of IMEI that are black listed but allowed due to IMSI over-rider | 5 | Performance | Simple |
| VstpEirBlackImsiFail | Single | Number of IMEI black listed which has IMSI match failed | 5 | Exception | Simple |
| VstpEirImsiRangeSucc | Single | Number of Response sent using IMSi range match. Response can be White, Black, Gray or Unknown. | 5 | Performance | Simple |
| VstpEirDiscLssFul | Single | Number of messages discarded due to Lss stack queue was full. | 5 | Exception | Simple |
| VstpEirDiscSccpTxFail | Single | Number of messages discarded by Lss because of send fail to SCCP Layer | 5 | Exception | Simple |
| VstpEirDiscCATxFail | Single | Number of messages discarded by Lss because of send fail to CA layer. | 5 | Exception | Simple |
| VstpEirDiscUnkSsn | Single | Number of messages discarded due to unknown SSN | 5 | Exception | Simple |
| VstpEirDiscIMEIMis | Single | Number of messages discarded to missing IMEI | 5 | Exception | Simple |
| VstpEirProcessMax | Single | Max time for processing of EIR message received from SCCP layer and sending back the response | 5 | Performance | Peak |
| VstpEirProcessAvg | Single | Avg time for processing of EIR message received from SCCP layer and sending back the response | 5 | Performance | Average |
| VstpEirProcesTime | Array | Processing time of EIR message received from SCCP layer and sending back the response. (Bucketed at 10ms intv) | 5 | Performance | Arrayed |
| VstpEirCAQurProcessMax | Single | Max time for CA query response time to UDR | 5 | Performance | Peak |
| VstpEirCAQueProcessAvg | Single | Avg time for CA query response time to UDR | 5 | Performance | Average |
| VstpEirCAQueProcesTime | Array | Processing time required for time for CA query response time to UDR. (Bucketed at 10ms intv) | 5 | Performance | Arrayed |
| VstpEirQueTimeOut | Single | Number of messages for which CA query to UDR timed out. | 5 | Exception | Simple |
| VstpEirDiscCADcdFail | Single | Number of messages discarded by LSS due to decode failed of CA response message | 5 | Exception | Simple |
| VstpEirDiscPduFul | Single | Number of messages discarded when PDU pool is exhausted | 5 | Exception | Simple |
| VstpEirDiscIntErr | Single | Number of messages discarded due to internal processing error. | 5 | Exception | Simple |
| VstpBadMessageFormat | Single | Number of badly formatted messages | 5 | Exception | Simple |
| VstpEirDbQueryFailUDRConnDown | Single | Number of EIR DB Queries not initiated due to UDR connectivity down | 5 | Exception | Simple |

      - Alarms & Events

Table 86 – Alarms & Events for vSTP EIR Feature

| Alarm/Event Name | Type | Description | Raise Condition | Clear Condition | Throttle Sec | Instance | Additional Information | Severity |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EIR Application Status Changed (70068) | Alarm | CA Service unavailable or Congested | When UDR connection is down. When CA service is down or degraded. | When UDR connection is up. CA service is available. | 300 | None | Reason for degrade status | Critical |
| TCAP Invalid Parameter or Decode failure (70069) | Event | Failed to decode TCAP parameter | Invalid len in transaction portion Invalid len in dialogue portion Invalid len in component portion No originating transaction ID Invalid transaction ID len Dest transaction ID in Begin No External element No External Object Identifier Not Structured Dialogue No External ASN1-Type No External element No External Object Identifier Not Structured Dialogue No Dialogue Request No Application Context Name No ACN Object Identifier No component portion No Invoke ID No operation code Unsupported network type Unsupported operation code | NA | 10 | None | CdPA PC, SSN and RI CgPA PC, SSN and RI TCAP data | NA |
| Message Encode failed (70070) | Event | Failed to encode message | Error in encoding of messages due to incorrect CGPA parameter. | NA | 30 | None | CdPA PC, SSN and RI CgPA PC, SSN and RI | NA |
| Missing IMEI (70071) | Alarm | IMEI is missing in the received message | IMEI is missing in the received message | Auto clear after 5 mins | 300 | None | CdPA PC, SSN and RI | Minor |
| Invalid IMEI length (70072) | Event | Invalid Length for Map IMEI Parameter | IMEI invalid length | NA | 10 | None | CdPA PC, SSN and RI CgPA PC, SSN and RI | NA |
| Unknown Message (70073) | Event | Unsupported TCAP message type | Unsupported TCAP message type | NA | 10 | None | CdPA PC, SSN and RI CgPA PC, SSN and RI | NA |
| Logging Error in MP (70078) | Alarm | Log write Error in MP | LogDirCreateError LogFileCreateError LogFileOpenError LogFileWriteError LogLowDiskSpace LogHighEventRate LogMetaFileCreateError LogMetaFileWriteError LogHighEventRate | Logging resumes in MP | 86400 | None | Cause of Error | Minor |
| Log fetch Error from SOAM (70077) | Alarm | Eir Log copy from MP to SOAM failure | SOAM fails to copy EIR log from SOAM | SOAM is able to copy the Eir Logs from SOAM | 86400 | None | Cause of failure | Minor |
| Lss Stack Event Queue Utilization (70075) | Alarm | The percent utilization of the VSTP MP's LSS Stack Event Queue is approaching its maximum capacity | Stack Queue utilization crosses 60/80/95 percent | Stack Queue utilization comes back to 50/70/90 percent | 86400 | None | NA | Minor |
| Logging Stack Event Queue Utilization (70076) | Alarm | The percent utilization of the VSTP MP's Logging Stack Event Queue is approaching its maximum capacity | Stack Queue utilization crosses 60/80/95 percent | Stack Queue utilization comes back to 50/70/90 percent | 86400 | None | NA | Minor |

Limitation

- Assumption is UDR NO’s replication will be in good health always.

- If for any reason replication is down between Active NO and Standby or Spare NO’s for longer duration there can be inconsistency of data between NO’s and can lead to different equipment status for same IMEI number.

# Release 8.4.0.5.0 Feature OAM Changes

At the time of upgrade to DSR 8.4.0.5.0 a number of features and enhancements will become visible on the interfaces to the DSR and may change certain existing OAM behaviors of the system.

OAM changes includes: User Interfaces (NO GUI, SO GUI), Measurements Reports, Alarms, and KPIs.

## SLS ROTATION

#### Purpose and Solution

Purpose

- In many cases, MSCs switches and other originating nodes do not send an adequate distribution of SLS values. For example, in case of ITU ISUP messages, SLS is obtained from the lower 4 bits of the CIC field representing the circuit being used. CIC selection can be determined based on an odd/even method where a SSP uses either all odd CICs or all even CICs to help prevent glaring. This causes the LSB of the SLS to be fixed (0 or 1) which means SSP sends either odd or even SLS. Thus the transit nodes (STPs), do not achieve a good distribution of traffic across links.

- For combined linkset, in ANSI and ITU MTP protocols, the LSB of the SLS is used to load share between linksets of a combined linkset and the remaining SLS bits are used to distribute traffic across different links within a linkset. Since STP receives improper distribution of SLS value (either LSB as 0 or 1), hence the STPs cannot perform proper loading sharing across the linkset and the links of a linkset.

- For single linkset, since STP receives improper distribution of SLS value (either LSB as 0 or 1), the STPs cannot perform proper loading sharing across all the links of a linkset.

Solution

SLS Rotation feature allows the user to use the below options for addressing the problem.

- Outgoing Bit Rotation

- Use of Other CIC Bit

- Incoming Bit Rotation

- Random SLS

- ANSI 5-bit to ANSI 8-bit SLS Conversion

- ITU to ANSI SLS Conversion

- ANSI to ITU SLS Conversion

Note: The SLS modified using the above mentioned solutions is only used for internal linkset and link selection. The actual SLS field of the message (i.e. the SLS value received by the vSTP is the SLS value sent out by the vSTP) is not modified.

Feature Overview

- Outgoing Bit Rotation

  - The User can have the vSTP to rotate the 4 bits of SLS, according to outgoing Linkset, thus changing the LSB of the SLS.

  - If configured, this option is applied and SLS will be converted from 5-bit to 8-bit.

- ITU to ANSI SLS Conversion

  - If the ITU 4-bit SLS is “ABCD” then the ANSI 5-bit SLS will be “D (~D) ABC”, which is already implemented as a part of ANSI<->ITU Conversion feature.

  - Secondly, this conversion “ITU 4-bit to ANSI 5-bit” may be followed by 5-bit ANSI to 8-bit ANSI SLS conversion to achieve more randomization for linkset/link selection during the network conversion.

- ANSI to ITU SLS Conversion

  - Firstly, 5 or 8 bit ANSI SLS value is converted to the 4-bit ITU SLS value by doing MOD 16.

  - Secondly, this conversion may be followed by 4-bit ITU to 8-bit ITU SLS conversion to achieve more randomization for linkset/link selection during the network conversion.

- ANSI 5-bit to ANSI 8-bit SLS Conversion

  - The User can have the vSTP to perform the 5-bit ANSI conversion to 8-bit ANSI.

  - If configured, this option is applied and SLS will be converted from 5-bit to 8-bit.

- ITU to ANSI SLS Conversion

  - If the ITU 4-bit SLS is “ABCD” then the ANSI 5-bit SLS will be “D (~D) ABC”, which is already implemented as a part of ANSI<->ITU Conversion feature.

  - Secondly, this conversion “ITU 4-bit to ANSI 5-bit” may be followed by 5-bit ANSI to 8-bit ANSI SLS conversion to achieve more randomization for linkset/link selection during the network conversion.

- ANSI to ITU SLS Conversion

  - Firstly, 5 or 8 bit ANSI SLS value is converted to the 4-bit ITU SLS value by doing MOD 16.

  - Secondly, this conversion may be followed by 4-bit ITU to 8-bit ITU SLS conversion to achieve more randomization for linkset/link selection during the network conversion.

Note: All algorithms are applicable to both MTP Routed and GT Routed MSUs.

#### meals

      - Measurements

- No new Measurements are being implemented as part of this feature.

- Appropriate Logging will be done accordingly to print the Rotated SLS value used for Linkset and Link selection. The logs should be optional only. User need to turn on the M3RL SLS INFO (13th bit) to see the logs.

Note: The traces are not supposed to be enabled for TPS rate>100 as it might cause undesired behavior.

      - Alarms & Events

No new Events are being implemented as part of this feature.

No new Alarms are being implemented as part of this feature.

Limitation

- Usage of 5th bit as LSB for incoming bit rotation is to be avoided if all the nodes are GR compliant. This is due to the fact that ANSI mandated outgoing 5 bit rotation causes the 5th bit to not have a uniform distribution of 0’s and 1’s.

- If 5 to 8 Bit Conversion is applied on incoming 5 bit SLS, then 3 new SLS bits (calculated based on the OPC) will be prefixed to the 5-bit SLS. If all 8 SLS bits are considered for applying ISLSBR, the 3 new sls bits will become sticky bits and will cause un-even distribution. In this scenario, ISLSRSB value 6-8 will cause even more un-even distribution.

- If 5-bits SLS is received on incoming linkset, 5-to-8 bit conversion is ‘OFF’ on outgoing linkset and 8-bits SLS are to be considered for applying ISLSBR, then no rotation shall happen. The “5-to-8 Bit Conversion” option should be turned ON to perform ISLSBR.

- When two linksets are used as a combined linkset, they should have the same settings for all SLS algorithms (Example “Other CIC Bit”, “Rotated SLS Bit”), else there can be random behavior..  This is not enforced in the vSTP, and there is no warning mechanism for incorrectly provisioned linksets and routes

- Different RANDSLS configurations on two linksets, which happen to be a part of combined linkset for the routes defined for a destination node, may result in undesired SLS distribution. vSTP shall not prompt or reject the linkset provisioning command, if provisioning is done contrary to the above.

- For different segments of the same MSU, randsls will generate different SLS and hence different link selection. For other SLS algorithms, we assume that the Incoming linkId/SLS is same for different segments of the same MSU, hence the outgoing linkId/linkset id will be same for different segments of the same MSU.

Dependencies

The SLS Rotation feature for vSTP has no dependency on any other vSTP operation. The following points must be considered for SLS Rotation functionality:

- Usage of 5th bit as LSB for incoming bit rotation must be avoided if all the nodes are GR compliant. This is due to the fact that ANSI mandated outgoing 5 bit rotation causes the 5th bit to not have a uniform distribution of 0's and 1's.

- If 5 to 8 Bit Conversion is applied on incoming 5 bit SLS, then 3 new SLS bits (calculated based on the OPC) are prefixed to the 5-bit SLS. If all 8 SLS bits are considered for applying ISLSBR, the 3 new SLS bits become sticky bits and cause uneven distribution. In this scenario, ISLSRSB value 6-8 cause even more uneven distribution.

- If 5 bits SLS is received on incoming linkset, 5 to 8 bit conversion is OFF on outgoing linkset, and 8 bits SLS are considered for applying ISLSBR, then no rotation happens. The 5 to 8 Bit Conversion option must be turned ON to perform ISLSBR.

- When two linksets are used as a combined linkset, they should have the same settings for all SLS algorithms (For example, Other CIC Bit, Rotated SLS Bit), otherwise there can be a random behavior. This is not enforced in vSTP , and there is no warning mechanism for incorrectly provisioned linksets and routes.

- Different RANDSLS configurations on two linksets , which happen to be a part of combined linkset for the routes defined for a destination node may result in undesired SLS distribution. vSTP does not prompt or reject the linkset provisioning command if the provisioning is done contrary to the above.

- For different segments of the same MSU, randsls generates different SLS and different link selection. For other SLS algorithms, it is assumed that the Incoming linkId or SLS is same for different segments of the same MSU, hence the outgoing linkId or linkset id will be same for different segments of the same MSU.

Troubleshooting Steps

The troubleshooting scenarios for SLS Rotation:

- If no SLS Rotation algorithm is applied.

- Ensure that correct parameters are set on ingress and egress Linkset connected to vSTP MP as per SLS Rotation Algorithm.

- Ensure that appropriate m3rloptions MO parameters are set.

- SLS Rotation algorithms are specific to domain and type of message such as, SCCP or ISUP. Therefore, the configurations must be done accordingly. For example, Algorithm Use of Other CIC bit is applicable only for ITU ISUP messages.

- If ANSI SLS in Egress Message is not correct as per the SLS Rotation Algorithm applied:

- Apart from SLS Rotation algorithms, for ANSI domain only Standard 5th Bit Rotation is always applied and is modified in Egress Message.

- If SLS Rotation on Domain Conversion is not working properly:

- Few parameters can be set on Linksets, so while performing Domain Conversion make sure correct parameter values are specified to get desired output.

- Slide 33 – 40 must be referred for these scenarios.

- For ANSI, check value of parameter ASLS8 in Incoming Linkset.

- Also, Interaction between different algorithms of SLS Rotation during Domain Conversion has certain exceptions, refer slide 38 and 40 for it.

- If certain SLS Algorithm does not get applied.

  - When multiple algorithms are applied to a particular domain message type, the SLS Rotation algorithms are applied as per points mentioned in slide 31 and 32. Combining SLS Rotation Options.

  - Modifying SLS Rotation related parameter values can render one of SLS Rotation Algorithm as inapplicable. Revert the modified parameter values to return to the previous manner of load sharing.

If issue still exists, then contact Oracle for support.

## SFAPP DYNAMIC LEARNING

#### purpose and solution

Purpose

vSTP to create a whitelist of VLRs it interacts with by learning from the results of the validation methods.

Solution

This use case will provide protection against all messages coming from VLRs that fail the validation and are not part of the whitelists created. A grey list and black list shall also be created for the VLRs that fail the validation.

Feature Overview

The Stateful Security Dynamic Learning feature enables vSTP to create and use a whitelist that is created as part of learning from the validation attempts defined in VLR Validation. This feature is independent of the category of messages but it provides protection against all the messages coming from VLRs that fail the validation and are

not part of the created whitelists. A grey list and black list is also created for the VLRs that fail the validation.

Learning is controlled by these modes using a mode parameter in the SFAPPOPTS table:

- Learn Mode: This mode allows to learn about new VLRs and no validations are performed. The newly learnt VLRs are considered as whitelisted.

Note: The user can configure the amount of time for which the vSTP operates in Learn mode. The time is configured in SFAPPOPTS table. Hence, the switch from Learn to Test mode can happen either by configuring the timer, or manual switch.

- Test Mode: This mode validates all the learned VLRs. In case the VLR is not validated, the learnt VLRs remains greylisted and measurements and alarms are generated.

- Active Mode : This mode allows validations based on the learned white lists in the system. New VLRs are also learned in this mode. The status of dynamically learnt VLRs are changed to whitelist or blacklist as per their status.

- OFF Mode: When none of the above modes is active, it is considered as OFF mode. In this mode, if VLR entry is in whitelist, then no validation is performed for that VLR. By default, the OFF mode remains enabled. That means the SFAPP dynamic learning functionality is disabled.

Note: In any mode, if VLR is in whitlist table, then it is considered as whitelisted, and the message is forwarded to HLR. If user has changed the mode from Learn/Test/Active mode to OFF mode, then the user has to wait for at least 10 mins before switching the mode again to Active/Learn/Test mode.

#### meals

      - Measurements

Table 87 – Measurements

| MeasID | Measurement Name | Description | Group | Interval | Type |
| --- | --- | --- | --- | --- | --- |
| 21937 | VstpDynNewVLR | Total number of New Dynamic VLRs Learned. | SFAPP Exception | 5 min | Single |
| 21938 | VstpDynNewRoamEntry | Total number of New Dynamic VLR Roaming entries Learned. | SFAPP Exception | 5 min | Single |
| 21939 | VstpDynVLRBL | Total number of VLRs moved to Blacklist | SFAPP Exception | 5 min | Single |
| 21940 | VstpDynVLRWL | Total number of VLRs moved to Whitelist | SFAPP Exception | 5 min | Single |
| 21941 | VstpDynVLRGL | Total number of VLRs moved to Graylist | SFAPP Exception | 5 min | Single |
| 21942 | VstpDynVelCrossed | Total number of entries for which Velocity check threshold crossed | SFAPP Exception | 5 min | Single |
| 21943 | VstpDynVLRProfAging | Total number of VLRs Profile entries aged out | SFAPP Exception | 5 min | Single |
| 21944 | VstpDynVLRRoamAging | Total number of VLRs Roaming entries aged out | SFAPP Exception | 5 min | Single |

      - Alarms & Events

Table 88 – Alarms & Events

| ID | Event Name | Type | Raise Condition | Severity | Throttle Sec |
| --- | --- | --- | --- | --- | --- |
| 70429 | VstpDynVlrStatusChanged | Event | When Dynamic VLR Status Changed from Graylist to Blacklist/Whitelist | Info | 10 |
| 70430 | VstpDynVeloThreshCrossed | Event | VLR crossed the Velocity Threshold limit | Info | 10 |
| 70431 | VstpDynVLRProfAging | Event | When VLR is aged out | Info | 10 |
| 70432 | VstpDynVLRRoamAging | Event | When any VLR relation is aged out | Info | 10 |
| 70433 | VstpVlrDynLearningOFF | Event | When Dynamic Learning is turned OFF | Info | 10 |
| 70434 | VstpVlrDynLearningLearntimer | Event | When Learn Timer is expired | Info | 10 |
| 70435 | VstpVlrDynProfileTableFull | Alarm | When Sfapp Dynamic Profile is full (50000 entries) | Major |  |
| 70436 | VstpVlrDynProfileTableFull | Alarm | When Sfapp Dynamic Roaming is full (50000 entries) | Major |  |

Limitation

Wait for latest 10 min before switching OFF mode to any other mode.

## TIF SUPPORT

#### purpose and solution

Feature Overview

For TIF features, TIF provides an overall structure that allows the vSTP to intercept ISUP messages that would normally be through-switched and apply special processing to them. For example, an IAM message could be intercepted and have the called number prefix replaced based on portability information.

TIF processing consists of two main sections:

- TIF uses MTP to select an ISUP MSU for processing, and forwards the MSU to Service Module cards for processing.

- TIF decodes the MSU, invokes the Numbering Plan Processor (NPP), and encodes the results. 
TIF features provide NPP with Service Action Handlers to perform database access, data evaluation, and any feature-specific handling for the MSU.

#### meals

      - Measurements

Table 89 – Measurements

| MeasID | Measurement Name | Description | Group | Interval | Type |
| --- | --- | --- | --- | --- | --- |
| 21921 | VstpTinpMsgRcv | Number of IAM messages received that require TIF processing | VSTP ISUP Performance | 5 min | Single |
| 21922 | VstpTinpMsgGen | Number of IAM messages received that required TIF processing and resulted in the modification of the IAM message or the generation of a REL message. | VSTP ISUP Performance | 5 min | Single |
| 21923 | VstpTinpErr | Number of IAM messages received that required TIF processing but resulted in execution of an error case. | VSTP ISUP Exception | 5 min | Single |
| 21924 | VstpTifRelease | Number of IAM messages received that were processed by TIF and found to be blacklisted by BLRLS Service Action. | VSTP ISUP Exception | 5 min | Single |
| 21925 | VstpTifNotFoundDnRelease | Number of IAM messages received that were processed by TIF and found to be blacklisted by BLNFNDRLS Service Action. | VSTP ISUP Exception | 5 min | Single |
| 21926 | VstpTifFpfxRelease | Number of IAM messages received that were processed by TIF and found to be blacklisted by FPFXRLS Service Action. | VSTP ISUP Exception | 5 min | Single |
| 21927 | VstpTifNoCgpnRelease | Number of IAM messages received that were processed by TIF and found to be blacklisted by NOCGPNRLS Service Action. | VSTP ISUP Exception | 5 min | Single |
| 21928 | VstpTifSelscrRelease | Number of MSUs processed by TIF and found to be blacklisted by SELSCR Service Action. | VSTP ISUP Exception | 5 min | Single |
| 21929 | VstpTifSelscrRelay | Number of MSUs processed by TIF and relayed by SELSCR Service Action. | VSTP ISUP Performance | 5 min | Single |
| 21930 | VstpIsupCAAvgProcessTime | Average time by CA to send query and receive the response from UDR. | VSTP ISUP Performance | 5 min | Single |
| 21931 | VstpIsupCAMaxProcessTime | Peak time by CA to send query and receive the response from UDR | VSTP ISUP Performance | 5 min | Single |
| 21932 | VstpIsupInternalError | Number of messages discarded due to internal processing error. | VSTP ISUP Exception | 5 min | Single |
| 21933 | VstpIsupCADecodeFail | Number of messages discarded by ISUP due to decode failed of CA response message. | VSTP ISUP Exception | 5 min | Single |
| 21934 | VstpIsupCATimeOut | Number of messages for which CA query to UDR timed out. | VSTP ISUP Exception | 5 min | Single |
| 21936 | VstpIsupCAProcessTime | Time required by CA to send query and receive the response from UDR. | VSTP ISUP Performance | 5 min | Arrayed |

      - Alarms & Events

Table 90 – Alarms & Events

| ID | Event Name | Type | Trigger Condition | Throttle Sec |
| --- | --- | --- | --- | --- |
| 70423 | VstpTifUnexpectedSi | Event | Only TUP and ISUP messages can be processed. | 10 |
| 70424 | VstpTifRouteFailed | Event | Message is too big (modification made it too large). | 10 |
| 70425 | VstpIsupDcdFailed | Event | ISUP Clg Party Decode Failed or ISUP Decode Failure Error. | 10 |
| 70426 | VstpIsupDcdCdpaFailed | Event | ISUP Cld Party Decode Failed. | 10 |
| 70427 | VstpIsupEcdFailed | Event | ISUP Cld Pty Encode Failed or ISUP Clg Pty Encode Failed or ISUP REL encoding failure. | 10 |
| 70428 | VstpTifCcMismatchDn | Event | CC mismatch in DN | 10 |

Limitation

The TIF feature has no dependency on any other vSTP operation.

## SEGMENTED XUDT

#### purpose and solution

Purpose

- When the destination for an XUDT message is determined by SCCP and MTP parameters then they will be routed properly by vSTP towards same destination. However, vSTP can route different segments of the same SCCP  large XUDT message to different destinations when features like TOBR, MBR  are applied on them.

- First segment (XUDT) usually contains TCAP/MAP layer parameters (Opcode, MSISDN, VLR, IMSI) are routed properly when TOBR/MBR feature is applied.

- Subsequent segments (XUDT) do not contain TCAP/MAP parameters needed for TOBR/MBR and hence these messages are routed differently without applying TOBR/MBR.

- Routing different segment of the same message to different destination is incorrect behavior.

Solution

- vSTP must implement a solution to ensure that all segments of the SCCP Class 1 XUDT messages are routed to the same destination irrespective of the service used for translation.

- To address this problem, vSTP need to support Segmentation and Reassembly of XUDT Class 1 SCCP Messages.

- vSTP shall perform Reassembly on the incoming segmented XUDT messages

- vSTP will then perform the services/translation on the Reassembled Message.

- vSTP shall perform Segmentation on the Outgoing XUDT Reassembled Message to generate segments and perform routing.

Feature Overview

The Segmented XUDT feature allows vSTP to perform the following operations:

- Reassembly of incoming XUDT Class 1 SCCP segmented messages

- Segmentation of the outgoing XUDT Class 1 SCCP reassembled messages

This functionality ensures that all segments of the SCCP Class 1 XUDT messages are routed to same destination, irrespective of the service used for translation.

vSTP performs reassembly on the incoming segmented XUDT messages. After the reassembly, the required services or translation is performed on the reassembled message.

The segmentation is performed on the outgoing XUDT reassembled message to generate segments and perform routing.

#### meals

      - Measurements

Table 91 – Measurements

| Measurement Id | Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- | --- |
| 21901 | VstpRxSccpReassProcSucc | Single | Number of times reassembly procedure completed successfully | 30 | VSTP SCCP Performance | Simple |
| 21902 | VstpRxSccpReassProcFail | Single | Number of times reassembly procedure failed | 30 | VSTP SCCP Exception | Simple |
| 21903 | VstpRxSccpXUDTSgmnts | Single | Number of ingress segmented XUDT messages received from network | 30 | VSTP SCCP Performance | Simple |
| 21904 | VstpRxSccpSgmntsDisc | Single | Number of segmented XUDT messages Discarded due to reassembly failure. | 30 | VSTP SCCP Exception | Simple |
| 21905 | VstpRxSccpSgmntsReassFail | Single | Number of segmented XUDT messages that encountered Reassembly failure due to any errors | 30 | VSTP SCCP Exception | Simple |
| 21906 | VstpTxSccpSegProcSucc | Single | Number of times segmentation procedure completed successfully | 30 | VSTP SCCP Performance | Simple |
| 21907 | VstpTxSccpSegProcFail | Single | Number of times segmentation procedure failed | 30 | VSTP SCCP Exception | Simple |
| 21908 | VstpTxSccpLargeMsgs | Single | Number of reassembled large messages received for segmentation | 30 | VSTP SCCP Performance | Simple |
| 21909 | VstpRxSccpReassSegSucc | Single | Number of Segmented XUDT Messages reassembled successfully | 30 | VSTP SCCP Performance | Single |

      - Alarms & Events

Table 92 – Alarms & Events

| Event Name | Event Id | Raise Condition |
| --- | --- | --- |
| SCCP XUDT Reassembly Failure | 70331 | When reassembly is failed due to any of the below conditions- out of sequence segments received, Internal Error, reassembly Timer Expired. Note: . The specific condition will be mentioned in the event reason. |
| SCCP XUDT Segmentation Failure | 70332 | If number of required segments is greater than the maximum number of segments, Maximum number of segments is 16. |

Limitation

- Segments of the same message received on different VSTP MPs (as result of CO or CB or any other scenario) will not be handled properly, and as a result reassembly error procedure will be initiated.

- Reassembly is performed for only segmented XUDT Class 1 messages. Segmentation functionality will be performed only on the reassembled messages(performed by vSTP)

- XUDT Reassembly functionality shall not be supported for Route on SSN messages.

Troubleshooting Steps

The troubleshooting steps for vSTP XUDT Segmentation feature are as follows:

- If a Segmented Class 1 XUDT message is received for reassembly, then the measurement VstpRxSccpXUDTSgmnts is pegged to count the Number of ingress segmented XUDT messages received from network.

- If the reassembly procedure is successful, then the measurement VstpRxSccpReassProcSucc is pegged to count the Number of times reassembly procedure completed successfully.

- If the reassembly procedure is successful, then the measurement VstpRxSccpReassSegSucc is pegged to count the Number of Segmented XUDT Messages reassembled successfully.

- If the reassembly procedure fails, then the measurement VstpRxSccpReassProcFail is pegged to count the number of times reassembly procedure failed.

- If the reassembly procedure fails, then the measurement VstpRxSccpSgmntsReassFail is pegged to count the Number of segmented XUDT messages that encountered Reassembly failure due to any errors.

- If the reassembly procedure fails, then the measurement VstpRxSccpSgmntsDisc is pegged to count the Number of segmented XUDT messages Discarded, this measurement is pegged if alwMsgDuringRsmblyErr in the sccpoptions MO is False.

- If a reassembled message is received for segmentation then the measurement VstpTxSccpLargeMsgs is pegged to count the number of reassembled large messages received for segmentation.

- If the segmentation procedure is successful, then the measurement VstpTxSccpSegProcSucc is pegged to count the number of times segmentation procedure completed successfully.

- If the segmentation procedure fails, then the measurement VstpTxSccpSegProcFail is pegged to count the number of times segmentation procedure failed.

- If reassembly procedure fails, then check the event SCCP XUDT Reassembly Failure is raised in the vSTP GUI with the following reasons:

    - out of sequence segments received

    - reassembly Timer Expired

    - Internal Error

If the reassembly failure occurs due to reassembly Timer Expiry, then user may need to adjust the value of the parameter reassemblyTimerDurationAnsi or reassemblyTimerDurationItu defined in sccpoptions MO.

- If segmentation procedure fails, then check the event SCCP XUDT Segmentation Failure raised in the vSTP GUI. The event is raised with the reason number of required segments is greater than the maximum number of segments. In case of this error, adjust the value of segmentedMSULength parameter in sccpoptions MO.

Contact My Oracle Support in case the problem persists.

Dependencies

The XUDT Segmentation feature has no dependency on any other vSTP operation.

The following points must be considered for XUDT Segmentation functionality:

- Segments of the same message received on different vSTP MPs (as result of CO or CB or any other scenario) are not completely supported. The reassembly error procedure will be initiated for such messages.

- Reassembly is performed for only segmented XUDT Class 1 messages. Segmentation functionality will be performed only on the reassembled messages(performed by vSTP).

- XUDT Reassembly functionality is not supported for Route on SSN messages.

## DUPLICATE POINT CODE SUPPORT

#### purpose and solution

Purpose

This feature will allow an vSTP to route traffic for two or more destinations/countries that may have overlapping point code values. For example, in the network shown in below figure, both Country 1 and Country 2 have SSPs with a PC value of 2047.

Solution

- The user must divide their ITU-National/Spare destinations into groups.  These groups would likely be based on Country.  However, 1 group could have multiple countries within it, or a single country could be divided into multiple groups.  The requirement for these groups would be:

- No duplicate point codes are allowed within a group

- ITU-National/Spare traffic from a group must be destined for a PC within the same group.

- The user must assign a unique two letter group code to each group.

- Each group will be identified with a group code and user will provide group code information while adding point code information.

- Group code will be two letter alphabet in the range ‘aa’ to ‘zz’

Feature Overview

The Duplicate Point Code support functionality allows vSTP to route traffic for two or more countries that may have overlapping point code values.

The users divide their ITU-National or Spare destinations into groups. These groups are based on the country. When the user enters an ITU National or Spare point code, they must also enter the group code to associate point code with groups. A group code is unique two letter code to identify a group.

ITU Point Code Support Functionality

When an ITU-N message arrives at vSTP, an internal point code based on the 14 bit PC is created in the message. Also, the group code gets assigned to the incoming linkset. The following points must be considered while configuring the Duplicate Point Code functionality:

- If the user does not assign any group code while adding ITU-N nodes (Local Signalling Point or Remote Signalling Points), then by default the aa group code is assigned.

- For every group that is used, either a True PC or secondary point code must be provided using the Local Signalling Point command.

- When a message is received from M3UA, then the group code is determined by the network appearance present in the message.

Operations for MTP3 and SCCP Management Messages

When vSTP receives a network management message concerning an ITU-National or Spare destination, the routeset to apply the message is determined based on the concerned point code and the group code of the message.

When vSTP generates MTP and SCCP management messages that concern an ITU- National or Spare destination, then only the messages with the same group code are sent to point codes.

When M3UA receives a management message (DAVA, DUNA), then the group code is determined by the NA present in the message.

Interaction

ITU-International linksets do not have a group code. ITU-National or Spare MSUs received on ITU-International linksets are assigned a group code of aa.

Gateway Screening has no impact of group codes support. However, the user can effectively screen on group codes by assigning a different screenset to linksets that have different group codes.

Each ITU-N destination and group code can have it’s own ITU-I or ANSI alias PC. Each ITU-I or ANSI node can be assigned one ITU-N destination. For conversion from ITU-I or ANSI to ITU-N to succeed, the ITU-N alias of the sending node must have the same group code as the destination group code. So each ITU-I or ANSI node can only send and receive messages from one ITU-N group.

#### meals

      - Measurements

No measurement changes for Duplicate Point Code.

      - Alarms & Events

No new event or alarm added for this feature.

Troubleshooting Steps

In case of the error scenarios, different vSTP alarms and measurements are pegged.

Dependencies

The Duplicate Point Code support feature has no dependency on any other vSTP operation.

The following points must be considered while configuring Duplicate Point Code functionality:

- The Duplicate Point Code support is applicable only for ITU-National/ITU-Spare Destinations.

- The ITU-National traffic from a group must be destined for a PC within the same group.

- No duplicate point codes are allowed within a group

- It is not possible to change the group code for a destination. To move a destination from one group to another, user must provision a new destination that uses the new group code and delete the old destination.

- If conversion between ITU-N and ITU-I or ANSI is used, then only one ITU-N group can send traffic to a specific ANSI or ITU-I node.

## VSTP IR21 BULK UPLOAD FOR SS7 SECURITY

#### purpose and solution

Purpose

Certain GSM MAP messages requires validation of information present at MAP and SCCP portion, based on that validation packets are either allowed or discarded. There are approx. 800 mobile network operators across world and information pertaining to their network resides in GSMA IR.21 document.  Operator wise Network information data viz. MCC-MNC, Node GT (HLR/VLR/MSC) and CC-NDC becomes huge and cumbersome for operator to maintain and upload that in vSTP to implement security check for CAT2 messages.

Solution

This feature allows a vSTP to provides security to detect anomalies on inbound packets through bulk upload of customer IR.21 documents.

Feature Overview

The goal is to develop a utility that will read and record all information present in GSMA IR.21 into a configuration file. This configuration file will be uploaded in vSTP directly from this Utility. The file (.csv) will have network node details for Operators Roaming Partner. This utility shall be external to vSTP node which can be installed on Linux machine.

SCPVAL GTT Action on EAGLE shall be enhanced to address SS7 CAT2 security checks. This GTT action will ensure MSU details viz. CGPA and IMSI belongs to same Operator after validating it with the newly generated table.

This feature will allow an vSTP  to read the IR.21 document and store that in tabular form which can then be referred by EAGLE for CAT2 security implementation.

There are three part of CAT2 feature.

- First part will parses IR.21 xml file on the Linux machine, extract the information needed from the IR.21 file for validation of the message and convert its data in  .csv format.

- Load this .csv file into vSTP through bulk upload. This data will be stored on SO’s and MP’s IR21RoutingInfo and IR21NetworkElement table. Note: We can use MMI also to populate individual entry in IR21RoutingInfo and IR21NetworkElement table, but the individual configuration is not the preferred way.

- Configure GTT to enforce CAT2 validation on received MSU. Validation shall be done through data available in IR21RoutingInfo and IR21NetworkElement table.

Following is the CAT2 flow diagram:

#### meals

      - Measurements

Table 93 – Measurements

| Measurement Name | Dimension | Description | Interval in Mins | Group | Type |
| --- | --- | --- | --- | --- | --- |
| VstpGttActScpvalCat2Total | Arrayed | The total number of messages received by SCPVAL CAT2 GTT Action. | 30 | Performance | Simple |
| VstpGttActScpvalCat2Discard | Arrayed | The total number of messages discarded by SCPVAL CAT2 GTT Action. | 30 | Performance | Simple |
| VstpGttActScpvalCat2NotApplied | Arrayed | The total number of messages where SCPVAL CAT2 GTT Action was not applied. | 30 | Performance | Simple |
| VstpCgpaGttActScpvalCat2Total | Arrayed | The total number of messages received by SCPVAL CAT2 GTT Action per CGTT. | 30 | Performance | Simple |
| VstpCgpaGttActScpvalCat2Discard | Arrayed | The total number of messages discarded by SCPVAL CAT2 GTT Action per CGTT. | 30 | Performance | Simple |
| VstpCgpaGttActScpvalCat2NotApplied | Arrayed | The total number of messages where SCPVAL CAT2 GTT Action was not applied per CGTT. | 30 | Performance | Simple |

      - Alarms & Events

No new event or alarm added for this feature.

Limitation

- Only Opcodes listed below shall be decoded to apply IMSI & CgPA check.

- provideRoamingNumber	4

- provideSubscriberInfo	70

- provideSubscriberLocation	83

- cancelLocation	                       3

- insertSubscriberData	7

- deleteSubscriberData	8

- getPassword	                       18

- reset	                                              37

- activateTraceMode	50

- unstructuredSS-Request	60

- unstructuredSS-Notify	61

- informServiceCentre	63

- alertServiceCentre	64

- setReportingState	                        73

- remoteUserFree	                        75

- istCommand	                        88

- Only First 5-6 digits from IMSI is considered for matching.

| <- 3 Digits -> | <- 2 or 3 Digits -> |  |
| --- | --- | --- |
| MCC | MNC | MSIN |
| < ----------------- Not more than 15 Digit ----------- > |  |  |

- IMSI is composed of three parts:

- Mobile Country Code (MCC)

- Mobile Network Code (MNC)

- Mobile Subscriber Identification Number (MSIN)

- MCC and MNC determines the Operator ID. This will be used for CAT2 validation.

- First match shall be performed with 6 digit and if match not found then it shall be performed with 5 digit else fail.

## DSA WITH UDR

Diameter Security Application (DSA) has implemented various Countermeasures to detect vulnerability in an ingress diameter message from a foreign network.

The Countermeasures can be divided into two categories.

- Stateful Countermeasure

- Stateless Countermeasure

Stateful Countermeasures are those Countermeasures which require to maintain State Data for validating vulnerability of the ingress diameter messages. These State-Data will be maintained in the UDR.

Stateless Countermeasures are those Countermeasure which do not require data from earlier diameter message for checking vulnerability of a given incoming diameter message. Message is screened for vulnerability by using DSA configuration data.

The Stateless Countermeasures are executed in the below sequence [if configured and enabled]:

- Application-Id Whitelist Screening

- Application-Id and Command-Code Consistency Check

- Origin Realm and Destination Realm Whitelist Screening

- Origin host and Origin Realm Consistency Check

- Destination-Realm and Origin-Realm Match Check

- Visited-PLMN-ID and Origin-Realm Consistency Check

- Realm and IMSI Consistency Check

- Subscriber Identity Validation

- Specific AVP Screening

- AVP Multiple Instance Check

The Stateful Countermeasures are executed in the below sequence [if configured and enabled] :

- Message Rate Monitoring

- Time-Distance Check

- Previous Location Check

- Source Host Validation HSS

- Source Host Validation MME

#### Upgrade

DSA with UDR in Release 8.4.0.0.5 does not support the upgrade.

#### Common security

Time distance check counter measure is common for DSA and VSTP applications and both the applications use UDR as a common DB for security across protocol use cases.

This Countermeasure is applicable across 4G network and also for Cross Protocol Security Use Case.

Time Distance Check validate the movement from 2G/3G location to 4G Location against configured min transit time between two location using IMSI as key value.

For example: First subscriber is roaming into 2G network, update location  comes to 2G home location  though vSTP application ( vSTP will update the subscriber details in UDR with IMSI as key. Then subscriber move to 4G network  within min transition time , update location comes to 4G Home network  through DSA Application , DSA application  read the data from UDR if already there and apply Business login and marked the message as vulnerable if transition from 2G  location to 4G location is within min transition time.

This counter measure provides common configuration for  both vSTP & UDR  for TimeDistChk_Country_Config table. This table is not recommended to edit when vSTP and DSA is running under same SOAM server for Common Security feature.

# MEAL INSERTS

This section will summarize the changes to Alarms, Measurements, KPIs and MIBs. In the following inserts pertain to DSR Release 8.4 MEAL snapshot and deltas to earlier releases 8.0.0, 8.1.0 and 8.2.1 and 8.3

- The DSR/SDS 8.4.0.0.0 GA Release is DSR/SDS 8.4.0.0.0-84.15.0

- The DSR/SDS 8.4.0.3.0 GA Release is DSR/SDS 8.4.0.3.0-85.17.0

- The DSR/SDS 8.4.0.5.0 GA Release is DSR/SDS 8.4.0.5.0-88.9.1

## DSR/SDS 8.4.0.5.0 MEAL Snapshot

`

### MEAL Delta between 8.4.0.0.0 and 8.4.0.5.0

### MEAL Delta between 8.4.0.3.0 and 8.4.0.5.0

## DSR/SDS 8.4.0.3.0 MEAL Snapshot

### MEAL Delta between 7.0.1.0.0 and 8.4.0.3.0

## DSR/SDS 8.4.0.0.0 MEAL Snapshot

### MEAL Delta between 8.0.0.0.0 and 8.4.0.0.0

### MEAL Delta between 8.1.0.0.0 and 8.4.0.0.0

### MEAL Delta between 8.2.1.0.0 and 8.4.0.0.0

### MEAL Delta between 8.3.0.0.0 and 8.4.0.0.0

# Reference list

The DSR 8.4 Release Notice and Customer Documentation can be found at the following OTN link.

http://docs.oracle.com/en/industries/communications/diameter-signaling-router/index.html

DSR IP Flow Document: CGBU_019284 (ORACLE Internal Document)

Platform IP Flow Document: CGBU_PM_1112 (ORACLE Internal Document)

- This represents the minimum release of the HP FUP 2.2.x series to support all content in the Platform 74  release.  It is recommended that the latest firmware release always be used in order to address known security issues.

- This represents the minimum release of the Oracle firmware series to support all content in the Platform 74 release.  It is recommended that the latest firmware release always be used in order to address known security issues.

- This represents the minimum release of the HP FUP 2.2.x series to support all content in the Platform 74  release.  It is recommended that the latest firmware release always be used in order to address known security issues.

- This represents the minimum release of the Oracle firmware series to support all content in the Platform 74 release.  It is recommended that the latest firmware release always be used in order to address known security issues.

- This represents the minimum release of the HP FUP 2.2.x series to support all content in the Platform 74  release.  It is recommended that the latest firmware release always be used in order to address known security issues.

- This represents the minimum release of the Oracle firmware series to support all content in the Platform 74 release.  It is recommended that the latest firmware release always be used in order to address known security issues.